3GPP TSG-RAN WG4 Meeting #115	R4-2505478

Malta, May 19 – May 23, 2025

Source:	Tejas Networks

Title:	RRM core requirement and testing framework for AIML based beam management

Agenda item:	7.21.4

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#114bis in the scope of Rel-19 WI. Agreements achieved during this meeting are summarized in the WF [1] and the ad-hoc minutes [2].

In this paper, we provide our views on issues related to requirements for beam prediction.

Discussion

Absolute RSRP accuracy

Agreement: Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams

RAN4 agreed on the definition of absolute RSRP accuracy in RAN4 #112bis meeting as follows:

The absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground truth of L1-RSRP of beam index i. Here, the index i may correspond to any beam index within the top-K beams based on the ground truth L1-RSRP.

The absolute RSRP accuracy metric provides insight into how much the predicted RSRP values deviate from the actual ground truth RSRP values. It is a critical parameter for evaluating the performance of AI/ML-based beam prediction. The absolute RSRP requirement should be applied to all top-k predicted beams to ensure consistent prediction quality. Since the network may not always be able to allocate the best beam, having multiple high-quality beam options allows it to fall back on the second or third best when needed. This flexibility enables the network to select the most suitable beam among the predicted candidates. Furthermore, the AI/ML model predicts a set of top-k beams from a larger set, and it is important to verify whether these top-k predictions follow the actual ranking based on real RSRP values. If not, they should at least show a close similarity to real measurements. Verifying RSRP accuracy for all top-k predicted beams helps assess the rank consistency of the prediction.

Additionally, to minimize performance degradation from AI/ML-based beam management, the predicted RSRP accuracy requirement should be close to that of legacy requirements.

Proposal 1: Absolute RSRP accuracy requirement should be applied to all the top K predicted beams.

Proposal 2: The predicted RSRP accuracy requirement should be close to the legacy requirement to reduce the performance degradation due to AIML based BM.

Measurement period for inference

Agreement: - For measurement delay component of the prediction delay: - Use “legacy” L1-RSRP measurement as the baseline for Case 1 - FFS whether same values for M, N, P should be reused - FFS for case 2

AI/ML based beam management includes two cases. BM-Case1: Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams. BM-Case2: Temporal Downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams. For BM-Case 1, the UE first needs to measure set B, and predicts the set A beams, which is similar to the legacy L1-RSRP measurement process; therefore, the legacy L1-RSRP measurement delay can be used as a baseline reference. In BM-Case 2, predictions are made based on historical measurement data, meaning the AI/ML model can utilize measurement results from multiple time instances and average them across several measurement occasions to achieve more accurate prediction results.

The prediction delay can be broadly divided into three components: measurement delay, inference delay, and reporting delay. For the AI/ML model, inference delay refers to the time taken from feeding data into the AI/ML model to generating the corresponding prediction result. This delay largely depends on the performance of the AI/ML model and the number of iterative measurement results used to generate the predicted results. Therefore, defining a separate requirement for inference delay may not be practical, and it could be more appropriate to include inference delay as part of the overall reporting delay.

Proposal 3: No need to define separate requirement for inference delay, it should be considered as a component of the overall reporting delay.

Compared to BM-Case 1, BM-Case 2 may need to utilize measurement results from multiple time instances, averaging them over several measurement occasions. However, in terms of accuracy, a unified accuracy requirement should be maintained for each L1-RSRP measurement delay in both BM-Case 1 and BM-Case 2, as there is no distinction between the two cases from an accuracy perspective. Nevertheless, due to differences in measurement approaches and delay components, defining a unified requirement for both BM-Case 1 and BM-Case 2 may not be feasible or optimal.

The new measurement reporting requirements will require to be addressed with inference delay to allow multiple time instances measurements for BM-Case2.

Observation 1: Increasing the inference delay to achieve better accuracy may lead to suboptimal system performance.

Proposal 4: RAN4 to define separate reporting delay requirement for BM case 1 and BM case 2.

Beam prediction KPIs

Beam prediction KPIs Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered Option 1: RSRP accuracy Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x Option 4: combinations of above options Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Both Top-K/1 (%) and Top-1/K (%) from Option 2 use similar approaches, evaluating the probability that either the Top-1 strongest beam is part of the Top-K predicted beams, or the Top-1 predicted beam is one of the Top-K strongest beams. However, when K is large, the likelihood of the Top-1 strongest beam becoming part of the Top-K predicted beams—and vice versa—increases. This makes it easier for the test to pass and reduces the ability to validate the reliability of the beam prediction. Therefore, it is important to keep K small to maintain meaningful validation. Furthermore, these metrics only ensure the presence of the Top-1 beam within the set of predicted beams, which may limit the network’s ability to select the best beam, particularly when the top beam cannot be allocated. To better address this, update option 2 such that more than one strongest beam is part of the top-K predicted beams.

Observation 2: When K is large, the likelihood of the Top-1 strongest beam becoming part of the Top-K predicted beams increases.

Proposal 5: The value of K should be kept small to ensure more meaningful validation and reduce the likelihood of the Top-1 strongest beam easily fitting into the Top-K predicted beams.

RAN4 #114bis agreed on the success rate for correct predictions as follows: "A correct prediction is considered when the maximum ground-truth RSRP among the top-K predicted beams is greater than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) minus x dB." This implies that if the Top-1 strongest beam is one of the top-K predicted beams (i.e., Top-K/1 (%) = 100%), the value of x will be zero. It is crucial that the requirement ensures a high probability that more than one strongest beam is included in the top-K predicted beams, meaning the value of x should be defined as low as possible

Proposal 6: The value of N should be at least 2.

Proposal 7: RAN4 to define the value of x to be sufficiently stringent to ensure meaningful validation of beam prediction accuracy.

Relative RSRP accuracy

Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

RAN4 #114bis agreed on the definition of Relative RSRP accuracy for reported beams as follows:

Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

Relative accuracy requirement is intended for comparing multiple beams, which ensures that any measurement error in a single beam has less influence on the overall assessment. Comparing the predicted RSRP value of the strongest beam with that of all top-K predicted beams can provide a more robust method to mitigate prediction errors. In this context, the reported L1-RSRP of the strongest beam with index n can be predicted. Which can provide the relative accuracy of all top-K predicted beams with respect to the accuracy of the strongest predicted beam.

However, if the measured RSRP is considered as the reported L1-RSRP of the strongest beam with index n, then according to the above definition, the relative RSRP accuracy would reflect a comparison between the accuracy of the legacy measurement and the AI/ML-based beam prediction. This is not the intended purpose, as the metric is meant to assess the relative accuracy among predicted beams, not between predicted and measured values.

Proposal 8: The reported L1-RSRP of the strongest beam should be predicted, not measured.

UE Rx beam knowledge

To accurately predict the L1-RSRP of the Tx beams, it would be beneficial for the UE to know the Rx beam paired with each predicted Tx beam. However, if the UE does not have prior knowledge of which Rx beam to use for a new Tx beam direction (due to a change in the TCI state), the UE would need to detect and adapt its Rx beam to align with the new Tx beam. While having knowledge of the Rx beam paired with the predicted Tx beam would help improve prediction accuracy, this knowledge should be internal to the UE and its implementation.

Observation 3: An unknown TCI state change can result in increased delay during the state transition.

Proposal 9: TCI state associated with the predicted beam should be known.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: Increasing the inference delay to achieve better accuracy may lead to suboptimal system performance.

Observation 2: When K is large, the likelihood of the Top-1 strongest beam becoming part of the Top-K predicted beams increases.

Observation 3: An unknown TCI state change can result in increased delay during the state transition.

Proposal 1: Absolute RSRP accuracy requirement should be applied to all the top K predicted beams.

Proposal 2: The predicted RSRP accuracy requirement should be close to the legacy requirement to reduce the performance degradation due to AIML based BM.

Proposal 3: No need to define separate requirement for inference delay, it should be considered as a component of the overall reporting delay.

Proposal 4: RAN4 to define separate reporting delay requirement for BM case 1 and BM case 2.

Proposal 5: The value of K should be kept small to ensure more meaningful validation and reduce the likelihood of the Top-1 strongest beam easily fitting into the Top-K predicted beams.

Proposal 6: The value of N should be at least 2.

Proposal 7: RAN4 to define the value of x to be sufficiently stringent to ensure meaningful validation of beam prediction accuracy.

Proposal 8: The reported L1-RSRP of the strongest beam should be predicted, not measured.

Proposal 9: TCI state associated with the predicted beam should be known.

- References

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated, Wuhan, Hubei, China, 7th – 11th April, 2025

- R4-2505106, Ad hoc minutes on AI/ML air interface, Qualcomm Incorporated, Wuhan, Hubei, China, 7th – 11th April, 2025

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Rel-18, ver. 18.0.0.