3GPP TSG RAN WG4 Meeting #115	R4-2505481

St. Julian, Malta, 19th – 23rd May 2025

Source:		Rohde & Schwarz

Title:	Discussion on aperiodic ZP CSI scheduling for 16Tx and 32Tx PMI requirements

Agenda Item:	4.6

Document for:	Approval

# Introduction

Several PMI requirements in TS 38.101-4 [1] use aperiodic CSI scheduling. However, due to the configuration of the test case the usage of aperiodic ZP CSI is not possible in this case and the requirements need to be corrected.

# Discussion

Several PMI requirements in TS 38.101-4 [1] use aperiodic CSI scheduling, an example from clause “6.3.2.1.3 Multiple PMI with 16TX TypeI-SinglePanel Codebook” from TS 38.101-4 [1] is shown below:

Table 1: CSI configuration from table 6.3.2.1.3-1

| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
| --- | --- | --- | --- |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(9) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |

When further looking at the associated RMCs with these TCs in Annex A of 38.101-4 [1], e.g. table A.3.2.1.1-6 it can be seen that for slots containing CSI no DL data is scheduled.

Table 2 Table A.3.2.1.1-6: PDSCH Reference Channel for FDD PMI reporting requirements

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-6.1 FDD | R.PDSCH.1-6.2 FDD | R.PDSCH.1-6.3 FDD |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 15 | 15 | 15 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 13 | 20 |  |  |
| Modulation |  | 16QAM | 16QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.55 |  |  |
| Number of MIMO layer |  | 1 | 2 | 2 |  |  |
| Number of DMRS REs (Note 3) |  | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..19} | Bits | 12040 | 24072 | 40976 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..19} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A |  |  |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..,19} | CBs | 2 | 3 | 5 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Slots i = 10 | Bits | 23712 | 47424 | 71136 |  |  |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..9,11,…,19} | Bits | 24960 | 49920 | 74880 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 9.030 | 18.054 | 30.732 |  |  |

To trigger aperiodic ZP CSI, the ZP CSI-RS trigger needs to be send via DCI 1-1 according to TS 38.212 [2] chapter 7.3.1.2.2.

Observation 1: Triggering aperiodic ZP CSI-RS needs to happen via DCI 1-1.

This however is currently not possible, since sending a DCI 1-1 to trigger a ZP CSI-RS requires to simultaneously also schedule a PDSCH in the same slot with the same DCI transmission. Since according to the RMCs there is no PDSCH present in the slots with CSI, it is not possible to use the aperiodic ZP CSI-RS as per the requirements.

Observation 2: DCI 1-1 cannot be used with the current requirements, since no PDSCH is present in the slots with CSI.

To solve this problem and make the test cases implementable and testable, it is proposed to change the aperiodic ZP CSI-RS scheduling to a periodic scheduling, since this does not have the same issues and works fine for other PMI test cases. Even with the current aperiodic scheduling, the ZP CSI-RS would be sent periodically, so switching to periodic scheduling should not impact the test cases.

Proposal 1: Update the ZP CSI-RS scheduling to periodic to make the test cases functional.

Observation 3: With current aperiodic ZP CSI-RS scheduling the CSI-RS are scheduled periodically, so no impact on the test cases is expected when changing the ZP CSI-RS scheduling.

As per our analysis this problem affects all 16Tx and 32Tx PMI requirements mentioned in the following table.

Table 3 List of affected requirements

| 2Rx FDD |  |
| --- | --- |
| 6.3.2.1.3 | Multiple PMI with 16TX TypeI-SinglePanel Codebook |
| 6.3.2.1.4 | Single PMI with 32TX TypeI-SinglePanel Codebook |
| 6.3.2.1.5 | Multiple PMI with 16TX TypeII Codebook |
| 6.3.2.1.6 | Multiple PMI with 16TX Enhanced Type II Codebook |
| 4Rx FDD |  |
| 6.3.2.2.3 | Multiple PMI with 16TX TypeI-SinglePanel Codebook |
| 6.3.2.2.4 | Single PMI with 32TX TypeI-SinglePanel Codebook |
| 6.3.2.2.5 | Multiple PMI with 16TX TypeII Codebook |
| 6.3.2.2.6 | Multiple PMI with 16TX Enhanced Type II Codebook |
| 2Rx TDD |  |
| 6.3.3.1.3 | Multiple PMI with 16TX TypeI-SinglePanel Codebook |
| 6.3.3.1.4 | Single PMI with 32TX TypeI-SinglePanel Codebook |
| 6.3.3.1.5 | Multiple PMI with 16TX TypeII Codebook |
| 6.3.3.1.6 | Multiple PMI with 16TX Enhanced Type II Codebook |
| 4Rx TDD |  |
| 6.3.3.2.3 | Multiple PMI with 16TX TypeI-SinglePanel Codebook |
| 6.3.3.2.4 | Single PMI with 32TX TypeI-SinglePanel Codebook |
| 6.3.3.2.5 | Multiple PMI with 16TX TypeII Codebook |
| 6.3.3.2.6 | Multiple PMI with 16TX Enhanced Type II Codebook |

Corresponding CRs are provided in [3] and [4] to make the required changes to update the specification.

Proposal 2: Agree to the associated CRs R4-2505482 and R4-2505485 and update the specification.

# Summary

In this contribution identify an issue with aperiodic CSI scheduling in various requirements and propose a solution. We have the following observation and proposals:

Observation 1: Triggering aperiodic ZP CSI-RS needs to happen via DCI 1-1.

Observation 2: DCI 1-1 cannot be used with the current requirements, since no PDSCH is present in the slots with CSI.

Proposal 1: Update the ZP CSI-RS scheduling to periodic to make the test cases functional.

Observation 3: With current aperiodic ZP CSI-RS scheduling the CSI-RS are scheduled periodically, so no impact on the test cases is expected when changing the ZP CSI-RS scheduling.

Proposal 2: Agree to the associated CR R4-250xxxx and update the specification.

# References

[1]	TS 38.101-4 v16.20.0

[2]	TS 38.212 v16.14.0

[3]	R4-2505482, “(NR_eMIMO-Perf) Correction to ZP CSI configuration for PMI requirements”, Rohde & Schwarz, RAN4#115 May 2025, Malta

[4]	R4-2505485, “(NR_perf_enh-Perf) Correction to ZP CSI configuration for PMI requirements”, Rohde & Schwarz, RAN4#115 May 2025, Malta