3GPP TSG-RAN WG4 Meeting #115	R4-2505502

Valletta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on Rel-19 NTN Demod performance requirements

Agenda item:	7.31.7

Document for:	Discussion

Introduction

In this document Nokia presents some perspectives on the ongoing Rel-19 work item on NR NTN in Rel-19.

Some views are presented based on the Work Item Description, as this work is due to start at RAN4#115 [1].

Discussion

PDCCH

In Rel-19 improvements have been defined in RAN1 to improve the performance of the downlink, specifically focusing on PDCCH and PDSCH, this includes the following:

- Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

Improvements to DL Link Level are being defined in Rel-19 for NR NTN

As it is expected that these will have performance impact performance requirements should therefore be defined for these channels.

RAN4 shall define requirements for PDCCH with repetition

PDSCH

Similarly with PDSCH, link level improvements have been designed as part of the work item, thus will require new requirements.

RAN4 shall define requirements for PDSCH with Msg4 via PDSCH repetition

RAN 4 shall define requirements for PDSCH repetitions within 20 ms duration

OCC for DFT-s-OFDM PUSCH

In Rel-19 improvements have been defined in RAN1 to improve the performance of the uplink, specifically focusing on PUSCH this includes the following:

- Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

Improvements to UL Link Level are being defined in Rel-19 for NR NTN

Therefore new performance requirements will need to be defined for the OCC design.

RAN4 shall define new requirements for OCC PUSCH

Conclusion

The paper discusses the topic of NR NTN in RAN4 for performance requirements derivation, and presents some of the Nokia views on this topic for discussion during RAN4#115.

The following Observations and Proposals were made:

Observation 1: Improvements to DL Link Level are being defined in Rel-19 for NR NTN

Proposal 1: RAN4 shall define requirements for PDCCH with repetition

Proposal 2: RAN4 shall define requirements for PDSCH with Msg4 via PDSCH repetition

Proposal 3: RAN 4 shall define requirements for PDSCH repetitions within 20 ms duration

Observation 2: Improvements to UL Link Level are being defined in Rel-19 for NR NTN

Proposal 4: RAN4 shall define new requirements for OCC PUSCH

- References

- RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Thales, CATT, RAN#106, Madrid, December 2025