3GPP TSG-RAN4 Meeting #115		R4-2505510

# St Julian, Malta 19th – 23th May, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for AI/ML Beam Management

Agenda item:	7.21.4

Document for:	Discussion

# Introduction

This paper focuses on RRM core requirements for AI/ML based beam management.

RAN4 made the following agreements during the last few meetings [7][8][9][14]

Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-4: QCL Source RS for TCI states Agreement: wait for RAN1 discussion to conclude without sending any LS Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]. Issue 2-5: Requirements for beam predictions Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed - Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point. Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

RAN1 made the following agreements in [5][6]:

- RAN1#117: Working Assumption - Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study). - RAN1#118 agreement: For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list are associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

# Discussion

## 2.1 Selection of Beam Prediction Metrics

RAN4 made the following agreements during the last couple of meetings.

RAN4 114 Agreement If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options RAN4 114bis Agreement - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

RAN4 114 agreement ensures defining both beam prediction accuracy and RSRP accuracy metrics if RSRP and beam ID are reported. RAN4 114bis agreement, however, defined beam prediction accuracy metric when only beam IDs are reported. That means, without any further RAN4 agreement, UE will have to meet only RSRP accuracy requirements when both beam ID and RSRP are reported.

One of the main objectives of AI-ML beam prediction is to allow gNB being informed of the strongest ground truth beam with some accuracy without actually transmitting the corresponding reference signal. If UE only has to meet RSRP accuracy metrics, UE’s predicted top beam can be very far from the strongest ground truth beam. Even if UE meets L1-RSRP accuracy for its predicted top beam, the reported top 1 predicted beam might not be a suitable beam as a reference for changing TCI state and uplink spatial relationship.

Observation 1: RAN4 114 agreement ensures defining both beam prediction accuracy and RSRP accuracy metrics if RSRP and beam ID are reported. Based on RAN4 114bis agreement, UE will have to meet only RSRP accuracy requirements when both beam ID and RSRP are reported.

Observation 2: If UE only has to meet RSRP accuracy metrics, UE’s predicted top beam can be very far from the strongest ground truth beam. Even if UE meets L1-RSRP accuracy for its predicted top beam, the reported top-1 predicted beam might not be a suitable beam as a reference for changing TCI state and uplink spatial relationship.

Proposal 1: Extend following metric/KPI for beam ID prediction metric to the scenario when both beam ID and RSRP are reported.

- “The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values

- Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

## 2.2 TCI State Status of Predicted Tx Beam

Legacy spec defines the conditions for known TCI state in the following way [18]:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB

RAN4 agreed to the following during a couple of meetings ago.

Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

We focus on the requirement of UE’s RX beam knowledge during TCI state switch from two perspectives.

- To meet performance requirement of AI-ML based beam prediction

- Legacy spec requirement

RAN4 has agreed to define RSRP accuracy as a metric for AI-ML based beam prediction. For both temporal and spatial beam prediction, UE must know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam. If UE does not know its RX beam corresponding to the gNB Tx beam, UE’s predicted L1-RSRP RSRP will be drastically different from the actual L1-RSRP.

Observation 3: 3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. UE has to know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam.

RAN4 has also agreed to define beam prediction accuracy as a metric for AI-ML based beam prediction. UE has to predict the top beams of setA based on what UE would see at its baseband to pass beam prediction metrics. To compare between gNB beams, UE needs to have some idea of the Rx gain associated with them. Hence, UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Observation 4: To compare between gNB beams of setA, UE needs to have some idea of the Rx gain associated with them. UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Legacy spec defines CSI-RS L1-RSRP measurement period in the following way:

The value of TL1-RSRP_Measurement_Period_CSI-RS is defined in Table 9.5.4.2-1 for FR1 and in Table 9.5.4.2-2 for FR2, where -… - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set. - For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided with QCL-TypeD for all resources in the resource set in the MAC CE activating the resource set. - For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided qcl-info is configured for all resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.19.2 and 10.1.20.2 if number of resources in the resource set is smaller than maxNumberRxBeam. The requirements apply provided qcl-info is configured with QCL-TypeD for all resources in the resource set.

Observation 5: Legacy spec requires UE to know the RX beam of CSI-RS if it is configured without “repetition = ON” and if it is QCLed with SSB.

Figure 1: Spatial Beam Prediction

3GPP is focusing on two different scenarios for spatial beam prediction. They are shown in Figure 1 and mentioned below:

- Scenario 1:Wide-to-narrow beam prediction

- Scenario 2: Narrow-to-narrow beam prediction

Scenario 1 requires UE to predict properties of set A CSI-RS resources based on set B SSB measurement. The setA CSI-RS resources whose properties that UE needs to predict are QCLed with respect to type D with the setB SSBs that UE has measured. Hence, UE should have knowledge of, at least, the wide RX beam that it can use to receive the setB beams.

Scenario 2 requires UE to predict properties of a larger set of CSI-RS resources (setA) based on the measurement of a smaller set of CSI-RS resources (setB). Although both setB and setA have CSI-RS resources, any RAN4 test focusing on scenario 2 will also allow transmission of SSBs. If the CSI-RS resources of setA are not configured with “repetition = ON”, they are expected to be QCLed w.r.t. type D with SSB resources that UE can measure. UE can use its RX beam of SSB to estimate its RX beam for setA.

Observation 6: In “wide-to-narrow” beam prediction, the setA CSI-RS resources whose properties that UE needs to predict are QCLed wrt type D with the setB SSBs that UE has measured. In order to receive a CSI-RS of setA beams after TCI state switch, UE can use the wide RX beam that it needs to measure the SSB that is QCLed with the CSI-RS.

Observation 7: In “narrow-to-narrow” beam prediction, if SSB resources are configured during the test, UE can use the RX beams corresponding to the SSBs as the RX beam for the setA CSI-RS resources if following conditions are satisfied:

- CSI-RS resources are configured without “repetition = ON” and QCLed wrt type D with SSB that are present during the tests.

Proposal 2: After TCI state switches to a setA beam, spec mandates UE to “know” the RX beam of that setA beam in, at least, the following scenarios:

- In “wide-to-narrow” beam prediction

- In “narrow-to-narrow” beam prediction if SSB resources are configured during the tests

- Note 1: In both “wide-to-narrow” and “narrow-to-narrow” beam prediction, tests ensure the following:

  - setA CSI-RS resources are configured without “repetition = ON”,

  - setA CSI-RS resources are QCLed wrt type D with SSBs

- Note 2: In “wide-to-narrow” beam prediction, UE measures SSBs and predicts properties of setA CSI-RS resources. In “narrow-to-narrow” beam prediction, UE measures a subset of CSI-RS resources (setB) and predicts properties of full set (setA) of CSI-RS resources.

The “known” TCI state definition needs to be updated for AI-ML based CSI prediction reporting in the scenarios of proposal 2. Legacy spec defines a TCI state to be “known” if the reference signal corresponding to the TCI state was measured during the last 1280 ms. In AI-ML beam management, UE is not measuring setA beams. Rather, it is predicting the properties of setA beams. Hence, in the scenarios of proposal 2, a TCI state should be known if the properties of corresponding reference signal were predicted during the last Y ms.

Legacy spec requires the SNR of SSB corresponding to the reference signal of the TCI state to be >= -3 dB. For AI-ML BM, this threshold of -3 dB needs to be further evaluated. The threshold SNR depends on the performance of UE’s AI-ML beam prediction in that SNR.

Proposal 3:

The TCI state of a beam in setA is known in the scenarios of proposal 2 if the following conditions are met:

-	During the period from the last prediction-based CSI reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for prediction-based CSI reporting is the RS in target TCI state or QCLed to the target TCI state

-	TCI state switch command is received within X ms upon the last prediction-based CSI reporting

-	The SSB associated with the TCI state remains detectable during the TCI switching period

- SNR of the SSB associated with the TCI state ≥ Y dB

- FFS: Value of X and Y

## 2.3 Inference Timeline of AI-ML Beam Prediction

RAN4 agreed to the following regarding delay timeline of AI-ML based beam prediction.

Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS Agreement: - For measurement delay component of the prediction delay: - Use “legacy” L1-RSRP measurement as the baseline for Case 1 - FFS whether same values for M,N,P should be reused - FFS for case 2

RAN1 recently made the following agreement for processing a CSI report for inference [17]

Agreement - For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, considering the following options for potential down selection: - Option 1: only dedicated AI/ML PU is occupied, OAPU=N1 is reported by UE. - And OCPU=0 - Option 2: only legacy CPU is occupied, OCPU=M it is reported by UE. - Option 3: both dedicated AI/ML PU and legacy CPU are occupied, OAPU=N2 is reported by UE. - And OCPU=1 Note: The supported option by UE is reported by UE capability, if multiple options are supported. - The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

The inference delay is dependent on the model complexity, e.g., the number of parameters of the AI-ML model. These numbers may depend on the particular model design and hardware architecture. The design of model and hardware architecture may vary from UE to UE. Hence, RAN4 should not define a fixed value for inference delay of AI-ML based beam prediction. Rather, UE capability should be introduced so that UE can indicate its supported value for inference delay of AI-ML based beam prediction.

UE may need to use different AI-ML models between beam management case 1 (“spatial only” beam prediction) and case 2 (“temporal” beam prediction). Hence, UE’s required time to infer beam might be different between beam management case 1 and case 2. Separate UE capabilities should be introduced for inference delay of these two features.

Also, UE’s required model complexity may depend on CSI report configuration. Since inference delay is inherently tied to model complexity, 3GPP will have to evaluate the dependence of inference delay with CSI report configuration.

RAN1 has started discussing the minimum time separation between setB resource and CSI-RS report containing AI-ML based prediction of setA resources. RAN4 can utilize the conclusion of that discussion to define inference delay of AI-ML based beam prediction.

Observation 8: The inference delay is dependent on the model complexity and hardware architecture. The design of model and hardware architecture may vary from UE to UE. Hence, 3GPP should not define a fixed value for inference delay of AI-ML based beam prediction.

Observation 9: UE may need to use different AI-ML models between beam management case 1 (“spatial only” beam prediction) and case 2 (“temporal” beam prediction). Hence, UE’s required time to infer beam might be different between beam management case 1 and case 2

Observation 10: Complexity of the required AI-ML model depends on CSI report configuration, too.

Observation 11: RAN1 has started discussing the minimum time separation between setB resource and CSI-RS report containing AI-ML based prediction of setA resources. RAN4 can utilize the conclusion of that discussion to define inference delay of AI-ML based beam prediction.

Proposal 4: RAN4 waits until RAN1’s conclusion on minimum time separation between setB resources and corresponding report containing predicted properties of setA to decide inference delay.

# Conclusions

Observation 1: RAN4 114 agreement ensures defining both beam prediction accuracy and RSRP accuracy metrics if RSRP and beam ID are reported. Based on RAN4 114bis agreement, UE will have to meet only RSRP accuracy requirements when both beam ID and RSRP are reported.

Observation 2: If UE only has to meet RSRP accuracy metrics, UE’s predicted top beam can be very far from the strongest ground truth beam. Even if UE meets L1-RSRP accuracy for its predicted top beam, the reported top-1 predicted beam might not be a suitable beam as a reference for changing TCI state and uplink spatial relationship.

Observation 3: 3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. UE has to know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam.

Observation 4: To compare between gNB beams of setA, UE needs to have some idea of the Rx gain associated with them. UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Observation 5: Legacy spec requires UE to know the RX beam of CSI-RS if it is configured without “repetition = ON” and if it is QCLed with SSB.

Observation 6: In “wide-to-narrow” beam prediction, the setA CSI-RS resources whose properties that UE needs to predict are QCLed wrt type D with the setB SSBs that UE has measured. In order to receive a CSI-RS of setA beams after TCI state switch, UE can use the wide RX beam that it needs to measure the SSB that is QCLed with the CSI-RS.

Observation 7: In “narrow-to-narrow” beam prediction, if SSB resources are configured during the test, UE can use the RX beams corresponding to the SSBs as the RX beam for the setA CSI-RS resources if following conditions are satisfied:

- CSI-RS resources are configured without “repetition = ON” and QCLed wrt type D with SSB that are present during the tests.

Observation 8: The inference delay is dependent on the model complexity and hardware architecture. The design of model and hardware architecture may vary from UE to UE. Hence, RAN4 should not define a fixed value for inference delay of AI-ML based beam prediction.

Observation 9: UE may need to use different AI-ML models between beam management case 1 (“spatial only” beam prediction) and case 2 (“temporal” beam prediction). Hence, UE’s required time to infer beam might be different between beam management case 1 and case 2

Observation 10: Complexity of the required AI-ML model depends on CSI report configuration.

Proposal 1: Extend following metric/KPI for beam ID prediction metric to the scenario when both beam ID and RSRP are reported.

- “The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values

- Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Proposal 2: After TCI state switches to a setA beam, spec mandates UE to “know” the RX beam of that setA beam in, at least, the following scenarios:

- In “wide-to-narrow” beam prediction

- In “narrow-to-narrow” beam prediction if SSB resources are configured during the tests

- Note 1: In both “wide-to-narrow” and “narrow-to-narrow” beam prediction, tests ensure the following:

  - setA CSI-RS resources are configured without “repetition = ON”,

  - setA CSI-RS resources are QCLed wrt type D with SSBs

- Note 2: In “wide-to-narrow” beam prediction, UE measures SSBs and predicts properties of setA CSI-RS resources. In “narrow-to-narrow” beam prediction, UE measures a subset of CSI-RS resources (setB) and predicts properties of full set (setA) of CSI-RS resources.

Proposal 3:

The TCI state of a beam in setA is known in the scenarios of proposal 2 if the following conditions are met:

-	During the period from the last prediction-based CSI reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for prediction-based CSI reporting is the RS in target TCI state or QCLed to the target TCI state

-	TCI state switch command is received within X ms upon the last prediction-based CSI reporting

-	The SSB associated with the TCI state remains detectable during the TCI switching period

- SNR of the SSB associated with the TCI state ≥ Y dB

- FFS: Value of X and Y

Proposal 4: RAN4 waits until RAN1’s conclusion on minimum time separation between setB resources and corresponding report containing predicted properties of setA to decide inference delay.

# References

- R4-2414446, ‘AI/ML ad-hoc meeting minutes’, Moderator (Qualcomm)

- R4-2414323, ‘WF on AI/ML’, Qualcomm

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP TSG RAN #105.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #118.

- 38.133, Requirements for support of radio resource management, V 17.11.0.

- R4-2417212, WF on Requirements for AI/ML air interface, 3GPP TSG-RAN WG4 #112bis.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 #118bis.

- R1-2308259, “Evaluations on AI/ML for Beam Management”, Qualcomm Incorporated, 3GPP TSG-RAN WG1 #114.

- R4-2417211, “Simulation assumptions for beam prediction”, 3GPP TSG-RAN WG4 #112bis

- R4-2420513, WF on AI/ML air interface part 1, RAN4 #113.

- R4-2420480, Updated Simulation Assumptions for Beam Prediction, RAN4 #113.

[14] R4-2502856, “WF on the requirements for AI/ML air interface”, 3GPP TSG-RAN WG4 Meeting#114

[15] R4-2505105, “WF on requirements for AI/ML air interface normative work”, 3GPP TSG-RAN WG4 Meeting#114bis

[16] R4-2505152, “Updated simulation assumptions for AI/ML BM”, 3GPP TSG-RAN WG4 Meeting#114bis

[17] 3GPP TSG WG RAN1 Chairman’s notes, RAN1 #120bis

[18] 38.133 V 18.7.0, Requirements for support of radio resource management.