3GPP TSG-RAN WG4 Meeting #115                                           R4-2505520

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on UE RF testability issues for Rel-19 LP-WUS

Agenda Item:	7.28.3.4

Document for:	Approval

Introduction

Some progress was made in previous RAN4 meeting on testability issues for LP-WUS [1].

Issue 4-1-1: Whether LR can be tested directly

Agreements:

  - Down select below two approaches for LP-WUR UE RF requirements:

  - Option 1: stick to 1% MDR LP-WUS as performance metric, but with a note in TS that LP-WUR ON is not mandated.

    - The note depends on RAN1 conclusion

  - Option 2: update it to 1% MDR of PDCCH triggered by LP-WUR

Issue 4-1-4: UE test mode for LP-WUR

Agreements:

  - Test mode design as well as the details of test mode for LP-WUS verification can be left to RAN5 with RAN4 inputs.

In this contribution, we will focus on two issues, i.e. Test under different RRC states and Configurations of LP-WUS signal for Rx requirements, which were not discussed in detail or address in past meetings.

Discussion

Test under different RRC states

From the study in RAN1, LP-WUS can operate in both RRC_IDLE/INACTIVE and RRC_CONNECTED modes. It is noted that the process for LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode are different. In addition, supporting IDLE/INACTIVE mode and CONNECTED mode is based on UE capabilities, which means a UE supporting LP-WUS operation in IDLE/INACTIVE mode does not necessarily to support operation in CONNECTED mode, and vice versa.

The UE capabilities can be found in Annex, which are cited from RAN1 feature list for LP-WUS [2].

Since the RF test is not the functionality test, ideally the test under one mode is sufficiently to verify the RF capability, and most likely UE would support IDLE/INACTIVE mode for better utilizing the WUS mechanism in terms of power saving. However, we are not so sure that RAN1 could provide confirmable feedback that all UE supporting LP-WUS should support the feature under IDLE/INACTIVE mode. Therefore, to cover the completed operation scenarios, test cases should be designed for both IDLE/INACTIVE mode and CONNECTED mode.

Observation 1: LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode depends on UE capability, which means UE may not support both modes.

Proposal 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different. If both IDLE/INACTIVE mode and CONNECTED mode are supported by the LR, which mode is selected for test should be up to UE declaration.

Configurations of LP-WUS signal for Rx requirements

Currently, for all NR Rx requirements - except spurious emissions, which are tested with the maximum CBW - all other requirements are tested with the lowest, middle, and highest CBWs supported by the UE. However, LP-WUS is fixed to 11 PRBs regardless of the SCS, so the choice of NR carrier CBW for LP-WUR testing has minimal impact. Additionally, some requirements, such as ACS, ASCS, and blocking, are tested in the presence of an interferer, while others are tested without an interferer but with an NR signal padded in the same carrier, such as SE; REFSENS, by contrast, is tested without an NR signal. Thus, the configurations of the LP-WUS signal for Rx requirements should be carefully considered. The following is the preliminary thinking of the configurations.

Table 1: Test configurations for LP-WUR

| Requirements | Test Configurations |
| --- | --- |
| REFSENS | - 5MHz, 15kHz SCS - LP-WUS is configured at center of the carrier - without NR signals in the same carrier |
| Maximum input level | - 5MHz, 15kHz/30kHz SCS - LP-WUS is configured at center of the carrier - without NR signals in the same carrier |
| ACS | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at edge of the carrier - without NR signals in the same carrier |
| ASCS | - L/M/H CBWs, 15kHz/30kHz SCS - LP-WUS is configured at center of the carrier - with adjacent NR signals padded in the same carrier |
| In-band blocking | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at edge of the carrier - without adjacent NR signals padded in the same carrier |
| Out-of-band blocking | - LP-WUS is configured at center of the carrier - without adjacent NR signals padded in the same carrier |
| Narrow band blocking | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at edge of the carrier - without adjacent NR signals padded in the same carrier |
| Spurious response | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at edge of the carrier - without adjacent NR signals padded in the same carrier |
| Wide band Intermodulation | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at center of the carrier - without adjacent NR signals padded in the same carrier |
| Spurious emissions | - Maximum CBW, 15kHz/30kHz SCS - LP-WUS is configured at center of the carrier - with adjacent NR signals padded in the same carrier |

Proposal 2: Test configurations in Table 1 could be considered as starting point for further discussion.

Conclusion

In this paper we provide our consideration on the framework of LP-WUS with following observations and proposals.

Observation 1: LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode depends on UE capability, which means UE may not support both modes.

Proposal 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different. If both IDLE/INACTIVE mode and CONNECTED mode are supported by the LR, which mode is selected for test should be up to UE declaration.

Proposal 2: Test configurations in Table 1 could be considered as starting point for further discussion.

Table 1: Test configurations for LP-WUR

| Requirements | Test Configurations |
| --- | --- |
| REFSENS | - 5MHz, 15kHz SCS - LP-WUS is configured at center of the carrier - without NR signals in the same carrier |
| Maximum input level | - 5MHz, 15kHz/30kHz SCS - LP-WUS is configured at center of the carrier - without NR signals in the same carrier |
| ACS | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at edge of the carrier - without NR signals in the same carrier |
| ASCS | - L/M/H CBWs, 15kHz/30kHz SCS - LP-WUS is configured at center of the carrier - with adjacent NR signals padded in the same carrier |
| In-band blocking | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at edge of the carrier - without adjacent NR signals padded in the same carrier |
| Out-of-band blocking | - LP-WUS is configured at center of the carrier - without adjacent NR signals padded in the same carrier |
| Narrow band blocking | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at edge of the carrier - without adjacent NR signals padded in the same carrier |
| Spurious response | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at edge of the carrier - without adjacent NR signals padded in the same carrier |
| Wide band Intermodulation | - 10MHz, 15kHz/30kHz SCS - LP-WUS is configured at center of the carrier - without adjacent NR signals padded in the same carrier |
| Spurious emissions | - Maximum CBW, 15kHz/30kHz SCS - LP-WUS is configured at center of the carrier - with adjacent NR signals padded in the same carrier |

- References

[1] R4-2505234, WF on NR LP-WUS UE requirements, vivo

[2] R4-2505303, LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#120bis

- Annex

RAN1 feature list for LP-WUS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 5 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |