3GPP TSG-RAN4 Meeting #115 	R4-2505541

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title:  	Ambient IoT general aspects

Agenda Item:	7.26.2

Document for:	Approval

- Introduction

The WI on solutions for Ambient IoT in NR was approved at RAN#106 [1] and revised at RAN#107[2]. Last meeting, a WF was agreed [3]. In this contribution, we further discuss the open issues and give our proposals on system parameters and other general aspects of A-IoT.

- Discussion

  - D2R Channel bandwidth

Copied from [3]

Issue 2-2-1: D2R bandwidth

Agreement:

- Define D2R channel bandwidth for A-IoT device: (RAN4#114bis, R4-2505230)

- Based on 2SB transmission,

- Consider SFO impact

- Consider small frequency shift defined in RAN1

- D2R transmission bandwidth (2 times of 1SB), consider bit rate defined in RAN1

For reference, here's what Tb, Tc, R, Btx, D2R represent and their relationship:

Copied from [4]

Background

For D2R transmission, the following parameters and relationships are considered:

- Tb: Time duration corresponding to a bit, where the bit is that after FEC if applied.

- Tc: Chip duration, where Tb = Tc×2R.

- R: SFS factor, where the small frequency shift location is determined by ± (R/Tb = 1/2/Tc) Hz

- Btx,D2R: D2R transmission bandwidth, where Btx,D2R = 4/Tb = 2/(R*Tc).

RAN1 progress in RAN1#120bis are as follows：

Copied from [5]

Agreement

The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table:

- FFS further down-selections of Tchip, Tb, and R to be supported

- Note: Detailed indication signaling of D2R scheduling information is discussed in AI 9.4.4.

|  |  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
|  | 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
|  | 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
|  | 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
|  | 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
|  | 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
|  | 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
|  | 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
|  | 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
|  | 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
|  | 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
|  | 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
|  | 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
|  | 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
|  | 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |
| Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The 2SB D2R transmission bandwidth Btx,D2R is 2/ (R*chip duration). The set of candidate values of D2R chip duration Tchip, bit duration Tb(=2R*Tchip), and SFS factor R are as the table. As the notes stated that bit duration Tb 266.67μs, 133.33μs and 66.67μs to be kept, and it is expected to down-select between bit duration Tb 1.39μs and 1.04μs. We take the bit duration 266.67μs, 133.33μs and 66.67μs, 1.39μs and 1.04μs as examples to analysis D2R channel bandwidth.

For R=1, the maximum Tc is 133.33 μs, corresponding to the minimum transmission BW of 15kHz (=2*1000/133.33), which is for low data rate and enhanced coverage applications.

For R=1, the minimum Tc is to be down selected between 0.69 μs and 0.52μs, corresponding to the maximum transmission BW, this case is for high data rate and capacity applications. 0.69μs corresponds to the maximum transmission BW 2880kHz (=2*1000/0.69) . 0.52μs corresponds to the maximum transmission BW 3840kHz (=2*1000/0.52).

Take Tchip = 133.33 μs as an example, the small frequency shift of FDMA can be demonstrated as the following figure. Note that only some of the R values are showed in the diagram.

Figure 1 Small frequency shift without SFO diagram of R = 1, 4, 8, 16, 32 with Tchip = 133.33 μs

The small frequency shift is defined as the offset between the 2SB carrier's center frequency and the shifted single-side carrier's center frequency.

Small Shift frequency=±（R/Tb）Hz=±（R* Btx,D2R /4）Hz

For R=1 (where no small frequency shift occurs), SFS=R*Btx,D2R/4= Btx,D2R/4, this represents a zero-frequency shift special case.

For R > 1: SFS=R*Btx,D2R/4

Figure 2 Small frequency shift diagram of FDMA without SFO

The impact of SFO can be assumed as the extension or the shortening of the chip length. Both D2R Transmission bandwidth and small frequency shift values are related with chip duration. Consequently, the SFO affects D2R channel bandwidth in two ways:

- SFO Alters D2R transmission bandwidth to Btx,D2R×(1±∣SFO∣)

- SFO additionally scales the frequency shift value to (R* Btx,D2R /4)* (1 ±∣SFO∣)

The figure of the SFO impact of FDMA is demonstrated as the following Figure 3. For convenience, Figure 2 is copied to compare:

Figure 2  Small frequency shift diagram of FDMA without SFO

Figure 3. SFO impact of transmission bandwidth and small frequency shift of FDMA

With the worst SFO assumption of 105 PPM for Device 1, D2R channel BW from device perspective can be calculated as follows:

D2R Channel bandwidth= Frequency shift * 2*1.1 + (2SB transmission bandwidth *1.1)/2

=((R*2SB transmission bandwidth)/4）* 2*1.1 + (2SB transmission bandwidth *1.1)/2

=(1.1*(R+1) * 2SB Transmission BW)/2

Note: The 1.1 factor accounts for 10% SFO effect.

SFO can either extent or reduce the D2R channel bandwidth. However, from a design standpoint, only the extention of the D2R channel bandwidth is calculated.

Observation 1: From device perspective, consider small frequency shift and SFO (assumed as 10^5 ppm) impact, D2R CBW can be calculated as follows:

D2R Channel BW device = (1.1*(R+1) * 2SB Transmission BW without SFO)/2

The 1.1 factor accounts for guard band due to 10% SFO effect.

On the device side, no filter is needed since device just backscatter CW signal. However, on BS side, the receiver use analogy and/or digital filter with guard band to filter wanted signals from interference. We propose using a 90% filter spectrum utility (with 10% guard band) as a preliminary assumption.

From A-IoT BS perspective, consider FMDA small frequency shift and SFO (assumed as 10^5ppm) impact, and anti-interference filter, D2R CBW can be calculated as follows:

D2R Channel BW Reader = (1.1*(R+1) * 2SB Transmission BW without SFO)/2/0.9

The 1.1 factor accounts for guard band due to 10% SFO effect.

The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band)

Observation 2: From A-IoT BS perspective, consider FMDA small frequency shift and SFO (assumed as 10^5ppm) impact, and anti-interference filter, D2R CBW can be calculated as follows:

D2R Channel BW Reader = (1.1*(R+1) * 2SB Transmission BW without SFO)/2/0.9

The 1.1 factor accounts for guard band due to 10% SFO effect.

The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band)

The possible CBW values for device and reader are calculated in Table 1.

Table 1 D2R CBW Calculation

| A | B | C=A*B/4 | D=1.1*(A+1)*B/2(from device perspective) | E=D/0.9 |
| --- | --- | --- | --- | --- |
| R Value | D2R 2SB Transmission BW（kHz） | Freq shift(kHz) | Calculated channel BW for device with 10% SFO (kHz) | Calculated channel BW for BS with 10% SFO and 90% BS receiver filter spectrum utility (kHz) |
| 1 | 15 | 3.75 | 16.5 | 18 |
| 2 | 15 | 7.5 | 24.8 | 28 |
| 4 | 15 | 15 | 41.3 | 46 |
| 8 | 15 | 30 | 74.3 | 83 |
| 12 | 15 | 45 | 107.3 | 119 |
| 16 | 15 | 60 | 140.3 | 156 |
| 24 | 15 | 90 | 206.3 | 229 |
| 32 | 15 | 120 | 272.3 | 303 |
| 48 | 15 | 180 | 404.3 | 449 |
| 64 | 15 | 240 | 536.3 | 596 |
| 96 | 15 | 360 | 800.3 | 889 |
| 128 | 15 | 480 | 1064.3 | 1183 |
| 256 | 15 | 960 | 2120.3 | 2356 |
| 1 | 30 | 7.5 | 33 | 37 |
| 2 | 30 | 15 | 49.5 | 55 |
| 4 | 30 | 30 | 82.5 | 92 |
| 6 | 30 | 45 | 115.5 | 128 |
| 8 | 30 | 60 | 148.5 | 165 |
| 12 | 30 | 90 | 214.5 | 238 |
| 16 | 30 | 120 | 280.5 | 312 |
| 24 | 30 | 180 | 412.5 | 458 |
| 32 | 30 | 240 | 544.5 | 605 |
| 48 | 30 | 360 | 808.5 | 898 |
| 64 | 30 | 480 | 1072.5 | 1192 |
| 96 | 30 | 720 | 1600.5 | 1778 |
| 128 | 30 | 960 | 2128.5 | 2365 |
| 1 | 60 | 15 | 66 | 73 |
| 2 | 60 | 30 | 99 | 110 |
| 4 | 60 | 60 | 165 | 183 |
| 6 | 60 | 90 | 231 | 257 |
| 8 | 60 | 120 | 297 | 330 |
| 12 | 60 | 180 | 429 | 477 |
| 16 | 60 | 240 | 561 | 623 |
| 24 | 60 | 360 | 825 | 917 |
| 32 | 60 | 480 | 1089 | 1210 |
| 48 | 60 | 720 | 1617 | 1797 |
| 64 | 60 | 960 | 2145 | 2383 |
| 1 | 2880 | 720 | 3168 | 3520 |
| 1 | 3840 | 960 | 4693 | 4224 |

Figure 4 shows potential D2R CBW diagram taking 133.33μs chip duration of R = 1, 4, 8, 16, 32 and without SFO. The dashed line shows the potential BS filters. The potential D2R channel BW become too many when accounting for combinations of transmission bandwidth, chip duration, and FDMA small frequency shift.

Figure 4 D2R CBW diagram example for R = 1, 4, 8, 16, 32 with Tchip = 133.33 μs and without SFO

If define all calculated channel BWs separately, there would be 79 possible CBW configurations. While RAN1 is expected to reduce this number, the final selection likely won’t be drastically narrowed down. The main challenge with having so many CBW option for BS is the design complexity—for example, requiring 79 different filter sets. This would not only complicate filter design but also network configuration.

Observation 3: Defining all calculated D2R channel BWs separately would complicate BS filter design and increase network configuration complexity.

Given the complexity of supporting numerous potential channel bandwidths (CBWs) as the above table, we recommend limiting D2R CBW variants to streamline both Network configuration processes and A-IoT base station receiver filter design efficiency.

Since the minimum R2D channel bandwidth has been agreed as 200 kHz, we propose setting the minimum D2R channel bandwidth (CBW) to the same value (200 kHz). The maximum D2R CBW and other D2R CBW can be determined based on RAN1's final decision.

Proposal 1: Define minimum D2R CBW (200 kHz) and maximum D2R CBW (depending on RAN1’s decision). Other D2R CBW FFS.

  - R2D Channel raster

Copied from [3]

Agreement:

- Consider following options for R2D channel raster(RAN4#114bis, R4-2505230)

  - Option 1: Define 10KHz as R2D channel raster (i.e. same as enhanced channel raster)

  - Option 2: Define channel raster considering the RB orthogonality between macro BS and A-IoT BS

  - Option 3: Define 100KHz as R2D channel raster

  - Other options are not precluded.

The channel raster determines the minimum frequency step for carrier center frequency configuration.

In NR FR1 bands, different channel raster values (e.g., 100 kHz, 15 kHz, 10 kHz,) are designed for specific deployment scenarios.

100 kHz channel raster is inherited from LTE, mainly used for NR bands re-farmed from LTE ones.Default supported for most FR1 bands

15 kHz channel raster is SCS based channel raster, it Aligns with LTE’s 15 kHz subcarrier spacing (SCS). Ensures SS/PBCH block (SSB) alignment with LTE CRS for interference avoidance.

10 kHz channel raster enhancement is introduced in Rel 18, to address configuring a narrower UE channel BW inside a wider gNB channel BW (WID RP-230813): 10 kHz applied to bands below 3 GHz that support 100 kHz channel raster: Optimizes spectrum efficiency in fragmented/low-bandwidth deployments.

Follows we discuss why100kHz channel raster is not suitable for A-IoT R2D, while10 kHz channel raster enhancement can be reused for A-IoT R2D.

Both A-IoT R2D and NR can be Odd-PRB or Even-PRB carriers. For Odd-PRB carriers, Transmission bandwidth center located between subcarriers #5 and #6(center of one PRB), center frequency position: 90kHz × (2m + 1)

For Even-PRB carriers, Transmission bandwidth center located between two PRBs，center frequency position is 180kHz × n

Channel Spacing between two carriers:
90kHz × (2m + 1 - 2n) = 90kHz × (2p + 1), where p = m - n

For channel raster of 100kHz，Requires (2p + 1) × 90kHz = q × 100kHz

However, there is no integer solution to (2p + 1) × 90kHz = q × 100kHz

Example: A-IoT 200kHz R2D CBW in 25MHz NR bandwidth fails PRB alignment

For channel raster of 10kHz， Requires (2p + 1) × 90kHz = q × 10kHz

It can be seen that 10kHz channel raster to enable proper PRB alignment

Proposal 2: R2D Channel raster from A-IoT BS perspective is 10 kHz.

Figure 5 Diagram of channel raster offset

AIoT may be deployed in the NR spectrum.

Figure 4 indicates Channel Raster and Transmission BW center location for NR carrier.

For Odd-PRB NR carriers, Channel Raster (Red Arrow) is centered on subcarrier #6; Transmission BW Center is at the Boundary between subcarriers #5 and #6;

For Even-PRB NR carriers, Channel Raster (Red Arrow) is centered on subcarrier #0; Transmission BW Center is at the Boundary between subcarriers #11 and #0.

An offset is necessary to align the channel raster (red Arrow) with the transmission BW center (blue Arrow)

Filters typically are symmetric designs, as asymmetry degrades performance. The carrier center frequency (Channel Raster Position )need to align with the transmission bandwidth center (Blue Arrow). This can be achieved by applying a raster offset of either -7.5 kHz or +2.5 kHz.

Proposal 3: R2D channel raster offset can be -7.5kHz or +2.5kHz.

  - Foffset

As described in TS36.104, for NB-IoT standalone operation, NB-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset. A-IoT with one PRB transmission bandwidth is with similar narrow bandwidth, it can be applied to A-IoT.

In section 2.1, the ACLR for OOK-4 OFDM signal (both pre- and post-filter, excluding PA degradation) is significantly worse than NB-IoT. This necessitates frequency offset for A-IoT operation.

Figure 6 Diagram of A-IoT ACLR Foffset

Proposal 4: Apply the Foffset for Ambient IoT as Table 6:

Table 2: Foffset for Ambient IoT

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Ambient IoT for R2D CBW 200/400/600/800kHz | 100 kHz |
| Note：Foffset is start from A-IoT Channel Bandwidth edge |  |

- Summary

In this contribution, we discuss key open issues regarding A-IoT general aspects and present the following observations and proposals:

Observation 1: From device perspective, consider small frequency shift and SFO (assumed as 10^5 ppm) impact, D2R CBW can be calculated as follows:

D2R Channel BW device = (1.1*(R+1) * 2SB Transmission BW without SFO)/2

The 1.1 factor accounts for guard band due to 10% SFO effect.

Observation 2: From A-IoT BS perspective, consider FMDA small frequency shift and SFO (assumed as 10^5ppm) impact, and anti-interference filter, D2R CBW can be calculated as follows:

D2R Channel BW Reader = (1.1*(R+1) * 2SB Transmission BW without SFO)/2/0.9

The 1.1 factor accounts for guard band due to 10% SFO effect.

The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band)

Observation 3: Defining all calculated D2R channel BWs separately would complicate BS filter design and increase network configuration complexity.

Proposal 1: Define minimum D2R CBW (200 kHz) and maximum D2R CBW (depending on RAN1’s decision). Other D2R CBW FFS.

Proposal 2: R2D Channel raster from A-IoT BS perspective is 10 kHz.

Proposal 3: R2D channel raster offset can be -7.5kHz or +2.5kHz.

Proposal 4: Apply the Foffset for Ambient IoT as Table 2:

Table 2: Foffset for Ambient IoT

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Ambient IoT for R2D CBW 200/400/600/800kHz | 100 kHz |
| Note：Foffset is start from A-IoT Channel Bandwidth edge |  |

- References

[1]RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei), RAN#106 Dec.,2024

[2]RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc.，RAN#107 Mar.,2025

[3] R4-2505230, WF on A-IOT device requirements, CMCC, RAN4#114bis, April, 2025

[4] R1-2503020, Summary #1 for coding aspects of physical channel design, RAN1#120bis, April, 2025

[5] R1-2503112, Session notes for 9.4 Solutions for Ambient IoT in NR, RAN1#120bis, April, 2025