3GPP TSG-RAN WG4 Meeting #115                                        R4-2505587

Malta, May 19 - 23, 2025

Source: 	Huawei, HiSilicon

Title: 	On SRS IL imbalance issue

Agenda Item:	7.1.2.3.4

Document for:	Approval

Introduction

During RAN4#114bis, SRS IL imbalance issue was finally making progress. Following contents are reproduced from [1].

Issue 1: Whether to solve SRS IL imbalance issue in Rel-19 Way forward: - No further discussion on the issue in future meetings and focus on identification of possible SRS IL imbalance issue solutions. - NOTE: This does not mean that any solution for SRS IL imbalance issue is specified in Rel-19 Issue 2: SRS IL solution applicability to 2Rx/4Rx/6Rx/8Rx Agreement - FFS on the potential SRS IL solution applicability to 2Rx/4Rx/8Rx UEs in addition to 6Rx UEs Issue 3: SRS IL imbalance solution as optional feature Agreement - If any new requirements or procedures for SRS IL handling are defined in Rel-19, they will be defined as optional UE features Way forward - RAN4 to further discuss the candidate SRS IL imbalance solution framework based on agreements for issues 4-2-1 and 4-2-2. - Companies are encouraged to provide views on additional aspects including - SRS output power testing - Enhancements to PCmax equations to improve SRS IL compensation Issue 4: SRS IL imbalance UE self-compensation Agreement: - RAN4 assumptions on UE behavior for Rel-19 SRS IL handling: - For SRS transmissions Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power - FFS if any additional clarifications on behavior are needed - No RAN1/4 specification impacts (i.e. this behavior is aligned with existing specifications) Issue 5: SRS IL imbalance UE assistance and reporting Agreement: - The following two possible general solutions may be considered for further consideration - Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed - Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility - Further check whether solution 2 is in the scope or not - Considerations of any solutions should be based on system level throughput evaluations, and implementation complexity

In this contribution, we would like to share our views regarding this topic and also UE feature list for 6Rx.

Discussion

- Examination on the two candidate solutions

Given that RAN4 would be more focus on the candidate solutions, we would like to provide our views accordingly. As clearly defined in TS 38.213, the actual power delivered per SRS occasion is the minimum value between PCMAX and another calculated one, and it should be clarified that the PCMAX utilized here is a configured value according to the definition in TS 38.101-1 instead of the range [PCMAX, L, PCMAX, H].

If a UE transmits SRS based on a configuration by SRS-ResourceSet on active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, the UE determines the SRS transmission power PSRS,b,f,c(i,qs,l) in SRS transmission occasion i as [dBm] where, - PCMAX,f,c(i) is the UE configured maximum output power defined in [8, TS 38.101-1], [8-2, TS 38.101-2] and [TS 38.101-3] for carrier f of serving cell c in SRS transmission occasion i - PO_SRS,b,f,c(qs) is provided by p0 for active UL BWP b of carrier f of serving cell c and SRS resource set qs provided by SRS-ResourceSet and SRS-ResourceSetId --- Omit the rest part for simplicity ---

Observation 1: For PSRS calculation as defined in TS 38.213, PCMAX is a value determined by UE according to the definition in TS 38.101-1, where ∆TRxSRS has already been counted respective to each SRS occasion, but not the range [PCMAX, L, PCMAX, H] itself.

Look closely at the PSRS definition, the network can do power control based on SRS resource set level configuration along with SRS power control adjustment state for each SRS occasion:

- Target power PO_SRS,b,f,c

- Scaling factor αSRS,b,f,c for the estimated path loss of each SRS resource set

- Power up or down controlled by hb,f,c for each SRS occasion

Moreover, PSRS equals to PCMAX as the UE determined for each SRS occasion if it is the smaller one. Thus we think SRS transmission power determination can be categorised as following cases.

| Case #1 | For all SRS resources within the set, none of PSRS equals to PCMAX | The estimated path loss could be relatively small, e.g. cell centre |
| --- | --- | --- |
| Case #2 | For all SRS resources within the set, some of PSRS equals to PCMAX | The estimated path loss could still be not so high while some of the diversity branches become MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. mildly away from cell centre |
| Case #3 | For all SRS resources within the set, all of PSRS equals to PCMAX | The estimated path loss could be high and all diversity branches become MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge |

In our understanding, at least Case #3 is the target scenario to consider SRS IL reporting because it is not the network duty to consider compensation for the path loss.

Observation 2: SRS transmission power determination can be categorised as following cases.

| Case #1 | For all SRS resources within the set, none of PSRS equals to PCMAX | The estimated path loss could be relatively small, e.g. cell centre |
| --- | --- | --- |
| Case #2 | For all SRS resources within the set, some of PSRS equals to PCMAX | The estimated path loss could still be not so high while some of the diversity branches become MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. mildly away from cell centre |
| Case #3 | For all SRS resources within the set, all of PSRS equals to PCMAX | The estimated path loss could be high and all diversity branches become MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge |

Observation 3: The following scenario can be considered as the target for SRS IL reporting:

- For all SRS resources within the set, all of PSRS equals to PCMAX

  - In this scenario, the estimated path loss could be high and all diversity branches become MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge

So for the candidate solution 2, the trigger event for such type 3 PHR would be quite hard to define. At least we can identify following factors that should be considered:

- The timing (or duration) whenever partial or all antenna ports for AS-SRS transmission become MOP limited

- Changing on the mapping between SRS port and Tx chain due to e.g. hand blocking

We understand such trigger event cannot be clearly defined without specification impact.

Observation 4: The trigger event for reusing PHR type 3 to resolve SRS IL reporting is hardly to be defined without specification impact because at least following factors should be considered:

- The timing (or duration) whenever partial or all antenna ports for AS-SRS transmission become MOP limited

- Changing on the mapping between SRS port and Tx chain due to e.g. hand blocking

Proposal 1: The specification impact for defining the trigger event under candidate solution 2 i.e. reusing PHR type 3 shall be clarified.

As for the candidate solution 1, we think this could be a way that can keep the current UE implementation untouched and avoid specification impact. But the question is what the additional information is that can be beneficial to the network. Per our thinking, the following details can be further considered with the intention that such indication would benefit the network by adjusting expectation on the antenna switching SRS based PMI estimation.

Proposal 2: Further clarifications on the candidate solution 1:

- UE is allowed to indicate its capability on handling SRS IL, which includes two states and the UE shall not indicate them simultaneously

- State 1: the applied ∆TRxSRS shall be smaller than the respective minimum requirements

- State 2: the applied ∆TRxSRS equals to the respective minimum requirements

- SRS IL solution applicability

For this issue, we think the bone of contention lies in the applicable UE type but rather Rx number. Because the main concern from companies is about overexposure or even impact to the UE implementation due to such reporting. Consequently, limitation to non-handheld UE could be a workaround if eventually RAN4 can adopt one solution in this late release of NR. However, RAN4 can consider to preclude 2Rx since it should have the least imbalance thus the expected gain from network perspective is relatively negligible.

Observation 5: 2Rx can be precluded since it should have the least imbalance thus the expected gain from network perspective is relatively negligible.

- UE feature list for 6Rx

For 6Rx WI, we propose to have following UE features based on RAN4 discussions so far.

Proposal 3: Adopt the following UE feature list for Rel-19 WI NR_ENDC_RF_Ph4 objective 6Rx.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 46-6 | Support maximum 6 MIMO layers for DL reception | Indicates whether the UE supports maximum 6 spatial multiplexing layers for DL reception for FWA. |  | Yes | N/A | UE does not support maximum 6 MIMO layers for DL reception | Per FSPC | N/A | Yes (only FR1) | N/A |  | Optional |
|  | 46-7 | SRS antenna switching for 6Rx | Indicates supporting of t3r6 SRS antenna switching. |  | Yes | N/A | UE does not support t3r6 SRS antenna switching | Per BC | N/A | Yes (only FR1) | N/A | This capability cannot be indicated simultaneously with Tx diversity | Optional |
|  | 46-8 | [SRS IL imbalance compensation] | Indicates whether UE performs SRS IL self-compensation | 2-55 or 23-8-3 | Yes | N/A |  | Per BC | N/A | Yes (only FR1) | N/A |  | Optional |

Conclusion

In this contribution we discussed on the SRS IL reporting. We have the following observations and proposals:

Observation 1: For PSRS calculation as defined in TS 38.213, PCMAX is a value determined by UE according to the definition in TS 38.101-1, where ∆TRxSRS has already been counted respective to each SRS occasion, but not the range [PCMAX, L, PCMAX, H] itself.

Observation 2: SRS transmission power determination can be categorised as following cases.

| Case #1 | For all SRS resources within the set, none of PSRS equals to PCMAX | The estimated path loss could be relatively small, e.g. cell centre |
| --- | --- | --- |
| Case #2 | For all SRS resources within the set, some of PSRS equals to PCMAX | The estimated path loss could still be not so high while some of the diversity branches become MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. mildly away from cell centre |
| Case #3 | For all SRS resources within the set, all of PSRS equals to PCMAX | The estimated path loss could be high and all diversity branches become MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge |

Observation 3: The following scenario can be considered as the target for SRS IL reporting:

- For all SRS resources within the set, all of PSRS equals to PCMAX

  - In this scenario, the estimated path loss could be high and all diversity branches become MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge

Observation 4: The trigger event for reusing PHR type 3 to resolve SRS IL reporting is hardly to be defined without specification impact because at least following factors should be considered:

- The timing (or duration) whenever partial or all antenna ports for AS-SRS transmission become MOP limited

- Changing on the mapping between SRS port and Tx chain due to e.g. hand blocking

Observation 5: 2Rx can be precluded since it should have the least imbalance thus the expected gain from network perspective is relatively negligible.

Proposal 1: The specification impact for defining the trigger event under candidate solution 2 i.e. reusing PHR type 3 shall be clarified.

Proposal 2: Further clarifications on the candidate solution 1:

- UE is allowed to indicate its capability on handling SRS IL, which includes two states and the UE shall not indicate them simultaneously

- State 1: the applied ∆TRxSRS shall be smaller than the respective minimum requirements

- State 2: the applied ∆TRxSRS equals to the respective minimum requirements

Proposal 3: Adopt the following UE feature list for Rel-19 WI NR_ENDC_RF_Ph4 objective 6Rx.

- References

- R4-2505101, “WF on 6Rx RF requirements and SRS IL imbalance”, Intel, 3GPP TSG-RAN4 Meeting #114bis, Wuhan, China, April 7-11, 2025.