3GPP TSG-RAN WG4 # 115	R4-2505595

Saint Julian’s, Malta, 19th May 2025 – 23rd May 2025

Agenda item:	7.16.3

Source:	MediaTek Inc.

Title:	TP to TR38.753: TDL approaches and related Annex

Document for:	Approval

# Introduction

This TP handles sections of “TDL approaches” and “Annex B TDL approaches”.

# Reference

[1]	RP-241610 SID: Study on spatial channel model for demodulation performance requirements

# Text proposal to TR 38.753

The following TP to TR 38.753 is proposed for approval.

# -------------------------------------------------- Start of text proposal --------------------------------------------------

# 5.2 TDL Approaches

The novel multi-cluster TDL approach builds on top of the widely used and well tested legacy RAN4 TDL channel models. In the legacy models, spatial selectivity is introduced by applying MIMO correlation matrices on top of uncorrelated fading MIMO channel. However, the legacy correlation models (TS 38.101-4: Annex B.2.3) amplify the signal only in the broadside direction (AOD/AOA) of the TX and RX antenna arrays. As a result, the spatial degrees of freedom remain limited, and only a relatively low number of spatial layers can be supported. In order to support a higher number of layers, multiple separate spatial signal directions should exist. This can be achieved by introducing spatial clusters, where each cluster has its own separable mean AOD/AOA.

The multi-cluster TDL models aim to be an easier-to-use and easier-to-configure alternative to CDL models. There is just a handful of high-level parameters that can be chosen to achieve desired channel properties, and to match the scope of different performance tests. To further improve the test coverage, the spatial properties of the clusters (in terms of AOA/AOD) could be defined to be time-varying.

# 5.2.1 MIMO correlation and angular spread

For both LTE and NR, RAN4 demodulation and CSI reporting tests until 3GPP Release 18 employ TDL fading channel models. In many of those tests, spatial selectivity is introduced by applying MIMO correlation matrices on top of an uncorrelated fading MIMO channel. Correlation models are defined for two different antenna array assumptions: Uniform Linear Array (ULA) and Cross-polarized Antennas (X-pol).

MIMO correlation (TDL) and angular spread (CDL) approaches are alternative ways to generate spatial selectivity. High angular spread corresponds to low MIMO correlation and vice versa. As can be seen in Figure 5-1, a typical 3GPP correlation model generates spatial selectivity that resembles that of a single-cluster CDL channel model. As a result of the real-valued spatial correlation matrix, the strongest beam direction is always at 0⁰ AOD/AOA angle (broadside) of each linear array dimension.

The spatial selectivity of a MIMO channel can be depicted as power angular distribution (PAD) that represents the average channel power response as function of TX or RX direction. Figure 5-1(a) illustrates the power angular distribution (PAD) measured from an 8TX-8RX X-pol medium correlated TDL channel model (α1=0.3, β=0.6). As can be seen, this channel favors the AODs and AOAs close to zero degrees. The PAD at the TX side is slightly wider than at the RX side because the TX-correlation is lower than the RX-correlation (α < β). Similarly, Figure 5-1(b) shows the PAD measured from a single-cluster 8TX-8RX CDL channel model with mean AOA = 0 degrees. Here, the angular spread is at the TX side CASD = 1.3710 and at the RX side CASA = 15.632 degrees, and the assumed antenna element radiation patterns is flat.

- (b)

Figure 5-1: Spatial selectivity of X-pol correlated TDL channel (a) and single-cluster CDL channel (b).

# 5.2.2 Multi-cluster TDL channel via TX-RX beam steering

The multi-cluster TDL channel model re-uses and extends the concept of beam steering that has earlier been specified for RAN4 PMI reporting tests ([X] TS 38.101-4: B.2.3.2.3 and B.2.3.2.3A). In TX-RX beam steering, the average AOA and AOD present in a correlated TDL MIMO channel are shifted to desired directions by post-processing. Note that beam steering is a part of the channel model, it is not an action taken by the transmitter or the receiver.

An example of multi-cluster channel is illustrated in Figure 5-2.

Figure 5-2: TX-RX beam steering channel with two spatial clusters.

The multi-cluster MIMO channel model is a combination of a number of spatial clusters, where each cluster is associated with a group of statistically independent time-varying channel taps employing some existing spatial correlation model. For each cluster, TX and RX steering corresponding to desired AOD/AOA angles is defined.

The general multi-cluster MIMO channel impulse response matrix with dimensions Nrx× Ntx at some time instant is

Hτ=n=1Nm=1Mnpn,m∙Dn(rx)Hn,mDn(tx)∙δτ-τn,m

where

- n is the cluster index, m is the delay tap index, and τ is the delay variable

- N is the number of clusters, and Mn is the number of delay taps in cluster n

- pn,m is the power of delay tap m of cluster n so that nmpn,m=1

- τn,m is the propagation delay of tap m of cluster n

- Hn,m is the spatially correlated fading MIMO channel matrix of delay tap m of cluster n with unit average power response and dimensions Nrx× Ntx

- Dn(rx) is the diagonal RX steering matrix of cluster n with dimensions Nrx× Nrx

- Dn(tx) is the diagonal TX steering matrix of cluster n with dimensions Ntx× Ntx

- δτ is the Dirac delta function

# 5.2.2.1 Steering matrices for cross-polarized antenna arrays

Steering parameters with cross-polarized (X-pol) antenna array assumption:

- gNB TX-array dimensions:

  - Ntx1 and Ntx2 are the number of antenna elements with the same polarization in the first and the second array dimension.

  - Ntx=2∙Ntx1∙Ntx2

- UE RX-array dimensions:

  - Nrx1 and Nrx2 are the number of antenna elements with the same polarization in the first and the second array dimension. Note that 3GPP X-pol model for TDL channels (TS 38.101-4 Annex B.2.3.2) supports only 1D RX-arrays: Nrx2=1.

  - Nrx=2∙Nrx1∙Nrx2

- TX phases: θtx1(n) and θtx2(n) per cluster n

- RX phases: θrx1(n) and θrx2(n) per cluster n

Steering matrix definitions:

- Dn(tx)=1001⊗Dθtx1(n),Ntx1⊗Dθtx2(n),Ntx2

- Dn(rx)=1001⊗Dθrx1(n),Nrx1⊗Dθrx2(n),Nrx2

- The steering matrix of size N× N as function of phase θ is defined as

  - Dθ,N=diag1ejθ⋯ejθ(N-1)

  - Assuming the antenna arrays employ uniform distance between elements in each linear dimension, the phase shift θ corresponds to AOA or AOD angle φ so that θφ,dλ=2π∙dλ∙sinφ, where d is the distance between adjacent antenna elements and λ is the carrier wavelength.

  - The steering phases θtx1(n), θtx2(n), θrx1(n), θrx2(n) may be specified either directly or by specifying the corresponding directional angles φtx1(n), φtx2(n), φrx1(n), φrx2(n).

The beam steering phases can be either constant or (slowly) time-varying. In general, the model parameters can be chosen to match the scope of each test case. For example, the number of clusters can be selected so that the desired number of layers (rank) is supported. Optionally, cluster-specific Doppler shifts and additional propagation delays can be specified.

# 5.2.2.2 Cluster model 1

In the first clustering option, each cluster is associated with a TDL channel model with full delay profile, such as RAN4 TDL-A, TDL-B, or TDL-C so that Mn=12 for each n. Thus, the complexity of the channel model grows with the number of clusters N. Cluster powers and tap powers have the relation pn,m=pn∙pmTDL, where pmTDL represents the power of tap m in the underlying TDL model. Furthermore, the tap delays τn,m=τmTDL are drawn directly from the underlying TDL model.

An example parameterization of a 3-cluster TDL channel is shown in Table 5-1. Assuming 8TX-8RX array setup with (Ntx1, Ntx2, Nrx1, Nrx2)= (2, 2, 4, 1), the steering phases both at TX and RX side are chosen so that the TX and RX beams become mutually orthogonal between clusters. Orthogonality between the beams maximizes the spatial multi-layer capacity of the channel for a given set of cluster powers. Similarly, the cluster power distribution affects the spatial channel properties. For a set of orthogonal beams, assigning equal power for all clusters results in the highest channel capacity.

Table 5-1: Cluster model 1 with 3 clusters

| TDL profile | Cluster index | Power [dB] | θtx1 | θtx2 | θrx1 |
| --- | --- | --- | --- | --- | --- |
| TDL-A 30 / XP-high | 1 | 0 | 0 | 0 | 0 |
| TDL-A 30 / XP-high | 2 | -3 | -π | 0 | π/2 |
| TDL-A 30 / XP-high | 3 | -6 | -π | -π | π |

# 5.2.2.3 Cluster model 2

In the second clustering option, the delay profile of a single TDL channel model is split into multiple clusters. As a result, the implementation complexity of the model remains almost equal to the underlying TDL model as the total number of delay taps n=1NMn=12, regardless of the number of clusters. Here, the channel tap delays and powers are drawn directly from the underlying TDL model.

Table 5-2 shows an example of TDL-C model with two clusters. Here, symmetric steering phases between TX and RX are assumed so that θtx1=θrx1, and θtx2=θrx2.

Table 5-2: Cluster model 2 with 2 clusters

| TDLC 300 delay profile |  |  |  | TX-RX beam steering |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | Cluster index | θtx1 | θtx2 | θrx1 | θrx2 |
| 1 | 0 | -6.9 | 1 | 0 | 0 | 0 | 0 |
| 2 | 65 | 0 |  |  |  |  |  |
| 3 | 70 | -7.7 |  |  |  |  |  |
| 4 | 190 | -2.5 |  |  |  |  |  |
| 5 | 195 | -2.4 | 2 | π | π | π | π |
| 6 | 200 | -9.9 |  |  |  |  |  |
| 7 | 240 | -8.0 |  |  |  |  |  |
| 8 | 325 | -6.6 |  |  |  |  |  |
| 9 | 520 | -7.1 |  |  |  |  |  |
| 10 | 1045 | -13.0 |  |  |  |  |  |
| 11 | 1510 | -14.2 |  |  |  |  |  |
| 12 | 2595 | -16.0 |  |  |  |  |  |

# 5.2.2.4 Cluster model 3

In the third clustering option, the delay profiles of two TDL channel models are split into multiple clusters. As a result, the implementation complexity of the model is twice that of the underlying TDL model, and the total number of delay taps n=1NMn=24, regardless of the number of clusters. The channel tap delays and powers are drawn directly from the underlying TDL model, while the two TDL models may have a further power scaling.

Table 5-3 shows an example parameterization of cluster model 3 with two TDL-C channels split to 4 clusters.

Table 5-3: Cluster model 3 with 2 TDL-C channels and 4 clusters

| TDLC 300 delay profile |  |  | TX-RX beam steering |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | TDL channel 1 Power: 0 dB |  |  | TDL channel 2 Power: -9 dB |  |  |
|  |  |  | Cluster index | θtx1 | θrx1 | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 | 3 | -π | -π |
| 2 | 65 | 0 |  |  |  |  |  |  |
| 3 | 70 | -7.7 |  |  |  |  |  |  |
| 4 | 190 | -2.5 |  |  |  |  |  |  |
| 5 | 195 | -2.4 | 2 | -π/2 | -π/2 | 4 | -3π/2 | -3π/2 |
| 6 | 200 | -9.9 |  |  |  |  |  |  |
| 7 | 240 | -8.0 |  |  |  |  |  |  |
| 8 | 325 | -6.6 |  |  |  |  |  |  |
| 9 | 520 | -7.1 |  |  |  |  |  |  |
| 10 | 1045 | -13.0 |  |  |  |  |  |  |
| 11 | 1510 | -14.2 |  |  |  |  |  |  |
| 12 | 2595 | -16.0 |  |  |  |  |  |  |

# Annex B

# B.1 TDL Approaches

# B.1.1 Multi-cluster TDL channel for 2-CW fixed-MCS testing

As one example, testing of two-codeword (up to 8 layers) MIMO demodulation – so that consistently different channel quality per CW is maintained – has turned out to be challenging with current RAN4 correlation models. This section presents examples how to construct PDSCH demodulation tests with fixed rank, fixed Type I PMI, and different fixed MCS per CW.

# B.1.1.1 Cluster model 1

For fixed PMI testing, the per-cluster phase shifts: θtx1(n) and θtx2(n) can be chosen to be fixed and to match the beams of the transmit PMI. In Type I codebook, each CW often maps to a subset of the orthogonal PMI-beams. This is illustrated below for the 5-layer precoder (TS 38.214: 5.2.2.2.1). Consequently, per-CW channel quality levels can be adjusted by adjusting the per-cluster powers pn.

To minimize cross-layer/CW interference at the UE, the RX-phases θrx1(n) can be chosen so that the corresponding steering vectors are orthogonal. Alternatively, by choosing non-orthogonal RX steering vectors, cross-layer/CW interference can be increased if so desired.

Finally, the selection of the X-pol spatial correlation model parameters (α1, α2, β) determines how strictly the mean spatial directions are maintained. Higher correlation is analogous to narrower angular spread per cluster.

To demonstrate the concept, a rank 5 PDSCH simulation setup in a three-cluster channel setup is detailed in Table B-1, and the simulation results are depicted in Figure B-1. Here, CW0 employs 16QAM while CW1 has QPSK, but as clusters corresponding to CW0 are 6dB stronger, the BLER performances of the two CWs are almost on par. Thus, it can be concluded that CW-specific TX-RX beam steering clusters can effectively be used to scale CW-specific demodulation and decoding performance.

Table B-1: Simulation setup (cluster model 1)

| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | 2, 2, 4, 1 |
| --- | --- |
| Rank5 Type1 TX-PMI: i1,1, i1,2, i2 1st dim PMI beam indexes: l, l’, l” 2nd dim PMI beam indexes: m, m’, m” | 0, 0, 0 0, O1, O1 => phase 2πl/O1N1 = 0, π, π 0, 0, O2 => phase 2πm/O2N2 = 0, 0, π |
| Cluster powers: p1, p2, p3 | 4/6, 1/6, 1/6 (beam 1 is 6dB stronger than beams 2 and 3) |
| Cluster TX BS in 1st array dim: θtx1(1), θtx1(2), θtx1(3) Cluster TX BS in 2nd array dim: θtx2(1), θtx2(2), θtx2(3) | 0, -π, -π 0, 0, -π |
| Cluster RX BS: θrx1(1), θrx1(2), θrx1(3) | 0, π/2, π |
| TDL channel model per cluster | TDLA-30 / 10Hz Doppler / X-pol high correlation |
| MCS / CW0 MCS / CW1 | MCS13, 16-QAM, code rate 0.48 MCS7, QPSK, code rate 0.51 |

Figure B-1: 2-CW fixed MCS case in three-cluster TDL-A channel.

# B.1.1.2 Cluster model 2

Cluster model 2 can satisfy the 2-CW fixed MCS testing requirements by setting mapping relation between clusters and taps to control the 2CWs power and delay difference in the reasonable level. Take channel model 2 in Table B-2 as an example, cluster 3 and cluster 4 correspond to last four taps, leading to large power and delay difference with cluster 1 and 2. Specific precoder mapping the 1st CW to first two clusters and 2nd CW to second two clusters leads to large SINR difference between two CWs. In the case, different MCSs could be configured to match performance difference.

Table B-2: Cluster model 2 with 4 TX-RX clusters

| TDLC 300 delay profile |  |  | TX-RX beam steering |  |  |
| --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 |
| 2 | 65 | 0 |  |  |  |
| 3 | 70 | -7.7 | 2 | -π/2 | -π/2 |
| 4 | 190 | -2.5 |  |  |  |
| 5 | 195 | -2.4 |  |  |  |
| 6 | 200 | -9.9 |  |  |  |
| 7 | 240 | -8.0 |  |  |  |
| 8 | 325 | -6.6 |  |  |  |
| 9 | 520 | -7.1 | 3 | -π | -π |
| 10 | 1045 | -13.0 |  |  |  |
| 11 | 1510 | -14.2 | 4 | -3π/2 | -3π/2 |
| 12 | 2595 | -16.0 |  |  |  |

A simulation is performed to verify the characteristic with simulation assumptions captured in table B-3, Rank 8 is assumed it can be seen that with fixed precoder i1,1=i1,2=i2=0, Single panel I, 2nd cluster and 3rd cluster are mapped to last four taps with low power and large delay, leading to big performance difference between two codewords.

Table B-3: Simulation setup (cluster model 2)

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1) for 8Tx. (1 dimension antenna array) |
| Precoder | Single panel I, Fixed precoder i1,1=i1,2=i2=0 |
| Rank | 8 |
| Antenna configuration | 8T8R |
| Channel | Cluster model 2 with 4 clusters, XPL high for each cluster, TDLC300-100 |
| MCS | 13 |

# -------------------------------------------------- End of text proposal --------------------------------------------------