3GPP TSG-RAN WG4 Meeting #115	R4-2505596

Saint Julian’s, Malta, 19th May 2025 – 23rd May 2025

Agenda item:	7.21.3

Source:	MediaTek Inc.

Title:	Discussion on CSI reporting requirement framework for CSI prediction

Document for:	Discussion

# Introduction

In RAN4#114bis, RAN4 agreed to continue discussion on CSI reporting requirement framework for CSI prediction issues [1]. Also, updated simulation assumptions for AI/ML CSI prediction were agreed [2]. In this contribution, we provide our views and simulation results on CSI reporting requirement framework for CSI prediction.

# Discussion

In this Chapter, we provide our views on CSI prediction requirement issues and share simulation results.

Way forward on simulation assumptions for CSI prediction was agreed in the previous meeting [2]. In Table 1 we list used simulation parameters. Compared to assumptions, we have limited our simulation scope to FDD 10MHz bandwidth with 15kHz SCS. We provide simulation results for all agreed Dopplers (20Hz, 50Hz, and 100Hz) and both agreed TX configurations (32 (N1, N2) = (4, 4), and 16 (N1, N2) = (4, 2)). Datasets are generated with link level simulator. Training datasets consist of channel realizations of all agreed Dopplers separately or mixed. Testing datasets consist of independent channel realizations of all agreed Dopplers (i.e. channel realizations are different from those used for model training).

Companies were encouraged to provide the model complexity information of their AI/ML CSI prediction models to help alignment. For this evaluation purpose we have used AI models that have <10k parameters and require <2 MFLOPs per inference for both 16Tx and 32Tx model.

Table 1: Simulation parameters for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Carrier frequency | 2GHz for FDD |
| Bandwidth | 10MHz for FDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD |
| Subcarrier spacing | 15kHz for 2GHz |
| gNB TX antennas | 32 (N1, N2) = (4, 4) as defined in Rel-15 type1 16 (N1, N2) = (4, 2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Doppler spread | 20Hz, 50Hz, 100Hz |
| Delay spread | 30ns |
| Channel estimation | Ideal DL channel estimation |
| Rank per UE | Rank 1 and 2 |
| CSI feedback assumption | Check the CSI prediction performance of raw channel per layer Check the CSI prediction performance with Rel-18 codebook quantization |
| KPI | Average of SGCS for intermediate results over all subbands per layer (Comparing the SVD of model output with SVD of the ground truth Raw channel and comparing the predicted Rel-18 quantized precoder with the ground truth Rel-18 quantized precoder) |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training and validation datasets | Set1: 16TX containing 20Hz Doppler ideal estimates Set2: 16TX containing 50Hz Doppler ideal estimates Set3: 16TX containing 100Hz Doppler ideal estimates Set4: 16TX containing 20Hz, 50Hz, and 100Hz Doppler ideal estimates (mixed) Set5: 32TX containing 20Hz Doppler ideal estimates Set6: 32TX containing 50Hz Doppler ideal estimates Set7: 32TX containing 100Hz Doppler ideal estimates Set8: 32TX containing 20Hz, 50Hz, and 100Hz Doppler ideal estimates (mixed) |
| Testing datasets | Set1: 16TX containing only 20Hz Doppler ideal estimates Set2: 16TX containing only 50Hz Doppler ideal estimates Set3: 16TX containing only 100Hz Doppler ideal estimates Set4: 32TX containing only 20Hz Doppler ideal estimates Set5: 32TX containing only 50Hz Doppler ideal estimates Set6: 32TX containing only 100Hz Doppler ideal estimates |

In the following Table 2 we share our average of SGCS for intermediate results over all subbands per layer for 3 different measures. Firstly, we have non-predicted (sample and hold) raw channel SGCS. Secondly, we have predicted raw channel SGCS. Finally, we have predicted Rel-18 quantized precoder SGCS where comparison is done to ground truth Rel-18 quantized precoder.

Table 2: Average of SGCS for intermediate results over all subbands per layer

| Test | Training Doppler [Hz] | Testing Doppler [Hz] | Channel sample and hold [SGCS per layer] | Channel prediction [SGCS per layer] | Rel-18 precoder prediction [SGCS per layer] |
| --- | --- | --- | --- | --- | --- |
| FDD 16TX | 20 | 20 | [0.795, 0.650] | [0.999, 0.999] | [0.992, 0.985] |
|  | 50 | 50 | [0.325, 0.187] | [0.969, 0.944] | [0.955, 0.922] |
|  | 100 | 100 | [0.301, 0.166] | [0.750, 0.572] | [0.821, 0.675] |
|  | Mixed (20, 50, 100) | 20 | [0.795, 0.650] | [0.957, 0.923] | [0.941, 0.898] |
|  |  | 50 | [0.325, 0.187] | [0.872, 0.777] | [0.873, 0.781] |
|  |  | 100 | [0.301, 0.166] | [0.666, 0.456] | [0.717, 0.533] |
| FDD 32TX | 20 | 20 | [0.814, 0.670] | [0.999, 0.998] | [0.990, 0.982] |
|  | 50 | 50 | [0.391, 0.201] | [0.972, 0.946] | [0.961, 0.924] |
|  | 100 | 100 | [0.309, 0.160] | [0.743, 0.561] | [0.836, 0.690] |
|  | Mixed (20, 50, 100) | 20 | [0.814, 0.670] | [0.954, 0.911] | [0.944, 0.894] |
|  |  | 50 | [0.391, 0.201] | [0.881, 0.770] | [0.891, 0.787] |
|  |  | 100 | [0.309, 0.160] | [0.664, 0.444] | [0.749, 0.563] |

From these results we can make the following observations.

Observation #1: FDD 16TX and FDD 32TX results with the same Doppler are quite similarly.

Observation #2: In lower Doppler, accuracy is better than in higher Doppler.

Observation #3: Rel-18 codebook quantization performance is close to raw channel prediction performance.

Test setup for FDD with Periodic CSI-RS configuration (based on R.PDSCH.1-24.1 FDD)

Note 1:	SS/PBCH block is transmitted in slot #0 with periodicity 20 ms
Note 2:	Slot i is slot index per 2 frames
Note 3:	Number of DMRS REs includes the overhead of the DM-RS CDM groups without data
An example for FDD test setup with Periodic CSI-RS configuration

In the updated simulation assumptions for AI/ML CSI prediction [2] there is a proposal for test setup for FDD with Periodic CSI-RS configuration based on R.PDSCH.1-24.1 FDD fixed reference channel. We see this proposal as a good and simple starting point for the next meeting Step-3 simulations where PDSCH throughput will be evaluated with codebook quantization. We see that first focusing to more flexibly configurable FDD to find test configuration with feasible precoder prediction gains would be more efficient working procedure. With observations from FDD experiments we can implement corresponding TDD test.

Observation #4: In simulation assumptions there exists sufficient test setup for FDD with Periodic CSI-RS configuration.

Proposal #1: We propose focusing to proposed test setup for FDD with Periodic CSI-RS configuration based on R.PDSCH.1-24.1 FDD as priority in Step-3.

In Step-3 we will start using practical channel estimation and we need to make assumptions of training dataset SNR. We propose to either list SNR range of training dataset or at minimum define some guidelines how to generalize dataset for SNR. This can potentially help companies to align with results and avoid overfitted training dataset.

Proposal #2: We propose to collectively define SNR range of training dataset for Step-3 experiments.

# Conclusion

In this contribution we provide the view on the CSI reporting requirement framework for CSI prediction issues. The following observations and proposals are made:

Observation #1: FDD 16TX and FDD 32TX results with the same Doppler perform quite similarly.

Observation #2: In lower Doppler accuracy is better than in higher Doppler.

Observation #3: Rel-18 codebook quantization performance is close to raw channel prediction performance.

Observation #4: In simulation assumptions there exists sufficient test setup for FDD with Periodic CSI-RS configuration.

Proposal #1: We propose focusing to proposed test setup for FDD with Periodic CSI-RS configuration based on R.PDSCH.1-24.1 FDD as priority in Step-3.

Proposal #2: We propose to collectively define SNR range of training dataset for Step-3 experiments.

# Reference

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm inc.

- R4-2505229, Updated simulation assumptions for AI/ML CSI prediction, OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, CMCC, CTC, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple