3GPP TSG-RAN WG4 Meeting #115	R4-2505598

Saint Julian’s, Malta, 19th May 2025 – 23rd May 2025

Agenda item:	7.24.3

Source:	MediaTek Inc.

Title:	Discussion on NR MIMO Phase 5 demodulation and CSI requirements

Document for:	Discussion

# Introduction

To address the issue of coverage, higher downlink spectrum efficiency, larger antenna arrays for single transmission point transmissions with an increased number of antennas have an increased interest in the industry, partly due to new available frequency bands. Enhancements to existing specifications that support both hybrid beamforming (e.g. with one beam for each DL transmission) and fully digital gNB implementation options are needed to maintain the cost-effectiveness vs performance tradeoff in implementation.  In current specifications, the support for such large antenna arrays with large number of CSI-RS ports for CSI measurement and reporting is limited. It is therefore justified to increase the number of ports for CSI reporting (up to 128 CSI-RS ports) and also to enhance hybrid beamforming to multi-beam reporting (extended up to 32 ports per reported beam) with PMI/CQI/RI per beam in order to extend hybrid beamforming to larger antenna arrays and increased MU-MIMO scheduling opportunities. [1]

# Discussion

In work item description of NR MIMO Phase 5 there is the following RAN1 objective for CSI support for up to 128 CSI-RS antenna ports.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

In work item description of NR MIMO Phase 5 there is the following RAN4 performance objective.

- Specify necessary performance requirements for the enhancements listed above.

In updated RAN1 UE features list for Rel-19 NR [2] there are several UE capabilities for CSI support for up to 128 CSI RS antenna ports. Note, support for each number of antenna ports has own dedicated capability.

UE capabilities of extended codebooks for CSI support for up to 128 CSI RS antenna ports - Enhanced Type-I SP codebook for [48, 64, 128] ports – Scheme-A - Enhanced Type-I SP codebook for [48, 64, 128] ports – Scheme-B - Enhanced Type-I MP codebook for [48, 64, 128] ports - Extended Rel-16 eType-II codebook for [48, 64, 128] Tx ports - Extended Rel-17 FeType-II codebook with [48, 64] Tx ports - Extended Rel-18 eType-II Doppler codebook for [48, 64, 128] Tx ports

In previous releases RAN4 has specified requirements for Type-I SP codebook, Rel-16 eType-II codebook and Rel-18 eType-II Doppler codebook. From these codebooks Type-I SP codebook is the most deployed codebook in live networks by far. Correspondingly, in previous releases RAN4 has not specified requirements for Type-I MP codebook and Rel-17 FeType-II codebook. Enhanced Scheme-A Type-I SP codebook, for Ranks 1 to 4, is straightforward extension from legacy Rel-15 Type-I SP codebook. Therefore, we consider this as the most potential use case. In addition, we see some increasing market interest towards Rel-16 eType-II codebook, and therefore, we see Extended Rel-16 eType-II codebook as credible secondary priority for test. We see that defining only PMI requirements for new Rel-19 CSI codebooks is sufficient.

Observation #1: In previous releases RAN4 has specified requirements for Type-I SP codebook, Rel-16 eType-II codebook and Rel-18 eType-II Doppler codebook.

Observation #2: Type-I SP codebook is the most deployed codebook in live networks.

Observation #3: In previous releases RAN4 has not specified requirements for Type-I MP codebook and Rel-17 FeType-II codebook.

Observation #4: Enhanced Scheme-A Type-I SP codebook, for Ranks 1 to 4, is straightforward extension from legacy Rel-15 Type-I SP codebook.

Proposal #1: We propose not to introduce CQI, Rank, or CRI requirements for Rel-19 CSI codebooks supporting for up to 128 CSI-RS ports.

Proposal #2: We propose to introduce PMI requirements for Rel-19 CSI codebooks supporting for up to 128 CSI-RS ports.

Proposal #3: We proposed to prioritize Enhanced Scheme-A Type-I SP codebook for PMI requirements.

Proposal #4: We proposed to consider Extended Rel-16 eType-II codebook for PMI requirements as secondary priority.

Proposal #5: We proposed to not consider Enhanced Scheme-B Type-I SP, Enhanced Type-I MP, Extended Rel-17 FeType-II, and Extended Rel-18 eType-II Doppler codebooks for PMI requirements.

Proposal #6: We propose to focus on introducing PMI requirements for Ranks ≤ 4.

New enhanced and extended codebooks introduce variants with different number of antenna ports. Available options are 48, 64, and 128 antenna ports, except FeType-II codebook do not support 128 antenna ports.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-1-1 | Enhanced Type-I SP codebook for 64 ports – Scheme-A | Support of enhanced Type-I SP codebook for Scheme-A with 64 Tx ports by aggregating multiple NZP CSI-RS resources | 2-35 |
| 59. NR_MIMO_Ph5 | 59-2-1-1a | Enhanced Type-I SP codebook for 48 ports – Scheme-A | Support of enhanced Type-I SP codebook for Scheme-A with 48 Tx ports by aggregating multiple NZP CSI-RS resources | 59-2-1-1 |
| 59. NR_MIMO_Ph5 | 59-2-1-1b | Enhanced Type-I SP codebook for 128 ports – Scheme-A | Support of enhanced Type-I SP codebook for Scheme-A with 128 Tx ports by aggregating multiple NZP CSI-RS resources | 59-2-1-1 |

Table 1: RAN1 UE feature list of Enhanced Type-I SP codebook Scheme A

Table 1 lists the main parts of RAN1 UE feature list of Enhanced Type-I SP codebook Scheme A. Here we can see that the baseline capability for Enhanced Type-I SP codebook is with 64 antenna ports. In addition, Enhanced Type-I SP codebook can support 48 and/or 128 antenna ports. This similar principle applies also to other Rel-19 Enhanced/Extended codebooks with 48 or more antenna ports.

Observation #5: The baseline capability for Rel-19 Enhanced/Extended codebooks is with 64 antenna ports.

Proposal #7: We propose to prioritize 64 port codebooks for PMI requirements.

Proposal #8: We propose to consider 128 port codebooks for PMI requirements as the secondary priority.

Proposal #9: We propose to not consider 48 port codebooks for PMI requirements.

# Conclusion

In this contribution we provide the view on the NR MIMO Phase 5 demodulation and CSI requirements issues. The following observations and proposals are made:

Observation #1: In previous releases RAN4 has specified requirements for Type-I SP codebook, Rel-16 eType-II codebook and Rel-18 eType-II Doppler codebook.

Observation #2: Type-I SP codebook is the most deployed codebook in live networks.

Observation #3: In previous releases RAN4 has not specified requirements for Type-I MP codebook and Rel-17 FeType-II codebook.

Observation #4: Enhanced Scheme-A Type-I SP codebook, for Ranks 1 to 4, is straightforward extension from legacy Rel-15 Type-I SP codebook.

Proposal #1: We propose not to introduce CQI, Rank, or CRI requirements for Rel-19 CSI codebooks supporting for up to 128 CSI-RS ports.

Proposal #2: We propose to introduce PMI requirements for Rel-19 CSI codebooks supporting for up to 128 CSI-RS ports.

Proposal #3: We proposed to prioritize Enhanced Scheme-A Type-I SP codebook for PMI requirements.

Proposal #4: We proposed to consider Extended Rel-16 eType-II codebook for PMI requirements as secondary priority.

Proposal #5: We proposed to not consider Enhanced Scheme-B Type-I SP, Enhanced Type-I MP, Extended Rel-17 FeType-II, and Extended Rel-18 eType-II Doppler codebooks for PMI requirements.

Proposal #6: We propose to focus on introducing PMI requirements for Ranks ≤ 4.

Observation #5: The baseline capability for Rel-19 Enhanced/Extended codebooks is with 64 antenna ports.

Proposal #7: We propose to prioritize 64 port codebooks for PMI requirements.

Proposal #8: We propose to consider 128 port codebooks for PMI requirements as the secondary priority.

Proposal #9: We propose to not consider 48 port codebooks for PMI requirements.

# Reference

- RP-242394, WID revision: NR MIMO Phase 5, Samsung

- R1-2502977, Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, AT&T, NTT DOCOMO, inc.