3GPP TSG-RAN4 #115	R4-2505649

Malta Island, Malta, 19th May 2025 – 23rd May 2025

Title:	Discussion on the usage of the terminology “FWA form factor” and its application for 6Rx, 4Rx and 8Rx requirements

Source:	Anritsu Limited

Agenda Item:	7.1.2.3.1

Document for:	Discussion

# Introduction

The requirements for the “Four antenna port reference sensitivity allowance ΔRIB,4R” and “Eight antenna port reference sensitivity allowance ΔRIB,8R” exist in TS 38.101-1 [1] but there is an inconsistency in the terms used.

As requirements for 6Rx are likely to be added either at this meeting or the next one, it would be useful to bring coherence in the terms used to avoid some readers being puzzled by the terminology used.

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands. For operating band frequency range ≤ 1 GHz, the 4Rx operation is primarily for FWA form factor, and when 4Rx operation is supported by handheld UE, ∆RIB,4R as indicated in Table 7.3.2-2 NOTE 2 is applied.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE. |  |

For UE(s) equipped with 8 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,8R in Table 7.3.2-2a for the applicable operating bands.

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n7 | -4.5 |
| n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor. |  |

In RAN4#114bis, a discussion paper [2] was submitted on how to improve the terminology, but it was agreed to “further discuss possible clarifications and improvements on the use of FWA terminology in TS 38.101-1” [3]. The present discussion paper is a resubmission of the proposals while adding another proposal.

# Discussion

Form factor is defined in Wikipedia as:

- “[…] a hardware design aspect that defines and prescribes the size, shape, and other physical specifications of components, particularly in electronics. A form factor may represent a broad class of similarly sized components, or it may prescribe a specific standard. It may also define an entire system, as in a computer form factor.” [4]

While the National Institute of Standards and Technology (NIST) gives the following definition:

- “The physical characteristics of a device or object including its size, shape, packaging, handling, and weight.” [5]

Using it in TS 38.101-1 [1] such as “FWA/CPE/Vehicle/Industrial devices form factor” can be interpreted by different people in different manners.

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aerial UE: A UE supporting UAS (Uncrewed Aircraft Systems) as indicated by the capability aerialUE-Capability-r18 [15] and that has an aerial subscription as described in TS 23.256 [17]. The UE is considered to have access to UAS services after the UE ha performed a successful authentication and authorization with the USS as described in TS 23.256 [17].

[…]

ATG UE: The terminals or user equipments which are mounted in aircraft and support ATG feature (i.e. UE capability airToGroundNetwork-r18) as defined in clause 4.2.2 from TS38.306[15].

[…]

eRedCap UE: The UE with enhanced reduced capabilities as defined in clause 4.2.22.1 from TS38.306 [15].

[…]

RedCap UE: The UE with reduced capabilities as defined in clause 4.2.21.1 from TS38.306 [15].

[…]

Vehicular UE: A UE embedded in a vehicle, permanently connected to an embedded antenna system that radiates externally for NR operating bands.

NOTE:	Vehicular UE does not refer to other UE form factors placed inside the vehicle.

While a specific UE type, particularly if its definition is given in section 3.1 [1] makes things much clearer. Not all the UE types referred in the sections 5, 6 and 7 have their definition given as shown below:

The following observations can be made:

Observation 1: “Form factor” can be understood in different ways, it “may represent a broad class of similarly sized components, or it may prescribe a specific standard”.

Observation 2: A specific UE type particularly if its definition is given in section 3.1 of TS 38.101-1 closes the doors to interpretation and misunderstanding.

Observation 3: Currently in section 3.1, the following UEs are defined: “aerial UE”, “ATG UE”, “eRedCap UE”, “RedCap UE”, “vehicular UE”. While “FWA UE” (“(FWA) CPE UE”), “Industrial (devices) UE” are missing (“handheld UE” does not require a definition as it is obvious).

Observation 4: “NOTE 1:	8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor” in Table 7.3.2-2a of TS 38.101-1 is using “form factor” in singular not plural. Like if FWA/CPE/Vehicle/Industrial devices where sharing a relatively similar form factor and as if “FWA form factor” could apply to all those devices.

Observation 5: “NOTE 1:	When 4 Rx operation is supported by FWA form factor” in Table 7.3.2-2 of TS 38.101-1 is using “FWA form factor” somehow like the terminology used (“FWA UE”, “fixed wireless access (FWA) type of devices”, “FWA devices”, “FWA”) in the RP-240782 WID [6] related to 6Rx.

Observation 6: It is not clear if “NOTE 1:	When 4 Rx operation is supported by FWA form factor” in Table 7.3.2-2 of TS 38.101-1 is equivalent to “NOTE 1:	When 4 Rx operation is supported by FWA(/CPE/Vehicle/Industrial devices) form factor” or not.

Observation 7: Stated in a different way, it is not clear if NOTE 1 for 4 Rx and 8 Rx refers to the same device types or not. Additionally, it is not clear to what UE device types, 6 Rx applies to.

Observation 8: The term “supported” is used for 4Rx in the notes of Table 7.3.2-2. While the term “targeted” is used for 8Rx in the note of Table 7.3.2-2a.

Based on those observations, we would like to propose:

Proposal 1: Remove any use of the term “form factor” in TS 38.101-1 and replace it by the intended UE types. Only the usage of “form factor” is generic manner should be allowed like for example in “Vehicular UE does not refer to other UE form factors placed inside the vehicle.”.

Proposal 2: Add the definition of “FWA UE” in sub-section 3.1, where CPE can be mentioned.

Proposal 3: Add the definition of “Industrial (devices) UE” in sub-section 3.1.

Proposal 4: For each NOTE 1 in the three tables: Table 7.3.2-2 (4Rx), Table 7.3.2-2a (8Rx) and the to-be-added Table 7.3.2-2b (6Rx), clearly give for respectively 4 Rx, 6 Rx and 8 Rx the specific list of UE types that do support them.

# Conclusion

Based on the discussion in section 2, the following can be noted, and the following proposal is proposed:

Observation 1: “Form factor” can be understood in different ways, it “may represent a broad class of similarly sized components, or it may prescribe a specific standard”.

Observation 2: A specific UE type particularly if its definition is given in section 3.1 of TS 38.101-1 closes the doors to interpretation and misunderstanding.

Observation 3: Currently in section 3.1, the following UEs are defined: “aerial UE”, “ATG UE”, “eRedCap UE”, “RedCap UE”, “vehicular UE”. While “FWA UE” (“(FWA) CPE UE”), “Industrial (devices) UE” are missing (“handheld UE” does not require a definition as it is obvious).

Observation 4: “NOTE 1:	8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor” in Table 7.3.2-2a of TS 38.101-1 is using “form factor” in singular not plural. Like if FWA/CPE/Vehicle/Industrial devices where sharing a relatively similar form factor and as if “FWA form factor” could apply to all those devices.

Observation 5: “NOTE 1:	When 4 Rx operation is supported by FWA form factor” in Table 7.3.2-2 of TS 38.101-1 is using “FWA form factor” somehow like the terminology used (“FWA UE”, “fixed wireless access (FWA) type of devices”, “FWA devices”, “FWA”) in the RP-240782 WID [6] related to 6Rx.

Observation 6: It is not clear if “NOTE 1:	When 4 Rx operation is supported by FWA form factor” in Table 7.3.2-2 of TS 38.101-1 is equivalent to “NOTE 1:	When 4 Rx operation is supported by FWA(/CPE/Vehicle/Industrial devices) form factor” or not.

Observation 7: Stated in a different way, it is not clear if NOTE 1 for 4 Rx and 8 Rx refers to the same device types or not. Additionally, it is not clear to what UE device types, 6 Rx applies to.

Observation 8: The term “supported” is used for 4Rx in the notes of Table 7.3.2-2. While the term “targeted” is used for 8Rx in the note of Table 7.3.2-2a.

Proposal 1: Remove any use of the term “form factor” in TS 38.101-1 and replace it by the intended UE types. Only the usage of “form factor” is generic manner should be allowed like for example in “Vehicular UE does not refer to other UE form factors placed inside the vehicle.”.

Proposal 2: Add the definition of “FWA UE” in sub-section 3.1, where CPE can be mentioned.

Proposal 3: Add the definition of “Industrial (devices) UE” in sub-section 3.1.

Proposal 4: For each NOTE 1 in the three tables: Table 7.3.2-2 (4Rx), Table 7.3.2-2a (8Rx) and the to-be-added Table 7.3.2-2b (6Rx), clearly give for respectively 4 Rx, 6 Rx and 8 Rx the specific list of UE types that do support them.

# References

[1] TS 38-101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, version 19.1.0

[2] R4-2504372, Discussion on the usage of the terminology “FWA form factor” and its application for 6Rx, 4Rx and 8Rx requirements, Anritsu, RAN4#114bis

[3] R4-2505101, WF on 6Rx RF requirements and SRS IL imbalance, Intel Corporation, RAN4#114bis

[4] Form factor (design), Wikipedia

[5] Form factor, National Institute of Standards and Technology (NIST)

[6] RP-240782, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Huawei, HiSilicon, TSG-RAN#104