3GPP TSG-RAN WG4 Meeting #115                                                             R4-25056754

Malta, Malta, 19th – 23rd May 2025

Source:	Nokia

Title:	Considerations for Tx on-off time mask for NR_LBCA_Sw

Agenda item:	7.7.3

Document for:	Approval

Introduction

During RAN4#114-bis meeting following issues was recorded [1]

Issue 1: Switching period capability

-	The switching period capability can be defined per band pair per band combination.

Issue 2: Switching period location

-	Await outcome of RAN1 design

Issue 3: Time mask for switching

-	Define the Tx on-off time mask requirements

-	FFS on how to define the Rx switching requirements to reflect where the switching period occurs

-	RAN4 has understanding that RAN1 will take switching period and timing advance into account for RAN1 scheduling pattern for low band CA via switching

Issue 4: Considerations related to the Tx on-off time mask

-	Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL)

-	Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier

-	Other options are not precluded

Issue 5: Test methodology

-	Companies are encouraged to contribute proposals related to the testability of the requirement on the time mask for switching

Agreements done in RAN1#120bis:

Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

Discussion

Work item [2] outlines FDD CA band combination in which the semi static switching framework shall be applied. During a previous meeting, switching period values of {35 µs, 140 µs} were recorded. [3]. Switched LBCA functions by alternating DL reception between two carriers: FDD and SDL. There is only one operational uplink carrier which maintains a constant carrier frequency even as the DL alternates between the two carriers. Therefore, switching period values concerns only DL. It is presumed that NW will schedule resources in time so that it covers both the switching period and the TTA. For UL, the switched LBCA operation is carrier power on/off operation where UL overlap with SDL carrier shall be avoided. There are no architectural considerations needed for defining TX on/off time mask.

Recorded switching period values {35us,140us} are applied for DL carrier switching

It is assumed that NW will schedule resources in time so that it covers both the switching period and the TTA.

For UL, switched LBCA operation is carrier power on/off operation where UL overlap with SDL carrier shall be avoided

There are no architectural considerations needed for defining TX on/off time mask.

Proposal for TX on/off time mask

RAN1 has not yet reached a conclusion regarding the location of the switching period. In this paper we assume that the switching period is positioned at the end of a slot in the FDD and SDL carriers (“switch from carrier”). The TX OFF (FDD slot) and ON (SDL slot) transient periods are contained inside the DL switching gap which includes switching period and TTA. This approach is adopted to prevent switching gaps during the PDCCH. Although RAN1 may end up with different conclusions the fundamental principles outlined in this document are applicable to other switching period locations.

We present two use cases that should be considered when defining the TX on/off time mask. The first is a DSS use case in which NTA,offset is 0us and the second is a normal use case in which NTA,offset is 13us. In principle, TTA =0us is feasible representing the worst-case scenario for UE RF.

Figure 1 TX on/off time mask with TTA =0us. Considering 3us TAE and transient period currently specified in 38.101-1 for on-off, 13us transient period is needed at the end of FDD UL slot. Default transient period 10us for off-on can meet the requirement not to overlap UL and SDL carriers.

In a DSS scenario FDD UL carrier requires 13us transient period at the end of the slot not to overlap the SDL carrier. This 13us includes the 10us on-off transient time curerntly specified in 38.101-1 and 3us TAE. Practically, the network must unschedule the last symbol for the UE. In this context for the UL off-on transition the 10us transient period currently recorded in 38.101-1 is sufficient to meet the requirement of preventing overlap between the UL and SDL carriers, as there is no scheduled DL symbol prior to UL start slot.

Figure 2 TX on/off time mask with TTA =13us. Considering 3us TAE, default transient periods for on/off time mask can meet the requirement not to overlap UL and SDL carriers.

In a normal TA CA scenario, the existing transient periods specified in 38.101-1 are sufficient to ensure that there is no overlap between the FDD UL and SDL carriers.

In a DSS scenario the last symbol of FDD UL must be unscheduled to prevent overlap with the supplementary downlink SDL carrier.

In a DSS scenario, when switching from the SDL to the FDD carrier, the existing transition times specified in 3GPP TS 38.101-1 are adequate, provided that the DL switching period is positioned at the end of the SDL slot.

In a normal TA CA scenario, the existing transient periods specified in 3GPP TS 38.101-1 are sufficient to prevent overlap with the SDL carrier, assuming that the DL switching periods are positioned at the end of both the FDD slot and the SDL slot.

Transient period of TX on-off time mask is contained inside the DL switching gap duration which is defined as the combined time of DL switching period + TTA.

In a DSS scenario when switching from FDD carrier to SDL carrier, the network should unschedule the last symbol of FDD UL to accommodate the existing transition times for the time mask specified in 38.101-1.

Conclusion

This paper discusses the TX on/off time mask requirements for switched LBCA operation. The analysis focuses on a scenario where the switching period is located in the switch-from carrier; however, the principles presented are applicable to other switch period locations as well.

The following Observations and Proposals were made:

Observation 1: Recorded switching period values {35us,140us} are applied for DL carrier switching

Observation 2: It is assumed that NW will schedule resources in time so that it covers both the switching period and the TTA.

Observation 3: For UL, switched LBCA operation is carrier power on/off operation where UL overlap with SDL carrier shall be avoided

Observation 4: There are no architectural considerations needed for defining TX on/off time mask.

Observation 5: In a DSS scenario the last symbol of FDD UL must be unscheduled to prevent overlap with the supplementary downlink SDL carrier.

Observation 6: In a DSS scenario, when switching from the SDL to the FDD carrier, the existing transition times specified in 3GPP TS 38.101-1 are adequate, provided that the DL switching period is positioned at the end of the SDL slot.

Observation 7: In a normal TA CA scenario, the existing transient periods specified in 3GPP TS 38.101-1 are sufficient to prevent overlap with the SDL carrier, assuming that the DL switching periods are positioned at the end of both the FDD slot and the SDL slot.

Proposal 1: Transient period of TX on-off time mask is contained inside the DL switching gap duration which is defined as the combined time of DL switching period + TTA.

Proposal 2: In a DSS scenario when switching from FDD carrier to SDL carrier,  the network should unschedule the last symbol of FDD UL to accommodate the existing transition times for the time mask specified in 38.101-1.

- References

- R4-2505223, WF on UE RF requirements for LB CA via switching, Apple, Wuhan, China, April 7-11, 2025

- RP-243317 NR_LBCA_Sw WID_cl, RAN#106

- R4-2503016 NR_LBCA_Sw WF, RAN4#114 Athens, Greece, February 17-21, 2025.