3GPP TSG-RAN WG4 Meeting #115	R4-2505677

Malta, 19th – 23rd May 2025

Title: 	Power domain enhancements for single carrier

Agenda Item:	7.1.2.2.1

Source: 	Nokia

Document for:	Approval

# 1				Introduction

In this contribution we express our views to enable MPR reduction.

# 2	Discussion

In previous RAN4 meeting WF [1], several mechanisms to enhance power domain performance in NR systems, particularly focusing on MPR (Maximum Power Reduction) optimization and channel bandwidth (CBW) extension for single carrier scenarios has been discussed.

Issue 1-1-4 Guard band for extended CBW

#### Issue 1-1-4: Guard band for extended CBW - Options - Option 1: No need to define multiple UE guard bands for initial CBW and extended CBW, UE should have one guard band per each side. All frequency segments within UE extended CBW and outside UE maximum transmission bandwidth (number of RBs in 38.101-1) act as UE total guard bands (i.e., UE initial guard band + additional guard band according to UE extended CBW NW indication), where this UE cannot be allocated and OOBE edges are the outer edges of UE total guard band. - Option 2: NRB based extension and associated guard band: - maximum bandwidth extension: The extended RB BW NRBext inherits the guard-bands (GBext) from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext. - flexible NRB-based extension: Similarly to the ceil(NRB/2) extension, the guard band mapping for the fractional NRB extension can inherit the guard band from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext - Option 3: The GBs of extended UE CBW should only be retained on the side that is far away from the original CBW. The GBs of extended UE CBW reuse the size of original GB of UE CBW. - Option 4: The GBs of the extended CBW shall be the remaining portion obtained by subtracting the RB numbers from the specified channel bandwidth when the extended CBW is selected from Table 5.3.2-1 in 3GPP TS 38.101-1. - Others - WF - FFS in next meeting

Multiple options are proposed for guard band handling in the context of extended UE channel bandwidth (CWB). Introducing multiple guard bands(GB) per each side for a UE (i.e., guard band for original UE CBW and guard band for extended UE CBW) and then separate them by defining a frequency segment (part of extended RBs) is not a constructive approach and over complication of the specification. Since all these resources are not intended for RB allocation of this UE applying UE extended CBW, these segments from UE RF perspective acts as total guard band (one GB per side), and this definition of single guard band per side maintain the full clarity and avoid unnecessary definitions.

Option 1 as described in the WF [1] aligns with the principle of simplicity, where UE considers all frequency segments outside UE original maximum transmission bandwidth (i.e., NRB of UE original CBW from Table 5.3.2-1 of 38.101-1) up to the outer edge of UE extended CBW as the new guard band (GB total in the above figure).

Proposal 1: We support Option 1 since there is no need to define multiple UE guard bands one for initial UE CBW, another guard bands for extended CBW and extended RB, UE should have one unified guard band per each side.

The total unified/single GB per side (symmetric or asymmetric extension) shall be defined as follows:

GBtotal=GBextended channel=GBchannel+GBadditional/extended

where GBchannel is from Table 5.3.3-1 in 38.101-1 and GBadditional/extended  is determined according to NW indication of the extension in kHz (e.g., Fixed value (0.5*CBW=0.5* BWchannel and/or 0.5*NRB  and NRB from Table 5.3.2-1) or using other extension factor).
This option shall be considered if the fixed extension is in terms of NRB or CBW, and thus
 GBadditional/extended=extension factor*NRB*12*SCS,  or GBadditional/extended=extension factor*BWchannel*1000KHz

Alternatively, the unified single GB per side in kHz (symmetric or asymmetric extension) can be defined as follows:

GBtotal=GBextended channel=(BWextended channel*1000KHz-NRB*SCS*12)/2-SCS/2
where NRB are from Table 5.3.2-1 in 38.101-1 according to UE original BW [before the extension] and GBextended channel and SCS are expressed in kHz. 
This option can be considered especially if the fixed extension is in terms of CBW (e.g., 0.5CBW), and thus BWextended channel=1+extension factorBW channel

Proposal 2: RAN4 shall define the new unified single UE guard band per side (symmetric or asymmetric extension) with rel-19 UE CBW extension according to one of the following options:

Option 1: GBtotal=GBextended channel=GBchannel+GBadditional/extended where GBchannel is from Table 5.3.3-1 in 38.101-1 according to original UE CBW and GBadditional/extended is determined according to NW indication of the extension in kHz (e.g., Fixed value (0.5*CBW=0.5* BWchannel and/or 0.5*NRB from Table 5.3.2-1) or using other extension factor). This option shall be considered if the fixed extension is in terms of NRB or CBW, and thus GBadditional/extended=extension factor*NRB*12*SCS, or GBadditional/extended=extension factor*BWchannel*1000KHz Option 2: GBtotal=GBextended channel=(BWextended channel*1000KHz-NRB*SCS*12)/2-SCS/2 where NRB are from Table 5.3.2-1 in 38.101-1 according to UE original BW [before the extension] and GBextended channel and SCS are expressed in kHz. This option can be considered if the fixed extension is in terms of CBW (e.g., 0.5CBW), and thus BWextended channel=1+extension factorBW channel where extension factor is fixed and determined according to NW indication.

In the issue 1-2-2 of the WF [1], sub-issue 1 over whether extended RBs should start from the outer edge of original GB or edge of original CBW, we propose to proceed with a single guard band per side, consistent with our earlier contributions and aligned with the configurations illustrated in Case 1 and Case 3 in the WF [1]. This approach ensures that the UE's original guard bands are kept transparent, simplifying specification.

Concerning sub-issue 2 in issue 1-2-2, case 1 shall be selected since the stepped IBE definition of option 2 beyond original UE CBW leads to unnecessary strict IBE requirement in the extended part (i.e., total guard band) limiting the possible MPR reduction and may require further hardware design consideration in the UE to meet the IBE mask.

Proposal 3: RAN4 shall select case 1 in issue 1-2-2 and option 1 in issue 1-2-3 for at least the following reasons:

- a single unified guard band per side is defined outside the original maximum transmission bandwidth up to the outer edge of UE extended CBW

- there is no need to add new strict IBE requirement as stepped IBE line in the extended part including previous UE original GB to avoid impact on 5G UE hardware/RF design.

gNB can currently allocated other UEs the guard band of first UE in scenario 2 having narrower CBW than BS CBW. Rel-19 shouldn’t impose any scheduling constraint on NW and NW should be able to allocate UEs in any RB within its RB grid.

Proposal 4: RAN4 shouldn’t impose any scheduling constraint on NW, and NW should be able to allocate UE(s) in any RB within RB grid of the gNB regardless of the specific UE RB grid or UE CBW or UE guard band after UE channel extension.

For the issue 1-2-4 related to Relaxation of the extended IBE (In-band Emission) requirements in the extended region, while we are ok with option 1 for IBE relaxation based on BPSK EVM. We also propose considering a correct IBE definition as shown in Fig. 1 fully aligned with IBE main purpose to limit the interference to other UE RB allocation only.

Hence, RAN4 shouldn’t define any IBE requirement overlapping with gNB’s guard band where there is no possible UE/gNB transmission, since IBE requirement in this segment is not necessary and against IBE main purpose to limit interference to other UEs.

Figure 1a: IBE requirement range within UE extended CBW where BS CBW guard bands and UE guard bands are non-overlapping

Figure 1b: IBE requirement range within UE extended CBW where BS CBW guard bands and UE guard bands are overlapping in at least one side.

Figure 1c: IBE requirement range within UE extended CBW where BS CBW guard bands and UE guard bands are overlapping in at least one side and UE extended CBW is across multiple DL intra-band contiguous CCs.

Proposal 5: IBE shall be applied in all non-allocated RBs, and guard bands for original UE CBW and the whole extended segment (i.e., the extended part beyond UE original CBW) to avoid interference to other UEs in the network

Proposal 6: IBE requirement shouldn’t be defined in the frequency range overlapping with the guard band(s) of the BS CBW to avoid defining unnecessary IBE requirement where there is no possible transmission for/from any UE and thus against IBE main purpose to limit interference to other UEs.

# 3	Conclusion

In this contribution we have made following proposal.

Proposal 1: We support Option 1 for Issue 1-1-4: Guard band for extended CBW, since there is no need to define multiple UE guard bands one for initial UE CBW, another guard bands for extended CBW and extended RB, UE should have one unified guard band per each side.

Proposal 2: RAN4 shall define the new unified single UE guard band per side (symmetric or asymmetric extension) with rel-19 UE CBW extension according to one of the following options:

Option 1: GBtotal=GBextended channel=GBchannel+GBadditional/extended where GBchannel is from Table 5.3.3-1 in 38.101-1 according to original UE CBW and GBadditional/extended is determined according to NW indication of the extension in kHz (e.g., Fixed value (0.5*CBW=0.5* BWchannel and/or 0.5*NRB from Table 5.3.2-1) or using other extension factor). This option shall be considered if the fixed extension is in terms of NRB or CBW, and thus GBadditional/extended=extension factor*NRB*12*SCS, or GBadditional/extended=extension factor*BWchannel*1000KHz Option 2: GBtotal=GBextended channel=(BWextended channel*1000KHz-NRB*SCS*12)/2-SCS/2 where NRB are from Table 5.3.2-1 in 38.101-1 according to UE original BW [before the extension] and GBextended channel and SCS are expressed in kHz. This option can be considered if the fixed extension is in terms of CBW (e.g., 0.5CBW), and thus BWextended channel=1+extension factorBW channel where extension factor is fixed and determined according to NW indication.

Proposal 3: RAN4 shall select case 1 in issue 1-2-2 and option 1 in issue 1-2-3 for at least the following reasons:

- a single unified guard band per side is defined outside the original maximum transmission bandwidth up to the outer edge of UE extended CBW

- there is no need to add new strict IBE requirement as stepped IBE line in the extended part including previous UE original GB to avoid impact on 5G UE hardware/RF design.

Proposal 4: RAN4 shouldn’t impose any scheduling constraint on NW, and NW should be able to allocate UE(s) in any RB within RB grid of the gNB regardless of the specific UE RB grid or UE CBW or UE guard band after UE channel extension.

Proposal 5: IBE shall be applied in all non-allocated RBs, and guard bands for original UE CBW and the whole extended segment (i.e., the extended part beyond UE original CBW) to avoid interference to other UEs in the networkProposal 6: IBE requirement shouldn’t be defined in the frequency range overlapping with the guard band(s) of the BS CBW to avoid defining unnecessary IBE requirement where there is no possible transmission for/from any UE and thus against IBE main purpose to limit interference to other UEs.

# 4	References

[1] 3GPP TSG-RAN WG4 Meeting #114bis, WF on power domain enhancements R4-2505099, Wuhan, P.R.China, 07th – 11th April, 2025.