3GPP TSG-RAN WG4 Meeting #115	                                                                                       R4-2505697

Malta, MT, 19th – 23th May, 2025

Agenda Item:	7.23.3.2

Source: 	OPPO

Title: 	Study of impacts on RAN4 requirements for measurement event prediction

Document for:	Approval

- Introduction

In RAN4#114bis meeting, the RRM requirements for AI mobility were discussed. It was agreed both indirect and direct cases should be discussed in RAN4 unless RAN2 give the prioritization.

Issue 3-1-1: Prioritization of cases for event prediction Agreement: - RAN4 should discuss both indirect and direct cases. If any prioritization will be agreed in RAN2, RAN4 to follow RAN2 prioritization

In this contribution, we will evaluate the impacts on RRM requirements for measurement event prediction based on RAN2 progress, and the agreements derived from RAN2 is provided in Annex for information.

- Discussion

In last meeting, it was agreed both indirect and direct cases should be discussed in RAN4 unless RAN2 give the prioritization. And the following agreement was reached for indirect case.

Issue 3-1-2: Performance metric for event prediction use case Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs

In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A or case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

For this case, the AI/ML model is used for providing RRM measurement requirements, and after that, legacy procedure can be used to derive measurement event. To verify the AI/ML functionality, we understand the requirements of intermediate output can be defined and the requirements for measurement prediction can be used as baseline. In last meeting, there are also proposals to not define requirements for the predicted RSRP since the results of RRM measurement prediction can be used for indirect event prediction. Technically, we understand it is correct, but considering measurement prediction and event prediction are independent use cases and the report may be separate, we think it is better to clarify the requirements for predicted RSRP in event prediction.

Proposal 1: For indirect prediction, if the report includes the predicted RSRPs, the requirements for measurement prediction can be reused.

In last meeting, it was mentioned that RAN4 may need to discuss the delay requirements for predicted event triggered reporting. But from our understanding, the event reporting is based on the predicted RSRP results and the event trigger condition, and it cannot be guaranteed that the event will always occur. So it is hard to define the delay for predicted event triggered reporting.

The possible delay may be the predicted RSRP reporting delay which can include the measurement delay and the inference delay. It would be similar as the discussion for measurement prediction and AI BM which has no conclusions so far. We can wait for more progress.

Proposal 2: For indirect prediction, RAN4 not to define delay requirements for predicted event triggered reporting.

Proposal 3: For indirect prediction, wait for more progress from measurement prediction to discuss whether the predicted RSRP reporting delay is needed.

For the metric/KPI for indirect and direct event prediction, RAN2 has the following agreements:

Agreements on inputs/outputs and KPIs - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, Continue discussion over email discussion to see if there is a difference.

And for indirect prediction, the following definitions are used in RAN2 evaluation:

4 To agree following definition for true event prediction, false event detection and missed event detection for indirect measurement event prediction Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD]

From RAN4 perspective, the above KPIs can be considered to define the requirements for prediction accuracy, e.g., the performance metrics can be defined as certain ETD or certain F1 score under some conditions.

For both direct and indirect event prediction, the HO failure number are also used in RAN2 evaluation which we think can also be used for performance definition.

Agreements on measurement event prediction

1.  Option 2 and Option3 outperform legacy solutions in terms of HO failure number per UE per second

2.	Capture that most companies show in their simulation results that option 3 outperforms option 2 when prediction accuracy is good enough.  There are some companies that show the opposite and indicate why.

3	For FR2 to FR2 intra-frequency temporal domain case A (Case 4), F1 score is higher for shorter TTT values of the predicted event.

4	From the small number of companies that provided results for direct prediction: Both direct event prediction and indirect prediction methodologies demonstrate a reduction in HOF number per UE per second when compared to the legacy approach at the system level performance.

5	No further simulation results are expected for future meetings

Proposal 4: For indirect prediction, the following can be considered as possible KPI to evaluate the performance:

- Alt 1: The requirements are defined as [maximum] ETD under certain conditions (e.g., certain probability of true event prediction, false event detection or F1 score etc.)

- Alt 2: The requirements are defined as probability of true event prediction or F1 score under certain conditions (e.g., ETD ranges)

- Alt 3: The handover failure rate per UE under certain time durations

But since all the above KPIs and procedures for event prediction and how to report the outputs are still under discussion in RAN2, we think RAN4 can list the possible metrics to define requirements but not to make the decision in SI. We can wait for more RAN2 progress to discuss whether and how to define new quantities, requirements or applicability for indirect prediction.

Proposal 5: For direct and indirect prediction, RAN4 to discuss whether and how to define new quantities or requirements based on further RAN2 progress.

- Conclusions

In this contribution, we discussed the possible RAN4 requirements for measurement event prediction and got the following proposals:

Proposal 1: For indirect prediction, if the report includes the predicted RSRPs, the requirements for measurement prediction can be reused.

Proposal 2: For indirect prediction, RAN4 not to define delay requirements for predicted event triggered reporting.

Proposal 3: For indirect prediction, wait for more progress from measurement prediction to discuss whether the predicted RSRP reporting delay is needed.

Proposal 4: For indirect prediction, the following can be considered as possible KPI to evaluate the performance:

- Alt 1: The requirements are defined as [maximum] ETD under certain conditions (e.g., certain probability of true event prediction, false event detection or F1 score etc.)

- Alt 2: The requirements are defined as probability of true event prediction or F1 score under certain conditions (e.g., ETD ranges)

- Alt 3: The handover failure rate per UE under certain time durations

Proposal 5: For direct and indirect prediction, RAN4 to discuss whether and how to define new quantities or requirements based on further RAN2 progress.

- References

R4-2504901 WF on RRM requirements for FS_NR_AIML_Mob

R4-2503620 Topic summary for [114bis][210] FS_NR_AIML_Mob

RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

RP-240082 Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

Annex RAN2 agreements

RAN2#127bis meeting:

Agreements

Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement).   FFS what is KPI.

Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing.  Focus on temporal case B.

Companies will prioritize simulations on indirect method.   Companies can bring simulations on direct method and should report what method is being used.

Measurement event prediction results are expected in RAN2#129

Agreements on simulation assumptions

The Simulation assumption of RRM measurement prediction can be reused unless otherwise specified.

Companies can pick and report what they are using for filtering options (similar to RRM prediction)

Companies will focus on sub-case 2 for measurement event prediction.   Companies can simulate other sub-cases if they wish and report what they are using.

Leave the simulation parameter discussion for email discussion.  Pick only one value for A3.

It is up to company implementation how to model UE behavior after A3 event is trigger.  Focus on intermediate KPIs for this exercise.  System level KPI is FFFs

For measurement event prediction, traffic is not simulated.

Agreements on inputs/outputs and KPIs - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, - Continue discussion over email discussion to see if there is a difference.

- RAN2#128 meeting:

For System level simulation 1 For measurement event prediction for temporal domain case B: UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation. Remove T0 from the picture 2 For measurement event prediction for temporal domain case A, company focus on option 2 or option 3. Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1)) Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT. 3 Handover Preparation time is 40ms and handover execution time: 40ms 4 RAN2 focus on indirect prediction methodology.

Agreements on direct prediction for RLF and event predictions 1 The time window for direct prediction goes for interpretation 2 2 For direct prediction, following values are agreed and company can report probability threshold for corresponding case (other values are allowed) Measurement event prediction; RLF prediction. Time window length (Interpretation 2); PW length as indirect case (FR1:200ms; FR2:400ms); PW length as indirect case FR1/FR2: 400ms.

Agreements on event prediction 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model 2 The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. 3 For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional. 4 To agree following definition for true event prediction, false event detection and missed event detection for indirect measurement event prediction Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD] 5 To agree following definition for true event prediction, false event detection and missed event detection for direct measurement event prediction Counter n3’ (true event prediction): it increases by 1 when a real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n1’ (false event detection): it increases by 1 when no real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n2’ (missed event detection): it increases by 1 when a real event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold 6 Agree following definition of F1 score: F1 score = 2*Precision*Recall/(Precision + Recall) Formula_3 (13/14) 7 In addition to F1 score, precision and recall in following formula are optional metrics to report. (12/14) Precision = n3/(n1+n3) Formula_1 Recall =n3/(n2+n3) Formula_2 Note: for direct prediction, the counter should be n1’,n2’,n3’

- RAN2#129 meeting:

Objective in Case A (FR2): Predict A3 events to enhance HO execution via earlier HO Prep upon A3 event prediction (as shown in Option 2 or Option 3), i.e., HO performance enhancement. - Objective in Case B (FR1): Predict A3 events in the presence of skipped measurements, i.e., overhead reduction while maintaining similar HO performance.

Agreements on Measurement event case A - Most of the results show that the F1 score for prediction of measurement event is very good. - For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate.

Agreements on Measurement event case B - Measurement event case B can have very good F1 score depending on filtering approach/PW - F1 score is good for the PW window for case B (i.e. low PW) - F1 score decreases with an increasing MRRT value - We will focus on finding cases that bring benefits to the system rather than trying to compare - With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism.

Other agreements Inter-frequency measurement event prediction will be considered for the specification impact study but no explicit simulations will be required.

Agreements on direct measurement event prediction - F1 score for direct measurement is very good based on the current simulation results.

##### RAN2#129bis Agreements on measurement event prediction 1. Option 2 and Option3 outperform legacy solutions in terms of HO failure number per UE per second 2. Capture that most companies show in their simulation results that option 3 outperforms option 2 when prediction accuracy is good enough. There are some companies that show the opposite and indicate why. 3 For FR2 to FR2 intra-frequency temporal domain case A (Case 4), F1 score is higher for shorter TTT values of the predicted event. 4 From the small number of companies that provided results for direct prediction: Both direct event prediction and indirect prediction methodologies demonstrate a reduction in HOF number per UE per second when compared to the legacy approach at the system level performance. 5 No further simulation results are expected for future meetings