3GPP TSG-RAN WG4 Meeting #115									     R4-2505704

Malta, MT, 19th – 23rd May, 2025

Source: 	OPPO

Title: 	On RRM requirement for on-demand SIB1

Agenda Item:	7.27.3.3

Document for:	Discussion

Introduction

In last meeting, RAN4 approved a WF on OD-SIB1[1]. In this document, we provide our views on the RRM requirement impacts from on-demand SIB1 in R19 NES.

Discussion

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Way forward: - Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. (HW, OPPO, Apple, Nokia, Xiaomi, CATT, MTK, vivo) - Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. (CMCC, QC, LGE) - Option C: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paing time. (E///, LGE, MTK) Issue 2-1-2: RRM requirements impacts for OD-SIB1 – timing Agreement - Clarify in transmit timing requirements to include UL WUS transmission (PRACH) for OD-SIB1: - reference cell is the NES cell - NTA for UL-WUS is defined as 0 - NTA offset is the [n-TimingAdvanceOffset] from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133 - Note: The IE can be updated based on other WG’s progress.

Based on RAN1/2/4 agreements for OD-SIB1, the UE procedure of acquiring OD-SIB1 can be summarized as follows.

- Step 1: The UE evaluates cell reselection based on measurements towards SSB of target NES cell.

- Step 2: After cell reselection delay, the UE initiates OD-SIB1 procedure.

  - Step 2-1: UE transmits the UL-WUS(PRACH)

  - Step 2-2: UE receives the RAR

  - Step 2-3: UE monitors OD-SIB1 window to decode the SIB1 and OSIs

In legacy cell re-selection requirements, the interruption on paging reception is defined in Clause 4.2.2.6 in TS 38.133, e.g., the interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. However, for NES OD-SIB1, we need to consider additional RF tuning time and baseband processing time for UL WUS transmission and RAR reception on non-serving cell which is also a NES cell. We support option 1, and the interruption requirements may need to be extended due to UL WUS and RAR.

Proposal 1: The interruption requirements need to be extended due to UL WUS and RAR reception.

We would like to propose the following text proposal:

In addition, if the target NR cell is an NES cell, the interruption time for UE capable of on demand SIB1 acquisition shall not exceed TOD-SIB1 +TSI-NR + 2*Ttarget_cell_SMTC_period ms. TOD-SIB1 is the time required for UL WUS transmission and RAR reception for on demand SIB1 acquisition of NES cell.

Proposal 2: If the target NR cell is an NES cell, the interruption time for UE capable of on demand SIB1 acquisition shall not exceed TOD-SIB1 +TSI-NR + 2*Ttarget_cell_SMTC_period ms. TOD-SIB1 is the time required for UL WUS transmission and RAR reception for on demand SIB1 acquisition of NES cell.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: The interruption requirements need to be extended due to UL WUS and RAR reception.

Proposal 2: If the target NR cell is an NES cell, the interruption time for UE capable of on demand SIB1 acquisition shall not exceed TOD-SIB1 +TSI-NR + 2*Ttarget_cell_SMTC_period ms. TOD-SIB1 is the time required for UL WUS transmission and RAR reception for on demand SIB1 acquisition of NES cell.

- References

[1] R4-2504907, WF on RRM requirements for Netw_Energy_NR_enh_Part2