3GPP TSG-RAN WG4 Meeting #115                                             R4-2505707

Malta, MT, 19th – 23th May, 2025

Agenda item:	7.28.5.2

Source:	OPPO

Title:	Discussion on RRM requirements for LP-WUSWUR

Document for:	Approval

- Introduction

Previous meeting discussed the RRM requirements regarding LP-WUS/WUR and reached some agreements in [1]. This contribution will provide our further considerations on the open issues captured in [2].

- Discussion

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Background: Case #1 and #3 have been agreed RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. RAN4 to further discuss case #2: RRM measurement case index; MR serving cell measurement; MR neighbouring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. Case #4 is covered by issue for higher priority frequency layer: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #4 Relaxed case c; Off; On, FFS the condition and the details; ON. Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading - Proposals Regarding case 2: P1-1: Case#2 is a possible case, its existence depends on network configuration. No separate discussion is needed for Case#2 (CMCC); P1-2: Prefer to consider (vivo) P2: De-prioritize (xiaomi); P3: Not consider (oppo ZTE Huawei Ericsson MTK); P3-1: No consider case 2, RAN4 can revisit case 2 when further RAN1/2 conclusion conflicting with RAN4 assumptions. (CATT Apple) P4: Up to RAN2’s decision (LG vivo Nokia) Issue 1-1-10: Considerations on higher priority frequency layer Agreement: - Support relaxed higher priority frequency layer search at case 1

In last meeting, the handling of high priority frequency layer in case #1 was discussed and it was agreed to support relaxed high priority frequency layer searcher at case 1. With this conclusion, case #4 can be supported. For case #2, as we discussed in previous meeting, it is not reasonable to only measure serving cell but skip neighbor cell as the MR is already turned on. And considering the timeline for this WI, we think we do not need to further discuss case #2 in this release.

Proposal 1: Do not consider case #2 in Rel-19.

After supporting high priority frequency layers in case #1, the MR measurement requirements in case #1 and case #3 should be clarified. Based on the existing requirements, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, only high priority frequency layers are measured, otherwise, all high, equal or lower priority frequency layers are measured.

For case #1, only the case when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is considered and the legacy relaxed requirements, i.e., K2*Thigher_priority_search can be reused, where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60.

For case #3, both cases are considered. when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is considered and the legacy relaxed requirements, i.e., K2*Thigher_priority_search can be reused. Otherwise, 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter are applied.

Proposal 2: For case #1, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the legacy relaxed requirements, i.e., 60*Thigher_priority_search can be reused.

Proposal 3: For case #3, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the legacy relaxed requirements, i.e., 60*Thigher_priority_search can be reused. Otherwise, 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter are applied.

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements Agreement: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity - Proposals - P1: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms (xiaomi) - Option 1: define the lower limit for measurement delay requirements; (Huawei) - Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity). - P2: No need for lower bound. (Nokia) - P3: Wait for more RAN1 progress before determining whether to have lower bound. (vivo oppo) Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity - Proposals - P1: No need to define upper bound for SSB-based LP-WUR measurement periodicity. (xiaomi oppo CATT CT Nokia) - P2: Prefer the periodicity for measurement and evaluation delay requirements for LP-WUR based on SSB in IDLE/Inactive mode is 1.28s if the LO periodicity is 2.56s. (vivo ZTE) - P3: Postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity (Huawei)

In last meeting, it was agreed that periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity regardless of whether LP-SS is configured or not, but whether to define upper bound is still FFS. If the upper bound is defined, the requirements would be defined as min(N*LO periodicity, Tupper bound). And it means when DRX cycle is large, UE is required to perform measurement based on an interval smaller than DRX cycle. We think it is not reasonable or necessary considering UE powering saving, and there were not any such requirements in legacy.

For LP-SS based measurement requirements, it has been agreed to use LP-SS periodicity as baseline, but it is still open whether to define lower bound on the delay requirements. In RAN1#120bis meeting, it was agreed to additionally support 160ms LP-SS periodicity.

Agreement Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1, M=2: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support LP-SS periodicity of 160ms

Since the LP-SS periodicity is large enough, we understand there is no need to define the lower bound.

Proposal 4: No need to define upper bound for SSB based LP-WUR measurement requirements.

Proposal 5: No need to define lower bound for LP-SS measurement periodicity.

Issue 1-2-6-1: On the number of SSBs Agreement: Aim to conclude the issue by Tuesday in RAN4 #115. For 10ms, 400ms 800ms ramping up time, the number of SSBs: Option 1: [ 3 5 5] (Apple vivo, LG, HW, MTK, oppo, CMCC) Side condition [>= -3 dB] Option 2: [3 3 3] (ZTE) (Side condition -3 dB) Option 3: [5 5 5] (xiaomi) (-6 dB) Option 4: [2 2 2] (Nokia) (-2 dB) Regarding SINR Option 1: -3 dB (Apple vivo LG CMCC Xiaomi oppo HW Ericsson QC) Option 2: -2 dB (Nokia QC) Option 3: -6 dB (MTK) AGC: 1-2 samples for side conditions (-3 dB) Issues for further discussion (for information): Issue 1: LR can provide rough time? Option 1: Yes Option 2: Related to the UE implementation, not mandate from RAN4 requirement perspective Issue 2: The side condition and number of samples Option 1: based on the value (side condition, number of samples) in the existing requirement. Issue 1-2-6-2: On SSBs number for different SSB periodicities Agreement Same SSB number for different SSB periodicity Agreement When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay. Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity

LS R4-2503110_R1-2501624: In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120: Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

Agreement On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, consider the following alternatives for possible down-selection in RAN1#121. - Alt 1: Do not report for SSB periodicities other than 20ms - Note: LP-WUS is not supported for SSB periodicities larger than 20ms - Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns): SSB periodicity (ms); Wake-up delay (ms) UE capability 1; Wake-up delay (ms) UE capability 2; Wake-up delay (ms) UE capability 3. [5]; [x]; [x]; [x]. [10]; [x]; [x]; [x]. 20; [70]; [500]; [900]. [40]; [x]; [x]; [x]. [80]; [x]; [x]; [x]. [160]; [x]; [x]; [x]. - Alt 3: It is up to RAN4

Based on the latest RAN1 agreements and the LS [4], RAN1 has defined the UE capability report for wake-up delay which indicates the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. With this agreement, we understand RAN4 does not need to define additional delay or interruption requirements.

In addition, RAN1 assume 3 and 5 SSB samples for synchronization when defining the candidate values of capability for different ramping up time. Based on the discussion in last RAN4 meeting, there are two aspects to be considered for deciding the number of SSB samples. The first one is whether LR can provide rough timing for MR wake-up and the second one is whether to link the number to certain SINR condition. For the timing information provided by LR, we think it is up to UE implementation, and we cannot always assume that LR can provide timing for MR, especially for the deep sleep case. Referring to the existing unknown cell search time in handover and the AGC assumption discussed in last meeting, we think the SSB number assumption in RAN1 LS is reasonable. For the SINR condition, we understand the condition for LR should follow the simulation assumption i.e., -3dB. In such case, the MR condition could be higher than that and is good enough. And we think there is no need to link the UE capability to certain conditions.

Proposal 6: RAN4 not to define additional delay or interruption requirements on top of UE capability report of wake-up delay defined in RAN1.

Proposal 7: The assumption of 3 and 5 SSB samples in wake-up delay definition in RAN1 is reasonable.

Proposal 8: No need to link the SSB number assumption to certain SINR condition.

In RAN1, the following agreements were made on the RRM measurement metrics based on SSS for OFDM-based LP-WUR: Agreement (RAN1#120) Confirm the following working assumption with the additional text in red: Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications. - The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement. - FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement. Agreement (RAN1#120bis) For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable. From RAN1 perspective, it was agreed that SMTC window is not applicable for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell. RAN1 respectfully asks RAN4 to confirm it.

In this meeting, we received RAN1 LS RRM measurement metrics for OFDM-based LP-WUR, it was agreed to reuse the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. But in legacy definition, the measurement resource is confined within SMTC window which may not applicable. Since for LP-WUR, only serving cell measurement is needed and the measurement requirements are based on LO periodicity, we think no need to limit the measurement resource within the SMTC window.

Proposal 9: Confirm to RAN1 that for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

- Conclusion

This contribution gave the following proposals on the core requirements for LP-WUR/WUS.

Proposal 1: Do not consider case #2 in Rel-19.

Proposal 2: For case #1, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the legacy relaxed requirements, i.e., 60*Thigher_priority_search can be reused.

Proposal 3: For case #3, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the legacy relaxed requirements, i.e., 60*Thigher_priority_search can be reused. Otherwise, 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter are applied.

Proposal 4: No need to define upper bound for SSB based LP-WUR measurement requirements.

Proposal 5: No need to define lower bound for LP-SS measurement periodicity.

Proposal 6: RAN4 not to define additional delay or interruption requirements on top of UE capability report of wake-up delay defined in RAN1.

Proposal 7: The assumption of 3 and 5 SSB samples in wake-up delay definition in RAN1 is reasonable.

Proposal 8: No need to link the SSB number assumption to certain SINR condition.

Proposal 9: Confirm to RAN1 that for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

References

- R4-2504908, WF on RRM requirements for NR_LPWUS, vivo, RAN4#114bis

- R4-2503627, Topic summary for [114bis][217] NR_LPWUS, vivo, RAN4#114bis

- R4-2503110 (R1-2501624), LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode, Apple(RAN1), RAN4#114bis

- R4-2505309 (R1-2503103), LS on the RRM measurement metrics for OFDM-based LP-WUR, Apple(RAN1), RAN4#115

Annex A

3GPP TSG-RAN WG4 Meeting #115                                             R4-250xxxx

Malta, MT, 19th – 23th May, 2025

Title:	Reply LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode

Response to:		R4-2503110 (R1-2501624)

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

#### Name:	Qiuge Guo

E-mail Address:	guoqiuge@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1. Overall Description:

RAN4 thanks RAN1 for the LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode (	R4-2503110 (R1-2501624)). RAN4 has discussed assumptions in RAN1 LS and reached the following conclusion:

- The assumption of 3 and 5 SSB samples in wake-up delay definition in RAN1 is reasonable.

- Same SSB number is assumed for different SSB periodicities.

- When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay.

  - Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity

# 2. Actions:

To RAN1

ACTION: 	RAN4 kindly asks RAN1 to take the above information into account.

# 3. Date of Next RAN4 Meetings:

3GPP TSG RAN WG4#116	25 – 29 August, 2025		India

3GPP TSG RAN WG4#116bis	13 – 17 October, 2025		Prague

Annex B

3GPP TSG-RAN WG4 Meeting #115                                             R4-250xxxx

Malta, MT, 19th – 23th May, 2025

Title:	Reply LS on the RRM measurement metrics for OFDM-based LP-WUR

Response to:	R4-2505309 (R1-2503103)

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

#### Name:	Qiuge Guo

E-mail Address:	guoqiuge@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1. Overall Description:

RAN4 thanks RAN1 for the LS on the RRM measurement metrics for OFDM-based LP-WUR (	R4-2505309 (R1-2503103)). RAN4 has discussed the agreements in RAN1 LS and reached the following conclusion:

- RAN4 confirm that for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

# 2. Actions:

To RAN1

ACTION: 	RAN4 kindly asks RAN1 to take the above information into account.

# 3. Date of Next RAN4 Meetings:

3GPP TSG RAN WG4#116	25 – 29 August, 2025		India

3GPP TSG RAN WG4#116bis	13 – 17 October, 2025		Prague