3GPP TSG-RAN WG4 Meeting #115                                 R4-2505716

Malta, MT, 19th – 23th May, 2025

Title: 	Discussion on RRM requirements for low band carrier aggregation via switching

Source: 	OPPO

Agenda item:	7.7.4

Document for:	Approval

Introduction

In RAN#106 meeting, a new WID [1] on low band carrier aggregation via switching was agreed with the following objectives:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In last meeting, RAN4 discussed the related RRM requirements and the conclusions were captured in the approved WF [2]. In this document, we will further discuss the RRM requirements based on the open issues in [3].

Discussion

Issue 1-1-1: SSB assumptions for R19 LB CA via switching - Option 1 (OPPO, LGE, MTK, Nokia): RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching. - Option 1a (LGE): RAN4 to specify SSB-based SCell operation as baseline, and also specify SSB-less SCell operation. - Option 1b (MTK): RAN4 to focus on discussing and defining requirements for SDL SCell operation for SSB-based Scell, i.e. RRM to deprioritise the scenario of SSB-less Scell based. - Option 2 (Apple): RAN4 to use SSB-less based operation as baseline for Rel-19 low band CA via switching. - Option 3 (xiaomi): RAN4 shall discuss and define requirement for SDL SCell operation for both SSB-based and SSB-less. - Option 4 (Ericsson): If the SSB less SCell is considered to be in scope, use NES rel-18 WI as baseline for specifying LB-LB CA between collocated FDD-carrier and SDL-carrier for via switching in rel-19. Issue 1-1-2: MRTD assumptions for R19 LB CA via switching - FFS: - Option 1 (OPPO, vivo, QC, ZTE, Nokia, Xiaomi, CATT, Samsung, HW, MTK, LGE): The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. - Option 3 (Ericsson): MRTD = CP for in LB-LB CA with switching.

For the SSB assumption of LB CA via switching in Rel-19, considering the limited time, we prefer to start from SSB based operation which is general case and defining requirements for SSB-less operation if time allows. But based on the discussion in last meeting, the proponent of SSB-less operation clarified that it does not mean SSB is not transmitted but only means the SSB-less related requirements i.e., skipping L3 measurement on SCell are reused as baseline. If this is the case, we understand we do not need to agree on the SSB assumption but only to define a general requirement according to whether the L3 measurement can be skipped. Considering the target scenario is co-located and synchronized deployment, we are fine to consider SSB-less based requirements as baseline, i.e., to skip the L3 measurement on SDL SCell. But even if the SSB-less related requirements are reused, it does not mean the UE capability of Rel-18 NES is also needed in order to support LB CA via switching, and the requirements for LB CA via switching is irrelevant to the UE capability of Rel-18 NES.

For the MRTD assumption, based on the WID, co-located and synchronized network deployment for both carriers are considered, and both carriers are in a single TAG. Referring to legacy MRTD requirements for CA, we think 3us can be reused.

Proposal 1: RAN4 not to conclude on SSB based operation or SSB-less operation but to directly discuss the requirements and decide on whether the L3 measurement on SDL SCell can be skipped.

Proposal 2: The requirements for LB CA via switching is irrelevant to the UE capability of Rel-18 NES.

Proposal 3: The MRTD assumption between the two low bands can be 3us.

Regarding the switching pattern applicability, the following agreements were reached in last meeting:

Issue 1-1-3: switching pattern applicability for RRM requirement in R19 LB CA via switching Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement. Note: the above FFS parts will be discussed and decided in RAN4.

In last meeting, it was agreed that the switching pattern is not applied in SDL SCell activation procedure and deactivated SCell. During the activation procedure, UE can utilize the RS (e.g., SSB or CSI-RS) to perform SDL SCell activation. And the existing normal SCell activation procedure and requirements can be used as baseline. And at the RS occasion, UE will switch to SDL SCell and NW will not schedule on the PCell. For the deactivated SCell, to guarantee the UE power saving gain, UE should follow legacy measurement requirements.

Proposal 4: RAN4 to define the scheduling restriction or interruption caused by the measurements during SDL SCell activation procedure.

Proposal 5: For SDL SCell activation, the legacy normal SCell activation requirements can be used as baseline.

Proposal 6: The legacy deactivated SCell measurement requirements are applied for UE to perform deactivated SDL SCell measurement.

Issue 1-2-1A: Necessity of interruption requirement in RRM between FDD CC and SDL CC - Option 1 (Xiaomi, CATT, Apple, HW, Ericsson, ZTE): RAN4 RRM to define interruption requirement for carrier switching between FDD PCell and SDL SCell. - Option 1a (HW): Interruption requirement for carrier switching between FDD CC and SDL CC are to be defined, - Whether interruption is present on PCell depends on RAN1 decision on the location of switch gap. - The interruptions on other activated SCell are present. - Option 2 (OPPO, vivo, Nokia): RAN4 to not define interruption or scheduling restriction requirement for carrier switching between FDD PCell and SDL SCell. - Option 2a (Nokia): If a third carrier is configured without switching pattern, no interruption is allowed on the third carrier due to LBCA-sw operation on PCell and SDL SCell. - Option 3 (LGE): The interruption requirements should be discussed after the RF conclusion on time mask issue is reached discussion. Issue 1-2-1B: Necessity of interruption requirement in RRM when more carriers are considered Option 1 (Nokia): If a third carrier is configured without switching pattern, no interruption is allowed on the third carrier due to LBCA-sw operation on PCell and SDL SCell.

In last meeting, the interruption requirements for LB CA via switching was discussed but companies have different understanding. For the scenario in issue 1-2-1B, i.e., more carriers are configured, we think no requirements are needed in Rel-19 based on RF session agreement that only two bands are considered for low band carrier aggregation via switching in Rel-19.

For the scenario 1-2-1A, i.e., only one FDD CC and one SDL CC are configured, since RF will define the switching period in which both carriers are not required to receive/transmit signals, we think no additional interruption requirements are needed. But based on the discussion in last meeting, considering the switching period is based on DL timing and may have impact on PCell transmission due to timing advance, we are fine to consider the interruption on UL transmission.

Proposal 7: RAN4 to consider the interruption/scheduling restriction on UL transmission due to carrier switching between FDD PCell and SDL SCell.

RAN1#120bis meeting: Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2. - The pattern starts from the beginning of SFN 0 of PCell. - Restriction of maximum X switch(es) within Y slot(s). - FFS: detailed value and relation of X and Y. - FFS: other restriction. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms.

Based on RAN1 conclusion, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). For activated SDL SCC, we understand UE can perform SSB based L3 measurement when switching to SDL, and the measurement requirements can be defined based on SMTC/SSB periodicity and the camping time on SDL SCC according to the switching pattern defined by RAN1.

Proposal 8: RAN4 to define SDL activated SCell L3 measurement requirement based on the effective SMTC/SSB periodicity considering the overlapping with switching pattern.

For the other BFD, CBD, L1-RSRP/SINR requirements for both PCell and SCell, the existing requirements can be used as baseline. RAN4 to further discuss whether to extend the measurement requirements due to switching.

Proposal 9: RAN4 to discuss whether to extend the measurement requirements for BFD, CBD, L1-RSRP/SINR due to switching.

Conclusions

In this paper, we give our views on RRM requirements for low band carrier aggregation via switching. And the following proposals are provided.

Proposal 1: RAN4 not to conclude on SSB based operation or SSB-less operation but to directly discuss the requirements and decide on whether the L3 measurement on SDL SCell can be skipped.

Proposal 2: The requirements for LB CA via switching is irrelevant to the UE capability of Rel-18 NES.

Proposal 3: The MRTD assumption between the two low bands can be 3us.

Proposal 4: RAN4 to define the scheduling restriction or interruption caused by the measurements during SDL SCell activation procedure.

Proposal 5: For SDL SCell activation, the legacy normal SCell activation requirements can be used as baseline.

Proposal 6: The legacy deactivated SCell measurement requirements are applied for UE to perform deactivated SDL SCell measurement.

Proposal 7: RAN4 to consider the interruption/scheduling restriction on UL transmission due to carrier switching between FDD PCell and SDL SCell.

Proposal 8: RAN4 to define SDL activated SCell L3 measurement requirement based on the effective SMTC/SSB periodicity considering the overlapping with switching pattern.

Proposal 9: RAN4 to discuss whether to extend the measurement requirements for BFD, CBD, L1-RSRP/SINR due to switching.

Reference

- RP-243317, New WID on low band carrier aggregation via switching

- R4-2504895, WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2503614, Topic summary for [114bis][204] NR_LBCA_Sw, Apple