3GPP TSG-RAN WG4 Meeting #115						R4-2505729

Malta, MT, 19th – 23rd May, 2025

Source:	Nokia

Title:	On remaining UEIBM requirements

Agenda item:	7.24.4.1

Document for:	Discussion

Introduction

In this document, we present Nokia’s views regarding the remaining core requirements that RAN4 needs to discuss in relation to the UE-initiated beam management feature.

Discussion on reporting delay requirements

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

Agreement:

- For T_RS (TSSB or TCSI-RS) in L1_measurement periods

  - T_RS (TSSB or TCSI-RS):

    - the value of current beam and new beams are the same.

- For M in L1_measurement periods:

  - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam

  - If timeRestrictionForChannelMeasurement is not configured,

    - Further discuss on:

      - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used

      - Option 2: No requirements even the NW can configure it.

      - Option 3: FFS

- For P (sharing factor) in L1_measurement periods:

  - For SSB based or CSI-RS based,

    - The definition is the same as legacy

    - The value can be the same or different for the current beam and new beam

In the last RAN4 meeting, there was a general agreement on how to define the measurement period with some open points.

One of the open points in RAN4#114bis was the expected UE behaviour when timeRestrictionForChannelMeasurement is not configured. We see no real need to change the number of samples (i.e. current value of 3 samples) that the UE is allowed to use during the measurement period. However, unlike traditional L1-RSRP reporting, in the case of UEIBM, the UE needs to measure a new beam and compare it with a current beam to determine if the event triggering condition is met. In such cases, to ensure that the reporting is accurate, the same set of conditions during event evaluation should apply for the current beam as well as the new beam. Hence, when the timeRestrictionForChannelMeasurement is not configured, the UE shall use the same number of samples (i.e. M=3) for the current beam as well as the new beam.

For UEI beam report, UE needs to measure a new beam and compare it with a current beam to determine if the event triggering condition is met.

During event evaluation, same set of conditions should apply for the current beam and the new beam.

Issue 1-2: Components of Event triggered measurement reporting delay?

Agreement:

- The event triggered measurement reporting delay includes the Tfirst UL channel

  - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH

Further discuss the two options and select one option in the next meeting:

- The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel

  - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK)

  - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

When the timeRestrictionForChannelMeasurement is not configured, the UE shall use the same number of samples (i.e. M=3) for the current beam as well as the new beam.

RAN4 has agreed that the reporting delay shall in totality be the time from when an event that triggers the measurement report occurs to the time instance when the UE sends the first PUCCH at the first available PUCCH occasion after the event that triggered the measurement report has occurred. In the last RAN4 meeting the definition of the Tfirst UL channel was agreed as well, while the definition of TL1_measurement period was left open.

Defining the TL1_measurement period based on the maximum of the L1 measurement periods of the current beam and the new beam will not accurately reflect the measurement period as the event can be triggered by the measurement of the current beam or the measurement of the new beam. Instead, the measurement period should be defined based on the RS which actually determined the triggering of the event which in the case of UEI beam report can be the triggered by the current beam or the new beam.

The event triggered measurement delay shall be defined based on the measurement period of the RS which determines the triggering of the UEI event.

If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement period_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement period_newbeam.

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

<Way forward >

- Option 1: (Qualcomm, Xiaomi, Apple, LGE)

  - Do not define requirements for eventInstanceCount-r19 > 1

- Option 1a: (Apple): To extend the requirements for case when eventInstanceCount-r19 > 1 is configured, add note longer delay is expected if eventInstanceCount-r19 > 1 is configured.

- Option 2: (Samsung, CMCC, Huawei, ZTE, MediaTek)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

- Option 3: (Nokia, Ericsson)

  - Wait for further RAN1 conclusion

- For how to specify the requirements in option 2 above:

  - Option 2a:

    - Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

    - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

    - C: event instances threshold is configured by NW.

    - Ttime_window: time window is configured by NW.

    - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

  - Other options are not precluded

Since a practical network deployment would have a mix of UEs, i.e., UEs only supporting the basic feature and UEs supporting the optional capability, RAN4 requirements should be defined for both the basic and the optional case.

Until the last meeting, RAN1 details about the window based reporting have not been sufficient to be reflected to RAN4 requirements. However, in the last meeting RAN1 has finally concluded that a semi-sliding window approach is to be used as shown in the below agreement:

Agreement (RAN1 #120Bis Wuhan) Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications.

The supported option 4 means that the window-related timer is started at the first event instance where the reporting condition is met for a new beam. Only one timer per new beam can be running at a time.

According to the RAN1 agreements and the draft CR, the UE shall trigger the transmission of the report immediately when the configured number M measurement occasions have occurred within the window/timer. We see no issue in defining a RAN4 reporting delay requirement by taking this into account. Hence, for completeness of the work item, we propose to define a separate event-triggered reporting delay requirement which takes the window/timer and the M samples that need to fulfil the condition into account.

Define reporting delay requirement also for the window/timer based approach, where the delay takes into account that the UE shall send the report after M event occasions have occurred within the window/timer.

Discussion on TCI state switching

Issue 1-7: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

<Way forward >

- Option 1: (Samsung)

  - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Option 2:

  - For such case, No RAN4 requirements.

- Option 3: (Apple)

  - Add the applicability rule of event triggered L1 measurement:

    - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

    - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

In the last RAN4 meeting, the following was left as open issue for discussion [1]:

On the topic of resetting the counting of beams for event trigger determination, the following was discussed and agreed in RAN1#120 and in RAN1#120-bis

Since the UE needs to reset and restart the counting for all the new beams when the indicated TCI state is changed, the evaluation of the determination of event triggering condition being fulfilled shall start once the UE has switched to the target TCI state. However, this is something that is to be defined by RAN1/2, not RAN4. From RAN4 point of view this would in our view be considered as a corner case, and hence we do not see a need for RAN4 to specifically define event-triggered reporting requirements or a corresponding test case for this scenario.

Issue 1-6: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- For MAC-CE based DL/UL TCI state switch delay

  - Option 1: (Qualcomm, Apple, Samsung (for known case), Ericsson)

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

  - Option 2: (ZTE)

    - Remove components of TL1-RSRP and Tfirst-SSB+TSSB-proc

  - Option 3: (MediaTek)

    - FFS on whether to remove Tfirst-SSB+TSSB-proc

- For DCI based DL/UL TCI state switch delay

  - Option 1:

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

  - Option 2:

    - Whether to further reduce the delay, wait for further RAN1 progress

If the indicated TCI state changes during the L1 measurement time, the evaluation of the event trigger determination shall start after the TCI state switch is completed. However, there is no need to capture this scenario in RAN4 requirements.

Agreement (RAN1 #120 Athens)

Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting.

- Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received;

  - FFS: whether to reset the counting for all new beams.

  - FFS: whether to maintain the counting whose new beam is NOT updated.

- Candidate#2: [The measured current beam based on] indicated TCI state is updated;

  - In such case, the UE need to reset the counting for all new beams.

- Candidate#3: UEI beam report is transmitted;

  - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report

- Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected.

- Candidate#5: The time window expires

- Candidate#6: The threshold for event evaluation is re-configured by RRC signaling

- (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured.

  - FFS: RRC parameter(s)

- FFS: Other candidates

Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point.

Agreement (RAN1 #120Bis Wuhan)

Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported.

- Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated;

  - In such case, the UE needs to reset the counting for all new beams.

  - FFS: Further details on the update (if necessary)

From RAN4 perspective, there should be no need to update the existing TCI state switching requirements based on UEIBM report. However, in RAN1#120 meeting various alternatives about beam application latency reduction after a UEI beam report is sent were discussed and agreed to be studied. No conclusion was reached about whether or which alternative to use.

Agreement (RAN1 #119 Orlando)

Study the following to reduce beam application latency after a UEI/ED beam report is sent

- Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

- after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

- update TCI state(s) with the reported new beam(s)

- activate new beam(s) without additional SSB reception

- Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception.

- Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent

- FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized)

- Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

- In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

- Other alternatives are not precluded

Based on the proposals in RAN1, the intention is that the UE would be DL synchronized with the RS reported in the event-triggered L1 report due to which the TCI states can be activated without the need for DL sync i.e. within Tharq+3ms after the reception of the MAC-CE command for TCI state activation.

- Various alternatives to reduce the beam application latency were proposed in RAN1, some of which may have an impact on the TCI state switch delay.

From RAN4 requirements point of view, the reduction in the TCI state activation delay will depend on whether the UE has performed a DL synchronization with the RS reported in the UEI report before the reception of the TCI state switch command. If UE performs the DL sync, there can be the following two scenarios where the UE can do this:

- Prior to sending the UEI beam report while doing the measurement of the RS configured with UEI event.

- When UEI event becomes fulfilled or after sending the UEI beam report and before receiving the TCI state activation command.

Depending on which option applies and depending on the time period between sending the report and receiving the TCI state activation command, the UE may or may not have DL synchronization at the time of receiving the TCI state activation command. Therefore, if the DL sync triggered by the report as discussed in RAN1 will be agreed, we propose that RAN4 discusses whether the UE is expected to have performed the DL synchronization before sending the report or will UE trigger DL synchronization based on the reporting condition becoming fulfilled.

If UE initiated DL synchronization based on UEIBM report is supported (as discussed by RAN1), RAN4 to discuss the time point at which the UE can be expected to have performed DL synchronization: before sending the UEI beam report or after the report is sent.

Issue 1-5: CSI-RS based L1-RSRP for inter-cell BM

<Way forward >

- Option 1:

  - Not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management

- Option 2:

  - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

Issue 1-9: Requirements for event-1 and event-7?

<Way forward >

- Option 1: When requirements for Event-2 are agreed, agree to reuse all requirements for Event-1 and Event-7.

- Option 2: others are not precluded

Discussion on other measurement requirements

There are currently no baseline RAN4 requirements defined for inter-cell CSI-RS based L1-RSRP measurements for beam management. Hence, unless a clear use case is identified we propose not to introduce CSI-RS based L1-RSRP requirements for inter-cell beam management.

No baseline RAN4 requirements defined for CSI-RS based L1-RSRP measurements for inter-cell beam management.

RAN4 not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not defined a clear use case.

On the topic of defining requirements for event-1 and event-7, RAN1 has agreed to reuse the same design as event-2 for all other supported events. In view of this we see no need for separate set of requirements for event-1 and event-7 at least for the basic single measurement based event determination.

Agreement (RAN1 #119 Orlando)

On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed:

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

- Q is RRC configured with subjective to UE capability signalling

- UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

RAN1 has agreed to reuse the same design as event-2 for all other supported events

Requirements defined for non-window based event determination shall be applicable to event-2 and any other supported event which reuses the same design as event-2.

Issue 1-10: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

<Way forward >

Further discuss the two options. Proponents of option 2 to bring proposals on the exact numbers for the capabilities.

- Option 1:

  - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

- Option 2:

  - Define capabilities for support of UE-initiated/event-driven beam reporting similar as L3.

Issue 1-11: Compatibility of capabilities with LTM

<Way forward >

- Option 1: (Apple, Ericsson)

  - Discuss it later.

- Option 2: (CMCC, Huawei, Nokia)

  - Not preferred to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO

- Option 3: (MediaTek)

  - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

UE capabilities

For L3 event-triggered reporting, RAN4 has defined capabilities for the support of event-triggered reporting, where the UE shall be able to support in parallel per category up to Ecat reporting criteria according to table 9.1.4.2-1. In the last meeting RAN4 discussed whether there is a need to update the existing table or add new table in section 9.1.4 for the number of events supported per category (Ecat) for L1 events.

Firstly, the L3 events per category obviously do not cover the L1 events. The L1 events the UE shall be able to support should be added on top of the existing L3 events i.e. introduction of L1 events shall not impact the existing UE capabilities to support L3 events. Hence, to ensure that the network is aware of the number of events that can be configured for a UE, we think RAN4 needs to define either a new table for L1 events to be supported or add new rows to the existing Ecat table to cover intra- and inter-frequency UEIBM events to be supported.

Introduce the number of supported UEIBM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4.

Within the NR_Mob_Ph4 work item, it was already agreed not to discuss common capabilities between mobility and MIMO work items, at least before the feature list discussion. We propose to make the same agreement for MIMO as was captured in NR_Mob_Ph4 way forward in RAN4#114bis (R4-2504965):

Issue 2-5: whether and how to coordinate with R19 MIMO Agreement: - RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

Reuse the agreement under NR_Mob_Ph4 regarding coordination between mobility and MIMO events: RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

Conclusion

In this paper, the following observations and proposals were made:

Observation 1: For UEI beam report, UE needs to measure a new beam and compare it with a current beam to determine if the event triggering condition is met.

Observation 2: During event evaluation, same set of conditions should apply for the current beam and the new beam.

Proposal 1: When the timeRestrictionForChannelMeasurement is not configured, the UE shall use the same number of samples (i.e. M=3) for the current beam as well as the new beam.

Proposal 2: The event triggered measurement delay shall be defined based on the measurement period of the RS which determines the triggering of the UEI event.

Proposal 3: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement period_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement period_newbeam.

Proposal 4: Define reporting delay requirement also for the window/timer based approach, where the delay takes into account that the UE shall send the report after M event occasions have occurred within the window/timer.

Proposal 5: If the indicated TCI state changes during the L1 measurement time, the evaluation of the event trigger determination shall start after the TCI state switch is completed. However, there is no need to capture this scenario in RAN4 requirements.

Proposal 6: If UE initiated DL synchronization based on UEIBM report is supported (as discussed by RAN1), RAN4 to discuss the time point at which the UE can be expected to have performed DL synchronization: before sending the UEI beam report or after the report is sent.

Observation 2: No baseline RAN4 requirements defined for CSI-RS based L1-RSRP measurements for inter-cell beam management.

Proposal 7: RAN4 not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not defined a clear use case.

Observation 3: RAN1 has agreed to reuse the same design as event-2 for all other supported events

Proposal 8: Requirements defined for non-window based event determination shall be applicable to event-2 and any other supported event which reuses the same design as event-2.

Proposal 9: Introduce the number of supported UEIBM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4.

Proposal 10: Reuse the agreement under NR_Mob_Ph4 regarding coordination between mobility and MIMO events: RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

- References

- R4-2504902     WF on RRM requirements for NR_MIMO_Ph5_Part1