3GPP TSG-RAN WG4 Meeting #115	R4-2505730

Malta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	On performance requirements for MIMO Ph5

Agenda item:	7.24.5

Document for:	Discussion

Introduction

In this contribution we discuss the necessary test cases for the objectives of the NR_MIMO_Ph5 work item.

UEIBM

For UEIBM, RAN4 has discussed and agreed to define the event-triggered L1 UEIBM reporting delay requirements. These apply to FR2 intra-cell SSB and CSI-RS based reporting, and inter-cell SSB based reporting. CSI-RS based inter-cell reporting is still under discussion in the core part. The reporting based on Mode A and Mode B is supported in the delay requirement depending on UE support of these reporting modes. Currently RAN4 has agreed to introduce reporting delay for the single measurement based report triggering. Window-based reporting requirements are still under discussion, and RAN4 should agree about the relevant test cases for this option if RAN4 defines the corresponding core requirements.

To cover the applicable reporting delay requirements, the following test case list should be considered:

- Reporting based on single measurement:

  - Intra-cell SSB-based event-triggered reporting delay

  - Inter-cell SSB-based event-triggered reporting delay

  - Intra-cell CSI-RS based event-triggered reporting delay

  - FFS: Inter-cell CSI-RS based event-triggered reporting delay

- FFS: Window-based reporting

RAN4 to consider the following test case list for UEIBM:

  - Reporting based on single measurement:

    - Intra-cell SSB-based event-triggered reporting delay

    - Inter-cell SSB-based event-triggered reporting delay

    - Intra-cell CSI-RS based event-triggered reporting delay

  - FFS: Inter-cell CSI-RS based event-triggered reporting delay

  - FFS: Window-based reporting

Furthermore, RAN4 shall discuss how to take Mode A and Mode B into account in the testing. Mode A is the basic UE mode of operation, i.e., all UEs supporting UEIBM reporting should support Mode A, whereas Mode B is subject to UE capabilities, i.e. only a subset of the UEs supporting UEIBM may support Mode B. It is straightforward to say that if the UE supports only Mode A, the UE shall be tested with that. But if the UE supports both modes, RAN4 to discuss whether the UE should be tested with also Mode B. Furthermore, it has to be discussed how to define the test cases for the two modes if UE is to be tested with both. Initially we would think there is no need to duplicate the test cases, but the modes can be listed as options under each test case, and the mode to be used for each test shall be selected.

UE to be tested with reporting Mode A, if the UE only supports that. FFS whether to test the UE also with Mode B if the UE supports both modes.

If Mode B tests are to be included, introduce reporting Mode A and Mode B as optional test configurations under each UEIBM test. No need to define separate test cases for Mode A and Mode B.

UL only TRP

The main change for UL TRP in MIMO Ph5 is that 2 TA can be used for sDCI, and that the 2nd TRP doesn’t necessarily have reference signal for timing reference.

Include UL timing accuracy test cases with 2 TAs for UL-only TRP with 2 TAs for

  - Scenario#1, UL-TRP does not transmit RSs.

  - Scenario#2, UL-TRP transmits RSs.

Reporting for CJT

Regarding CJT, the main RRM requirements that have been agreed so far are for CJT reporting.

RAN4 to define test cases for measurement delay of CJT reporting.

Conclusion

The following Observations and Proposals were made:

Proposal 1: RAN4 to consider the following test case list for UEIBM:

a.	Reporting based on single measurement:

i.	Intra-cell SSB-based event-triggered reporting delay

ii.	Inter-cell SSB-based event-triggered reporting delay

iii.	Intra-cell CSI-RS based event-triggered reporting delay

b.	FFS: Inter-cell CSI-RS based event-triggered reporting delay

c.	FFS: Window-based reporting

Proposal 2: UE to be tested with reporting Mode A, if the UE only supports that. FFS whether to test the UE also with Mode B if the UE supports both modes.

Proposal 3: If Mode B tests are to be included, introduce reporting Mode A and Mode B as optional test configurations under each UEIBM test. No need to define separate test cases for Mode A and Mode B.

Proposal 4: Include UL timing accuracy test cases with 2 TAs for UL-only TRP with 2 TAs for

a.	Scenario#1, UL-TRP does not transmit RSs.

b.	Scenario#2, UL-TRP transmits RSs.

Proposal 5: RAN4 to define test cases for measurement delay of CJT reporting.