3GPP TSG-RAN4 Meeting #115	R4-2505737

# St Julians, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR for 38.101-5 Ku bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Eutelsat Group |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-05-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Running CR for agreements to specify Ku bands |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to section 5 channel arrangement endorsed at RAN4 #114bis in R4-2504721 and with the draft CR in R4-2508771 endorsed at RAN4 #115. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operaton in Ku bands will not be possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present Release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future Releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NTN satellite can operate according to this version of the specification are identified as described in Table 5.1-1.

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

## 5.2	Operating bands

### 5.2.1	General

NTN satellite covers FR1-NTN and FR2-NTN operating bands in the present specification.

### 5.2.2	Operating bands for FR1-NTN

NTN satellite is designed to operate in the operating bands defined in Table 5.2.2-1.

Table 5.2.2-1: NTN satellite bands in FR1-NTN

| NTN satellite operating band1 | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |
| n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127502,3 MHz | FDD |
| n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127502,3 MHz | FDD |
| NOTE 1: NTN satellite bands are numbered in descending order from n256. NOTE 2: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 3: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT |  |  |  |

### 5.2.3	Operating bands for FR2-NTN

NTN satellite is designed to operate in the operating bands defined in Table 5.2.3-1.

Table 5.2.3-1: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n512(Note 1) | 27500 MHz - 30000 MHz | 17300 MHz - 20200 MHz | FDD |
| n511(Note 2) | 28350 MHz - 30000 MHz | 17300 MHz - 20200 MHz | FDD |
| n510(Note 3) | 27500 MHz - 28350 MHz | 17300 MHz - 20200 MHz | FDD |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 127504,5 MHz | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 127504,5 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries subject to or referring to FCC 47 CFR part 25. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). NOTE 4: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 5: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. |  |  |  |

## 5.3	UE channel bandwidth

### 5.3.1	General

The UE channel bandwidth supports a single RF carrier in the uplink or downlink at the UE. From a SAN perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the SAN.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the SAN channel bandwidth or how the SAN allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the SAN channel bandwidth.

The relationship between the channel bandwidth, the guardband and the maximum transmission bandwidth configuration is shown in Figure 5.3.1-1.

Figure 5.3.1-1: Definition of the channel bandwidth and the maximum transmission bandwidth configuration for one channel

### 5.3.2	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1 for FR1-NTN and table 5.3.2-2 for FR2-NTN.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB for FR1-NTN

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 270 | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 133 | 189 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 65 | 93 | 135 |

Table 5.3.2-2: Maximum transmission bandwidth configuration NRB for FR2-NTN

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

### 5.3.3	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each UE channel bandwidth and SCS is specified in Table 5.3.3-1 for FR1-NTN and in table 5.3.3-2 for FR2-NTN.

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz) for FR1-NTN

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 692.5 | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 1045 | 965 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1570 | 1490 | 1370 |

Table 5.3.3-2: Minimum guardband for each UE channel bandwidth and SCS (kHz) for FR2-NTN

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N/A |
| 120 | 1900 | 2420 | 4900 | 9860 |

NOTE:	The minimum guardbands have been calculated using the following equation: (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-1 and Table 5.3.2-2.

Figure 5.3.3-1: Void

The number of RBs configured in any channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.3-2: UE PRB utilization

In the case that multiple numerologies are multiplexed in the same symbol, the minimum guard band on each side of the carrier is the guard band applied at the configured UE channel bandwidth for the numerology that is transmitted/received immediately adjacent to the guard band.

Figure 5.3.3-3: Guard band definition when transmitting multiple numerologies

NOTE:	Figure 5.3.3-2 is not intended to imply the size of any guard between the two numerologies. Inter-numerology guard band within the carrier is implementation dependent.

### 5.3.4	RB alignment

The RB alignment for FR1-NTN refers to NR RB alignments as specified in 3GPP TS 38.101-1 [5] clause 5.3.4.

The RB alignment for FR2-NTN refers to NR RB alignments as specified in 3GPP TS 38.101-2 [15] clause 5.3.4.

### 5.3.5	UE channel bandwidth per operating band

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 for FR1-NTN and table 5.3.5-2 for FR2-NTN. The transmission bandwidth configuration in Table 5.3.2-1 and Table 5.3.2-2 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the Tx and Rx path.

Table 5.3.5-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |
| n256 | 30 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |
| n255 | 30 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |
| n254 | 30 |  | 10 | 15 |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 |  |  |  |  |  |  |  |
| n248 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.5-2: Channel bandwidths for each NTN satellite band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n511 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n510 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n509 | 120 | 50 | 100 | 200 | 400 |
| n508 | 120 | 50 | 100 | 200 | 400 |

### 5.3.6	Asymmetric channel bandwidths

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in table 5.4.4-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

The operating bands and supported asymmetric channel bandwidth combinations are defined in table 5.3.6-1.

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n254 | 5 | 10,15 | 0 |
|  | 10 | 15 | 0 |
| n248, n247 | 10 | 15, 25, 35, 50 | 0 |
|  | 15 | 10, 25, 35, 50 | 0 |
|  | 25 | 10, 15, 35, 50 | 0 |
|  | 35 | 10, 15, 25, 50 | 0 |
|  | 50 | 10, 15, 25, 35 | 0 |
|  | 10 | 20, 25, 35, 50, 70 | 1 |
|  | 20 | 10, 25, 35, 50, 70 | 1 |
|  | 25 | 10, 20, 35, 50, 70 | 1 |
|  | 35 | 10, 20, 25, 50, 70 | 1 |
|  | 50 | 10, 20, 25, 35, 70 | 1 |
|  | 70 | 10, 20, 25, 35, 50 | 1 |
|  | 10 | 20, 25, 35, 50, 70, 100 | 2 |
|  | 20 | 10, 25, 35, 50, 70, 100 | 2 |
|  | 25 | 10, 20, 35, 50, 70, 100 | 2 |
|  | 35 | 10, 20, 25, 50, 70, 100 | 2 |
|  | 50 | 10, 20, 25, 35, 70, 100 | 2 |
|  | 70 | 10, 20, 25, 35, 50, 100 | 2 |
|  | 100 | 10, 20, 25, 35, 50, 70 | 2 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [11], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

## 5.4	Channel arrangement

### 5.4.1	Channel spacing

#### 5.4.1.1	Channel spacing for adjacent NTN satellite carriers

The channel spacing for adjacent NTN satellite carriers in FR1-NTN refers to the NR channel spacing as specified in 3GPP TS 38.101-1 [5] clause 5.4.1.1.

The channel spacing for adjacent NTN satellite carriers in FR2-NTN refers to the NR channel spacing as specified in 3GPP TS 38.101-2 [15] clause 5.4.1.1.

### 5.4.2	Channel raster

#### 5.4.2.1	NR-ARFCN and channel raster

The global frequency channel raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements.

The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range (0…2016666) on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NREF-Offs are given in Table 5.4.2.1-1 and NREF is the NR-ARFCN.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |
| 24250 – 30000 | 60 | 24250.08 | 2016667 | 2016667 – 2112499 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

For the uplink of FDD FR1 NTN bands defined in Table 5.2.2-1 except n247, n248.

FREF, shift = FREF + Δshift, Δshift = 0 kHz or 7.5 kHz.

where Δshift is signalled by the network in higher layer parameter frequencyShift7p5khz [7].

The mapping between the channel raster and corresponding resource element is given in clause 5.4.2.2. The applicable entries for each operating band are defined in clause 5.4.2.3.

#### 5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on the channel raster and the corresponding resource element for FR1-NTN refers to the NR requirements specified in 3GPP TS 38.101-1 [5] clause 5.4.2.2.

The mapping between the RF reference frequency on the channel raster and the corresponding resource element for FR2-NTN refers to the NR requirements specified in 3GPP TS 38.101-2 [15] clause 5.4.2.2.

#### 5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NTN satellite operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1 and Table 5.4.2.3-2 for FR1-NTN, and in Table 5.4.2.3-3 for FR2-NTN, using the channel raster to resource element mapping in clause 5.4.2.2.

For NTN satellite operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <20> for FR1-NTN.

For FR1-NTN satellite operating bands with 15 kHz and 30 kHz channel raster above 10 GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {1,2}. Every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <I>.

For frequency bands with two ΔFRaster in FR1, the higher ΔFRaster applies to channels using only the SCS that is equal to the higher ΔFRaster and SSB SCS is equal to the higher ΔFRaster.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
|  | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 15 | 1316667 – <1> – 1333333 | 1113334 – <1> – 1250000 |
|  | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

For NTN operating bands with 100 kHz channel raster, Enhanced channel raster is defined with ΔFRaster = 2 × ΔFGlobal. In this case every 2th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-2 is given as <2>.

Table 5.4.2.3-2: Applicable NR-ARFCN per operating band in FR1-NTN for enhanced channel raster

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| n256 | 10 | 396000 – <2> – 402000 | 434000 – <2> – 440000 | Yes |
| n255 | 10 | 325300 – <2> – 332100 | 305000 – <2> – 311800 | Yes |
| n254 | 10 | 322000 – <2> – 325300 | 496700 – <2> – 500000 | Yes |
| NOTE: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |  |

For FR2-NTN satellite operating bands, ΔFRaster = I1 × ΔFGlobal for UL channel and ΔFRaster = I2 × ΔFGlobal for DL channel, where (I1, I2) ϵ {(1,4) or (2, 8) or (8.8)}. But (I1, I2) = (2, 8) only applies under the condition that 120kHz SCS is configured in the channel and SSB SCS is equal to or larger than 120kHz, and (I1, I2) = (8, 8) only applies under the condition that 120kHz SCS is configured in the channel and SSB SCS is equal to 120kHz. In this case, every I1th NR-ARFCN for UL channel and I2th NR-ARFCN for DL channel are applicable for the UL and DL channel raster correspondingly within the operating band and the <(UL_step size, DL_step size)> for the UL and DL channel raster in Table 5.4.2.3-3 is given as <(I1, I2)> for FR2-NTN.

Table 5.4.2.3-3: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 60 | 2070833 – <1> – 2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2112499 | 1553336 – <8> – 1746664 |
| n511 | 60 | 2084999 – <1> –2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2084999 – <2> –2112499 | 1553336 – <8> – 1746664 |
| n510 | 60 | 2070833 – <1> – 2084999 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2084999 | 1553336 – <8> – 1746664 |
| n509 | 120 | 1333336 – <8> – 1366664 | 1113336 – <8> – 1250000 |
| n508 | 120 | 1316672 – <8> – 1333328 | 1113336 – <8> – 1250000 |

### 5.4.3	Synchronization raster

#### 5.4.3.1	Synchronization raster and numbering

The synchronization raster indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition when explicit signalling of the synchronization block position is not present.

A global synchronization raster is defined for all frequencies. The frequency position of the SS block is defined as SSREF with corresponding number GSCN. The parameters defining the SSREF and GSCN for all the frequency ranges are in Table 5.4.3.1-1.

The resource element corresponding to the SS block reference frequency SSREF is given in clause 5.4.3.2. The synchronization raster and the subcarrier spacing of the synchronization block is defined separately for each band.

The synchronization raster and the corresponding SS block do not cover all possible RF channel bandwidth and locations on Enhanced channel raster.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5}1 | 3N + (M-3)/2 | 2 – 7498 |
| 3000 – 24250 | 3000 MHz + N * 1.44 MHz, N = 0:14756 | 7499 + N | 7499 – 22255 |
| NOTE: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

#### 5.4.3.2	Synchronization raster to synchronization block resource element mapping

The mapping between the synchronization raster and the corresponding resource element of the SS block in FR1-NTN refers to 3GPP TS 38.101-1 [5] clause 5.4.3.2.

The mapping between the synchronization raster and the corresponding resource element of the SS block in FR2-NTN refers to 3GPP TS 38.101-2 [15] clause 5.4.3.2.

#### 5.4.3.3	Synchronization raster entries for each operating band

The synchronization raster for each band is give in Table 5.4.3.3-1 and Table 5.4.3.3-2. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 5.4.3.3-1 for FR1-NTN and Table 5.4.3.3-2 for FR2-NTN.

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6220 – <1> – 6238 |
| n248 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n509 | 120 kHz | Case D | 12864 – <12> – 14256 |
| n508 | 120 kHz | Case D | 12864 – <12> – 14256 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

### 5.4.4	TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 5.4.4-1 for FR1-NTN.

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz1 165 to 215 MHz2 |
| n255 | -101.5 MHz1 -72.5 to -130.5 MHz2 |
| n254 | 862 – 885 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

[Next modified section]

# 9	Radiated transmitter characteristics

## 9.1	General

Unless otherwise stated, the transmitter characteristics are specified over the air (OTA) with a single or multiple transmit chains under either LHCP (Left Hand Circular Polarization) or RHCP (Right Hand Circular Polarization) or Linear Polarization.

## 9.2	Transmitter power

### 9.2.1	NTN VSAT maximum output power

#### 9.2.1.0	General

The NTN VSAT classes are specified based on the assumptions of certain NTN VSAT types with specific device architectures including antenna beam steering types. The requirements are specified for different NTN VSAT types. For the hybrid beam steering capable NTN VSAT, which can adjust its antenna(s) or beam(s) in both electronic steering and mechanical steering ways, the applicable requirements shall follow either electronic or mechanical beam steering requirements depending on the NTN VSAT type it declared. The NTN VSAT types are specified in Table 9.2.1.0-1 below.

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. |  |  |

#### 9.2.1.1	Minimum requirements for Fixed VSAT

The following requirements define the maximum output power radiated by the Fixed VSAT for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are specified in Table 9.2.1.1-1. The requirement should be verified with test metrics of EIRP (Link=Tx beam peak direction, Meas=Link angle).The peak EIRP of Tx beam peak direction should be verified within the declared minimum elevation angle supported for transmitting. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type. Where the supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 9.2.1.1-1 below is taken as an example.

Figure 9.2.1.1-1 Example measurement grid for min peak EIRP with the declared supported minimum elevation angle

Table 9.2.1.1-1: Minimum peak EIRP for Fixed VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |
| --- | --- | --- |
| n512, n511, n510 | 1 | 70 |
|  | 2 | 70 |
|  | 3 | 61 |
| NOTE: Minimum peak EIRP is defined as the lower limit without tolerance. |  |  |

The maximum output power values for TRP and EIRP for fixed VSAT are specified in Table 9.2.1.1-2 below.

Table 9.2.1.1-2: Maximum output power limits for Fixed VSAT

| Operating band | UE Type | TRPMAX (dBm) | EIRPmax (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 1 | 35 | 76.2 |
|  | 2, 3 | 43 | 76.2 |
| NOTE: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. |  |  |  |

#### 9.2.1.2	Minimum requirements for Mobile VSAT

The following requirements define the maximum output power radiated by the Mobile VSAT for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are specified in Table 9.2.1.2-1. The requirement should be verified with test metrics of EIRP (Link=Tx beam peak direction, Meas=Link angle).

The peak EIRP of Tx beam peak direction should be verified within the declared minimum elevation angle supported for transmitting. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type. Where the supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 9.2.1.2-1 below is taken as an example.

Figure 9.2.1.2-1 Example measurement grid for min peak EIRP with the declared supported minimum elevation angle

Table 9.2.1.2-1: Minimum peak EIRP for Mobile VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |
| --- | --- | --- |
| n512, n511 | 4 | 70 |
|  | 5 | 70 |
| NOTE: Minimum peak EIRP is defined as the lower limit without tolerance. |  |  |

The maximum output power values for TRP and EIRP for mobile VSAT are specified in Table 9.2.1.2-2 below.

Table 9.2.1.2-2: Maximum output power limits for Mobile VSAT

| Operating band | UE Type | TRPMAX (dBm) | EIRPmax (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 4 | 35 | 76.2 |
|  | 5 | 43 | 76.2 |
| NOTE: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. |  |  |  |

### 9.2.2	Off-axis EIRP emission density limit within the operating band

#### 9.2.2.1	General

The Off-axis EIRP density envelope is applicable within the band to NTN VSAT transmitting towards geostationary satellite orbit.

#### 9.2.2.2	Minimum requirement for bands n510 and n511

For co-polarized transmissions in the plane tangent to the GSO arc, the requirements specified in Table 9.2.2.2-1 apply to NTN VSAT.

Table 9.2.2.2-1: Off-axis EIRP density limits for co-polarized transmissions in the plane tangent to the GSO arc

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 62.5 – 25log10(θ) | 1 |
| 7° ≤ θ ≤ 9.2° | 41.5 | 1 |
| 9.2° ≤ θ ≤ 19.1° | 65.5 – 25log10(θ) | 1 |
| 19.1° < θ ≤ 180° | 33.5 | 1 |

For co-polarized transmissions in the plane perpendicular to the GSO arc, the requirements specified in Table 9.2.2.2-2 apply to NTN VSAT.

Table 9.2.2.2-2: Off-axis EIRP density limits for co-polarized transmissions in the plane perpendicular to the GSO arc

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 3.5° ≤ θ ≤ 7° | 65.5 – 25log10(θ) | 1 |
| 7° ≤ θ ≤ 9.2° | 44.4 | 1 |
| 9.2° ≤ θ ≤ 19.1° | 68.5 – 25log10(θ) | 1 |
| 19.1° < θ ≤ 180° | 36.5 | 1 |

The EIRP density levels specified in Table 9.2.2.2-1 and Table 9.2.2.2-2 may be exceeded by up to 3 dB, for values of θ > 7°, over 10% of the range of theta (θ) angles from 7–180° on each side of the line from the NTN VSAT to the target SAN.

For cross-polarized transmissions in the plane tangent to the GSO arc and in the plane perpendicular to the GSO arc, the requirements specified in Table 9.2.2.2-3 apply to NTN VSAT.

Table 9.2.2.2-2: Off-axis EIRP density limits for cross-polarized transmissions in the plane tangent to the GSO arc and in the plane perpendicular to the GSO arc

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 52.5 – 25log10(θ) | 1 |

#### 9.2.2.3	Minimum requirement for band n512

##### 9.2.2.3.1	Fixed VSAT

For co-polarized transmissions, the regional requirements specified in Table 9.2.2.3.1-1 apply to Fixed VSAT type 1 or 2 when transmitting towards satellites in geostationary orbit.

Table 9.2.2.3.1-1: Off-axis EIRP density limits for co-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.8° ≤ θ ≤ 7° | 49 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 28 – K | 40 |
| 9.2° ≤ θ ≤ 48° | 52 – 25log10(θ) – K | 40 |
| 48° < θ | 20 – K | 40 |
| NOTE: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Fixed VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.4.2 in [18]. The manufacturer shall declare the value of N. |  |  |

For cross-polarized transmissions, the regional requirements specified in Table 9.2.2.3.1-2 apply to Fixed VSAT type 1 or 2 when transmitting towards satellites in geostationary orbit.

Table 9.2.2.3.1-2: Off-axis EIRP density limits for cross-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.8° ≤ θ ≤ 7° | 39 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 18 – K | 40 |
| NOTE: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Fixed VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.4.2 in [18]. The manufacturer shall declare the value of N. |  |  |

##### 9.2.2.3.2	Mobile VSAT

For co-polarized transmissions, the regional requirements specified in Table 9.2.2.3.2-1 apply to Mobile VSAT.

Table 9.2.2.3.2-1: Off-axis EIRP density limits for co-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 49 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 28 – K | 40 |
| 9.2° ≤ θ ≤ 48° | 52 – 25log10(θ) – K | 40 |
| 48° < θ ≤ 180° | 20 – K | 40 |
| NOTE1: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17]. The manufacturer shall declare the value of N. NOTE2: The manufacturer shall declare the operational conditions of the system e.g. motion of the platform with 6 degrees of freedom and the duration for which the limits will not be exceeded for more than 0,01% of the time. |  |  |

Mobile VSAT with low elevation angles may exceed the limits specified in Table 9.2.2.3.2-1 by the amount specified in Table 9.2.2.3.2-2.

Table 9.2.2.3.2-2: Off-axis EIRP density limits for co-polarized transmissions

| Elevation angle to Satellite (ε) | Increase in EIRP density (dB) |
| --- | --- |
| ε < 5o | 2.5 |
| 5o < ε < 30o | 3 – 0.1 * ε |

For cross-polarized transmissions, the regional requirements specified in Table 9.2.2.3.2-3 apply to Mobile VSAT.

Table 9.2.2.3.2-3: Off-axis EIRP density limits for cross-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 39 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 18 – K | 40 |
| NOTE 1: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17]. The manufacturer shall declare the value of N. NOTE 2: The manufacturer shall declare the operational conditions of the system e.g. motion of the platform with 6 degrees of freedom and the duration for which the requirement will not be exceeded for more than 0,01% of the time. |  |  |

##### 9.2.2.3.3	Additional Off-axis EIRP density requirements for protection of fixed services

For NTN VSAT, the regional requirements specified in Table 9.2.2.3.3-1 apply.

Table 9.2.2.3.3-1: Off-axis EIRP density limits for protection of fixed services

| Frequency Range (GHz) | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 27.8285 – 28.4445 |  |  |
| 28.8365 – 28.9485 (Note 1) | -5 | 1 |
| 28.9485 – 29.4525 |  |  |
| NOTE1: When applicable, if this frequency range is allocated to fixed service. |  |  |

### 9.2.3	Configured transmitted power

The NTN VSAT can configure its maximum output power. The configured NTN VSAT maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11].

The configured NTN VSAT maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds

PUEType - TEIRP≤ PUMAX,f,c ≤ EIRPmax + TEIRP

with PUEType is the NTN VSAT minimum peak EIRP as specified in sub-clause 9.2.1, EIRPmax is the applicable maximum EIRP as specified in sub-clause 9.2.1 and TEIRP is equal to 3.4 dB. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

while the corresponding measured total radiated power PTMAX,f,c is bounded by the maximum TRP limit TRPMAX for NTN VSAT defined in sub-clause 9.2.1:

PTMAX,f,c ≤ TRPMAX + TTRP

where, TTRP is specified as 3 dB. The PTMAX,f,c requirement is verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode.

## 9.3	Output power dynamics

### 9.3.1	Minimum output power

The requirement is not applicable in this version of the specification.

### 9.3.2	Transmit OFF power

#### 9.3.2.1	General

The transmit OFF power is defined as the TRP in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the NTN VSAT is not allowed to transmit on any of its ports.

The transmit OFF power shall be less than -36 dBm/MHz. The requirement is verified with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

### 9.3.3	Transmit ON/OFF time mask

#### 9.3.3.1	General

The transmit ON/OFF time mask defines the transient period(s) allowed

-	between transmit OFF power and transmit ON power symbols (transmit ON/OFF)

-	between continuous ON-power transmissions when power change or RB hopping is applied.

In case of RB hopping, transition period is shared symmetrically.

Unless otherwise stated the minimum requirements in clause 9.5 apply also in transient periods.

The transmit ON/OFF time mask is defined as a directional requirement. The requirement is verified in beam locked mode at beam peak direction. The maximum allowed EIRP OFF power level is -36dBm/MHz at beam peak direction. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

In the following sub-clauses, following definitions apply:

-	A slot transmission is a Type A transmission.

-	A long subslot transmission is a Type B transmission with more than 2 symbols.

-	A short subslot transmission is a Type B transmission with 1 or 2 symbols.

#### 9.3.3.2	General ON/OFF time mask

The general ON/OFF time mask defines the observation period allowed between transmit OFF and ON power. ON/OFF scenarios include contiguous and non-contiguous transmission.

The OFF power measurement period is defined in a duration of at least one slot excluding any transient periods. The ON power is defined as the mean power over one slot excluding any transient period.

Figure 9.3.3.2-1: General ON/OFF time mask for NR UL transmission in FR2-NTN

#### 9.3.3.3	Transmit power time mask for slot and short or long subslot boundaries

The transmit power time mask for slot and a long subslot transmission boundaries defines the transient periods allowed between slot and long subslot PUSCH transmissions. For PUSCH-PUCCH and PUSCH-SRS transitions and multiplexing the time masks in sub-clause 9.3.3.7 apply.

The transmit power time mask for slot or long subslot and short subslot transmission boundaries defines the transient periods allowed between slot or long subslot and short subslot transmissions. The time masks in sub-clause 9.3.3.8 apply.

The transmit power time mask for short subslot transmissiona boundaries defines the transient periods allowed between short subslot transmissions. The time masks in sub-clause 9.3.3.9 apply.

#### 9.3.3.4	PRACH time mask

The PRACH ON power is specified as the mean power over the PRACH measurement period excluding any transient periods as shown in Figure 9.3.3.4-1. The measurement period for different PRACH preamble format is specified in Table 9.3.3.4-1.

Table 9.3.3.4-1: PRACH ON power measurement period

| Format | SCS | Measurement period |
| --- | --- | --- |
| A1 | 60 kHz | 0.035677 ms |
|  | 120 kHz | 0.017839 ms |
| A2 | 60 kHz | 0.071354 ms |
|  | 120 kHz | 0.035677 ms |
| A3 | 60 kHz | 0.107031 ms |
|  | 120 kHz | 0.053516 ms |
| B1 | 60 kHz | 0.035091 ms |
|  | 120 kHz | 0.0175455 ms |
| B4 | 60 kHz | 0.207617 ms |
|  | 120 kHz | 0.103809 ms |
| A1/B1 | 60 kHz | 0.035677 ms for front X1 occasion 0.035091 ms for last occasion X1 = [2,5] |
|  | 120 kHz | 0.017839 ms for front X1occasion 0.017546 ms for last occasion X1 = [2,5] |
| A2/B2 | 60 kHz | 0.071354 ms for front X2 occasion 0.069596 ms for last occasion X2 = [1,2] |
|  | 120 kHz | 0.035677 ms for front X2 occasion 0.034798 ms for last occasion X2 = [1,2] |
| A3/B3 | 60 kHz | 0.107031 ms for first occasion 0.104101 ms for second occasion |
|  | 120 kHz | 0.053515 ms for first occasion 0.052050 ms for second occasion |
| C0 | 60 kHz | 0.026758 ms |
|  | 120 kHz | 0.013379 ms |
| C2 | 60 kHz | 0.083333 ms |
|  | 120 kHz | 0.0416667 ms |
| NOTE: For PRACH on PRACH occasion start from begin of 0ms or 0.5 ms boundary, the measurement period will plus 0.032552 μs |  |  |

Figure 9.3.3.4-1: PRACH ON/OFF time mask

#### 9.3.3.5	Void

#### 9.3.3.6	SRS time mask

In the case a single SRS transmission, the ON power is defined as the mean power over the symbol duration excluding any transient period; Figure 9.3.3.6-1.

Figure 9.3.3.6-1: Single SRS time mask for NR UL transmission

In the case multiple consecutive SRS transmission, the ON power is defined as the mean power for each symbol duration excluding any transient period. See Figure 9.3.3.6-2

Figure 9.3.3.6-2: Consecutive SRS time mask for the case when no power change is required

When power change between consecutive SRS transmissions is required, then Figure 9.3.3.6-3 and Figure 9.3.3.6-4 apply.

Figure 9.3.3.6-3: Consecutive SRS time mask for the case when power change is required and when 60kHz SCS is used in FR2-NTN

Figure 9.3.3.6-4: Consecutive SRS time mask for the case when power change is required and when 120kHz SCS is used in FR2-NTN

#### 9.3.3.7	PUSCH-PUCCH and PUSCH-SRS time masks

The PUCCH/PUSCH/SRS time mask defines the observation period between sounding reference symbol (SRS) and an adjacent PUSCH/PUCCH symbol and subsequent UL transmissions. The time masks apply for all types of frame structures and their allowed PUCCH/PUSCH/SRS transmissions unless otherwise stated.

Figure 9.3.3.7-1: PUCCH/PUSCH/SRS time mask when there is a transmission before or after or both before and after SRS

When there is no transmission preceding SRS transmission or succeeding SRS transmission, then the same time mask applies as shown in Figure 9.3.3.7-1.

#### 9.3.3.8	Transmit power time mask for consecutive slot or long subslot transmission and short subslot transmission boundaries

The transmit power time mask for consecutive slot or long subslot transmission and short subslot transmission boundaries defines the transient periods allowed between such transmissions.

Figure 9.3.3.8-1: Consecutive slot or long subslot transmission and short subslot transmission time mask

#### 9.3.3.9	Transmit power time mask for consecutive short subslot transmissions boundaries

The transmit power time mask for consecutive short subslot transmission boundaries defines the transient periods allowed between short subslot transmissions.

The transient period shall be equally shared as shown on Figure 9.3.3.9-1.

Figure 9.3.3.9-1: Consecutive short subslot transmissions time mask where DMRS is not the first symbol in the adjacent short subslot transmission

Figure 9.3.3.9-2: Consecutive short subslot (1 symbol gap) time mask for the case when transient period is required on both sides of the symbol and when 120 kHz SCS is used in FR2

### 9.3.4	Power control

#### 9.3.4.1	General

The requirements on power control accuracy apply under normal conditions and are defined as a directional requirement. The requirements are verified in beam locked mode on beam peak direction.

The applicant shall declare the method and accuracy of uplink power control.

## 9.4	Transmitter signal quality

### 9.4.1	Frequency Error

The NTN VSAT basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN VSAT pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] sub-clause 16.14.2. The mean value of basic measurements of NTN VSAT modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the NTN VSAT by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the NTN VSAT is explicitly provided to the NTN VSAT from the test equipment.]

Requirement will be verified for at least two cases of which one has zero Doppler conditions.

The frequency error is defined as a directional requirement. The requirement is verified in beam locked mode with the test metric of Frequency (Link=TX beam peak direction, Meas=Link angle).

### 9.4.2	Transmit modulation quality

#### 9.4.2.1	General

Transmit modulation quality defines the modulation quality for expected in-channel RF transmissions from the NTN VSAT. The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

All the parameters defined in sub-clause 9.4.2 are defined using the measurement methodology specified in Annex F.

All the requirements in sub-clause 9.4.2 are defined as directional requirement. The requirements are verified in beam locked mode on beam peak direction, with parameter maxRank (as defined in TS 38.331 [11]) set to 1. The requirements are applicable to UL transmission from each configurable antenna port (as defined in TS 38.331 [11]) of UE, enabled one at a time.

#### 9.4.2.2	Error vector magnitude

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM, the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

For DFT-s-OFDM waveforms, the EVM result is defined after the front-end FFT and IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a percentage value (%). For CP-OFDM waveforms, the EVM result is defined after the front-end FFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a percentage value (%).

The basic EVM measurement interval in the time domain is one preamble sequence for the PRACH and one slot for PUCCH and PUSCH in the time domain. The EVM measurement interval is reduced by any symbols that contains an allowable power transient in the measurement interval as as defined in sub-clause 9.3.3.

The RMS average of the basic EVM measurements over 10 subframes for the average EVM case, and over 60 subframes for the reference signal EVM case, for the different modulation schemes shall not exceed the values specified in Table 9.4.2.2-1 for the parameters defined in Table 9.4.2.2-2. For EVM evaluation purposes, all 13 PRACH preamble formats and all 5 PUCCH formats are considered to have the same EVM requirement as QPSK modulated.

The requirement is verified with the test metric of EVM (Link=TX beam peak direction, Meas=Link angle).

Table 9.4.2.2-1: Minimum requirements for error vector magnitude

| Parameter | Unit | Average EVM level | Reference signal EVM level |
| --- | --- | --- | --- |
| Pi/2 BPSK | % | 30.0 | 30.0 |
| QPSK | % | 17.5 | 17.5 |
| 16QAM | % | 12.5 | 12.5 |
| 64QAM | % | 8.0 | 8.0 |

Table 9.4.2.2-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| NTN VSAT EIRP | dBm | [Min peak EIRP] |
| NTN VSAT EIRP for UL 16QAM | dBm | [Min peak EIRP] |
| NTN VSAT EIRP for UL 64QAM | dBm | [Min peak EIRP] |
| Operating conditions |  | Normal conditions |

## 9.5	Output RF spectrum emissions

### 9.5.1	Occupied bandwidth

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. The occupied bandwidth for all transmission bandwidth configurations (Resources Blocks) shall be less than the channel bandwidth specified in Table 9.5.1-1.

The occupied bandwidth is defined as a directional requirement. The requirement is verified in beam locked mode with the test metric of OBW (Link=TX beam peak direction, Meas=Link angle).

Table 9.5.1-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Channel bandwidth (MHz) | 50 | 100 | 200 | 400 |

### 9.5.2	Out of Band Emissions

#### 9.5.2.1	General

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio. Additional requirements to protect specific bands are also considered.

The requirements in sub-clause 9.5.2.2 only apply when both UL and DL of an NTN VSAT are configured for single CC operation, and they are of the same bandwidth.

All out of band emissions for FR2-NTN are specified as TRP.

The spectrum emission mask of the NTN VSAT applies to frequencies starting from the  edge of the assigned NR channel bandwidth.

#### 9.5.2.2	Spectrum emission mask

##### 9.5.2.2.1	General NR spectrum emission mask

The power of any NTN VSAT emission shall not exceed the Basic limits specified in Table 9.5.2.2-1 for the specified channel bandwidth. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid). Where:

-	f is the separation between the Transmission BW channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.Table 9.5.2.2.1-1: General NR spectrum emission mask for FR2-NTN

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2× BW | 0.5 MHz f_offset < 2× BW + 0.5 MHz | max11, TRPrated – 10log10(BW) –40×log10 f_offset-0.5BW×2+1dBm | 1 MHz |
| NOTE 1: TRPrated is the declared rated output power lower than or equal to TRPmax specified in sub-clause 9.2.1; NOTE 2: Transmission BW is in the unit of MHz; NOTE 3: The 11dBm/1MHz value corresponds to the spurious emission limit specified in spurious emission sub-clause 9.5.3, and is converted from the SE limit requirement defined on 4 kHz to a value defined over 1 MHz; NOTE 4: PSD attenuation as in ITU-R SM.1541-6 [6], Annex 5 OoB domain emission limits for earth stations. |  |  |  |

##### 9.5.2.2.2	Additional spectrum emission mask

For bands n511 and n510 the mean power of emissions shall be attenuated below the mean output power of the transmitter (measured in dBm) in accordance with [FCC 25.202].

The power of any NTN VSAT emission shall not exceed the Basic limits specified in Table 9.5.2.2.2-1 for the specified channel bandwidth. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid). Where: f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

Table 9.5.2.2.2-1: Additional spectrum emission mask

| Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0.002MHz+0.5xBW f_offset < 1xBW-0.002MHz | TRPrated(dBm) - 25 dB | 4 kHz |
| 0.002MHz+1xBW f_offset < 2.5xBW-0.002MHz | TRPrated(dBm) - 35 dB | 4 kHz |
| 0.002MHz+2.5xBW f_offset < 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 4 kHz |
| NOTE 1: TRPrated is the declared rated output power lower than or equal to TRPmax specified in sub-clause 9.2.1; NOTE 2: Transmission BW is in the unit of MHz; NOTE 3: Measurement bandwidths as in ITU-R SM.329 [16], s4.1. NOTE 4: Upper frequency as in ITU-R SM.329 [16], s2.5 table 1. |  |  |

#### 9.5.2.3	Adjacent channel leakage ratio

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. ACLR requirement is specified for a scenario in which adjacent carrier is another NR channel.

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 9.5.2.3-1 for FR2-NTN.

If the measured adjacent channel power is greater than [–35] dBm then the NRACLR shall be higher than the value specified in Table 9.5.2.3-1. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 9.5.2.3-1: General requirements for NRACLR for FR2-NTN

|  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n512, n511, n510 | 14 dB | 14 dB | 14 dB | 14 dB |
| NR channel measurement bandwidth (MHz) | 47.58 | 95.16 | 190.20 | 380.28 |
| Adjacent channel centre frequency offset (MHz) | +50 / -50 | +100 / -100 | +200 / -200 | +400 / -400 |

### 9.5.3	Spurious Emissions

#### 9.5.3.1	General

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 [16] and NR operating band requirement to address UE co-existence. Spurious emissions are measured as TRP.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 9.5.3.1-1 starting from the edge of the assigned NR channel bandwidth. The spurious emission limits in Table 9.5.3.1-2 apply for all transmitter band configurations (NRB) and channel bandwidths. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 9.5.3.1-1: Boundary between NR out of band and spurious emission domain

| Channel bandwidth | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| OOB boundary FOOB (MHz) | 100 | 200 | 400 | 800 |

Table 9.5.3.1-2: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 4 kHz |

#### 9.5.3.2	On-axis spurious requirement

##### 9.5.3.2.1	Applicability

The regional On-axis spurious requirement is applicable to NTN VSAT operating in band n512. The On-axis spurious emissions are measured as EIRP.

These limits are applicable to the complete NTN VSAT equipment, including cabling between the units.

##### 9.5.3.2.2	“Emissions disabled” and “Carrier-off” states

The requirements specified in table 9.5.3.2.2-1 apply to NTN VSAT in “Emissions disabled” and “Carrier-off” states. They apply outside the transmission bandwidth.

Table 9.5.3.2.2-1: On-axis spurious limits in “Emissions disabled” and “Carrier-off” states

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 27.5 – 30.0 | 19 | 1 |

##### 9.5.3.2.3	“Carrier-on” state

The requirements specified in Tables 9.5.3.2.3-1 and 9.5.3.2.3-2 apply to NTN VSAT in “Carrier-on”.

The requirement specified in Table 9.5.3.2.3-1 apply outside a bandwidth of 5 times the occupied bandwidth centred on the carrier centre frequency.

The requirement specified in Table 9.5.3.2.3-2 apply inside a bandwidth of 5 times the occupied bandwidth centred on the carrier centre frequency, and outside the transmission bandwidth.

NOTE:	The on-axis spurious radiations, outside the frequency range 27.5 GHz to 30.0 GHz, are indirectly limited by sub-clause 9.5.3.3.

Table 9.5.5.2.2.3-1: On-axis spurious limits in “Carrier-on” state - outside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 27.5 – 30.0 | 4, 5 | 44 - K (NOTE) | 1 |
|  | 1, 2, 3 | 4 - K (NOTE) | 1 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17] for Mobile VSAT or sub-clause 4.2.4.2 in [18] for Fixed VSAT. The manufacturer shall declare the value of N. |  |  |  |

Table 9.5.5.2.2.3-2: On-axis spurious limits in “Carrier-on” state - inside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 27.5 – 30.0 | 4, 5 | 58 - K (NOTE) | 1 |
|  | 1, 2, 3 | 48 - K (NOTE) | 1 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17] for Mobile VSAT or sub-clause 4.2.4.2 in [18] for Fixed VSAT. The manufacturer shall declare the value of N. |  |  |  |

#### 9.5.3.3	Off-axis spurious requirement

##### 9.5.3.3.1	Applicability

The regional Off-axis spurious requirement is applicable to NTN VSAT operating in band n512. The Off-axis spurious emissions are measured as EIRP.

These limits are applicable to the complete NTN VSAT equipment, including cabling between the units.

##### 9.5.3.3.2	General

The requirements specified in table 9.5.3.3.2-1 apply to NTN VSAT at 10 meters distance from the NTN VSAT.

Table 9.5.3.3.2-1: Radiated field strength limits at 10m from the NTN VSAT

| Frequency range (MHz) | EIRP Limit (dBμV/m) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 30 – 230 | 30 | 120 |
| 230 – 1000 | 37 | 120 |

##### 9.5.3.3.3	“Emissions disabled” state

The requirements specified in table 9.5.3.3.3-1 apply to NTN VSAT in “Emissions disabled” state for all off-axis angles greater than 7° or greater than the minimum elevation angle supported, whichever is lower.

Table 9.5.3.3.3-1: Off-axis spurious limits in “Emissions disabled” state

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.0 – 2.0 | -48 | 100 |
| 2.0 – 10.7 | -42 | 100 |
| 10.7 – 21.2 | -36 | 100 |
| 21.2 – 60.0 | -30 | 100 |

##### 9.5.3.3.4	“Carrier-on” and “Carrier-off” states

The requirements specified in table 9.5.3.3.4-1 apply to NTN VSAT in “Carrier-on” and “Carrier-off” states for all off-axis angles greater than 7° or greater than the minimum elevation angle supported, whichever is lower.

Table 9.5.3.3.4-1: Off-axis spurious limits in “Carrier-on” and “Carrier-off” states

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 1.00 – 3.40 | -41 | 0.1 |
| 3.40 – 10.70 | -35 | 0.1 |
| 10.70 – 21.20 | -29 | 0.1 |
| 21.20 – 27.35 | -23 | 0.1 |
| 27.35 – 27.50 | -5 (Note 1) | 1 |
|  | -23 (Note 2) | 0.1 |
| 27.50 – 29.35 | -5 (Note 1) | 1 |
|  | -23 (Note 2) | 0.1 |
| 29.35 – 29.50 | -5 | 1 |
| 30.00 – 30.15 | -5 | 1 |
| 30.15 – 60.00 | -23 | 0.1 |
| NOTE 1: For mobile VSAT transmitting in the frequency range 29.5 – 30.0 GHz NOTE 2: For mobile VSAT transmitting in the frequency range 27.5 – 29.5 GHz |  |  |

## 9.6	Antenna pointing accuracy and performance

### 9.6.1	Antenna pointing accuracy

#### 9.6.1.1	Minimum requirements for NTN VSAT

##### 9.6.1.1.1	Applicability

Except if otherwise stated, the following regional requirements are applicable to NTN VSAT types 1, 2, 3, 4 or 5 operating in band n512.

##### 9.6.1.1.2	Pointing Accuracy

The manufacturer shall declare the peak pointing accuracy () and the associated statistical basis.

The antenna shall maintain the declared peak pointing accuracy (), such that the off-axis EIRP emission density pattern projected onto the geostationary arc remains within the mask specified in sub-clauses 9.2.2.2 and 9.2.2.3 when shifted by an angle of ±(°), taking into account the following factors [17]:

-	the worst case operational environmental conditions;

-	maximum dynamics for Mobile VSAT (e.g. maximum movement of the platform e.g. airplane, boat, vehicle during the connectivity time); and

-	the range of latitude, longitude and altitude relative to the satellite orbital position.

##### 9.6.1.1.3	On-axis cross polarization isolation

9.6.1.1.3.1	Linearly polarized NTN VSAT

This requirement is applicable to NTN VSAT type 1, 2, 4 and 5.

For linearly polarized NTN VSAT, the manufacturer shall declare the on-axis cross polarization isolation of the NTN VSAT [17, 18].

The polarization angle shall be continuously adjustable within the operational range as declared by the manufacturer.

It shall be possible to fix the transmit antenna polarization angle with an accuracy of at least 1°.

When linear polarization is used for both transmission and reception, the angle between the receive and corresponding transmit polarization planes shall not deviate by more than 1° from the nominal value declared by the manufacturer.

9.6.1.1.3.2	Circularly polarized NTN VSAT

For circularly polarized NTN VSAT, the manufacturer shall declare the voltage axial ratio.

#### 9.6.1.2	Minimum requirement for Fixed VSAT types 1 or 2

##### 9.6.1.2.1	Applicability

The following regional requirements are applicable to Fixed VSAT types 1 or 2 operating in band n512 when connected to Geostationary Satellite Orbit (GSO) SAN.

##### 9.6.1.2.2	Pointing Stability

Pointing stability: Under the condition of 100 km/h maximum wind speed, with gusts of 130 km/h lasting 3 seconds, the installation shall not show any sign of permanent distortion and shall not need repointing after the application of the wind load.

##### 9.6.1.2.3	Pointing Accuracy

9.6.1.2.3.1	General

The manufacturer shall declare the usage area in terms of the range of latitude and longitude relative to the satellite orbital position where the alignments specified below are possible.

9.6.1.2.3.2	Main beam pointing accuracy

The antenna sub-system alignment facilities shall enable the main beam axis to be adjusted and fixed with a pointing accuracy (δφ) of either:

-	1) 0,1º; or

-	2) a greater value declared by the applicant, subject to the following restrictions:

-	the pointing accuracy (δφ) shall not exceed 30 % of the antenna transmit main beam half power beamwidth;

-	the off-axis e.i.r.p. emission density pattern remains within the mask specified in sub-clause 9.2.2.3 when shifted by an angle of ±(δφ – 0,1º).

9.6.1.2.3.3	Alignment with the geostationary satellite orbit

Alignment with the geostationary satellite orbit. For antennas with asymmetric main beam, the antenna shall be capable of having the plane defined by the antenna main beam axis and its major axis aligned with the tangent to the geostationary orbit in accordance with the method declared by the manufacturer.

##### 9.6.1.2.4	Polarization angle alignment capability for linear polarization

Following conditions will apply:

-	The polarization angle shall be continuously adjustable within the operational range as declared by the manufacturer.

-	It shall be possible to fix the transmit antenna polarization angle with an accuracy of at least 1°.

-	When linear polarization is used for both transmission and reception, the angle between the receive and corresponding transmit polarization planes shall not deviate by more than 1° from the nominal value declared by the manufacturer.

### 9.6.2	Antenna performance

The following requirements are applicable to NTN VSAT type 1, type 2, type 4 or type 5 operating in band n511 or in band n510 and communicating with Geostationary Satellite Orbit (GSO) SAN.

The co-polarization gain in the plane tangent to the GSO arc shall not exceed the levels specified in Table 9.6.2-1. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2-1: Co-polarization gain limit in the plane tangent to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 2° ≤ θ ≤ 7° | 29 – 25log10(θ) |
| 7° ≤ θ ≤ 9.2° | 8 |
| 9.2° ≤ θ ≤ 19.1° | 32 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 0 |

The co-polarization gain in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2-2. This envelope may be exceeded by up to 3 dB in 10% of the range of θ angles from ±7–180°, and by up to 6 dB in the region of main reflector spillover energy.

Table 9.6.2-2: Co-polarization gain limit in the plane perpendicular to the GSO arc

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 3.5° ≤ θ ≤ 7° | 32 – 25log10(θ) |
| 7° ≤ θ ≤ 9.2° | 10.9 |
| 9.2° ≤ θ ≤ 19.1° | 35 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 3 |

The off-axis cross-polarization gain in the plane tangent and in the plane perpendicular to the GSO arc shall not exceed the levels specified in Table 9.6.2-3.

Table 9.6.2-3: Cross-polarization gain limit

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 2° ≤ θ ≤ 7° | 19 – 25log10(θ) |

## 9.7	Additional regional requirements indicated by NS

### 9.7.1	General

Additional regional requirements can be signalled by the network. Each group of additional regional requirements is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR NTN frequency band number of the applicable FR2-NTN operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [8].

Table 9.7.1-1 specifies the additional regional requirements with their associated network signalling values, the applicable satellite orbit scenario(s) and applicable FR2-NTN operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 9.7.1-2.

Table 9.7.1-1: Additional regional requirements indicated by Network Signalling label

| Network Signalling label | Requirements (clause) | Applicable Satellite orbit scenario | NR satellite Band | Channel bandwidth (MHz) |
| --- | --- | --- | --- | --- |
| NS_200N |  | GSO and LEO | Table 5.2.3-1 | 50, 100, 200, 400 |
| NS_201N | Clause 9.2.2.3 Clause 9.5.3.2 Clause 9.5.3.3 Clause 9.6.1.1 Clause 9.6.1.2 Clause 10.8 | GSO | n512 | 50, 100, 200, 400 |
| NS_202N | Clause 9.5.3.2 Clause 9.5.3.3 Clause 9.6.1.1 | LEO | n512 | 50, 100, 200, 400 |

Table 9.7.1-2: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n512 | NS_200N | NS_201N | NS_202N |  |  |  |  | Reserved |
| n511 | NS_200N |  |  |  |  |  |  | Reserved |
| n510 | NS_200N |  |  |  |  |  |  | Reserved |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. NOTE 2: For band n511 and n510, only NS_200N can be used to map. |  |  |  |  |  |  |  |  |

# 10	Radiated receiver characteristics

## 10.1	General

Unless otherwise stated, the receiver characteristics are specified over the air (OTA) at the RIB for Ka bands fixed and mobile VSAT. The power level for all DL wanted signals and interference is defined assuming a 0 dBi reference antenna located at the center of the quiet zone.

## 10.2	Polarization characteristics

The minimum requirements on the receiver characteristics apply under either LHCP (Left Hand Circular Polarization) or RHCP (Right Hand Circular Polarization) or Linear Polarization.

## 10.3	OTA reference sensitivity level

### 10.3.1	General

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level at the centre of the quiet zone in the RX beam peak direction. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received over the air at the RIB, at which the throughput shall meet or exceed the requirements for a specified reference measurement channel.

### 10.3.2	Minimum requirement

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2.1.2 and A.3.2.1.3 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with peak reference sensitivity specified in Table 10.3.2-1. EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

The EIS of Rx beam peak direction should be verified within the declared minimum elevation angle supported for receiving. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type. Where the supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 10.3.2-1 below is taken as an example.

Figure 10.3.2-1 Example measurement grid for EIS with the declared supported minimum elevation angle

Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT

| NTN VSAT channel bandwidth (MHz) | UL/DL RB allocation | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- |
| 50, 100, 200, 400 | Full RB allocation NRB as specified in sub-clause 5.3.2 | EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor) (NOTE 1) |
| NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |

Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT

| Operating band | NTN VSAT class | NTN VSAT type | The range of EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- |
| n512, n511 | Fixed VSAT | 1, 2 | ≤ -122 |
|  |  | 3 | ≤ -115.6 |
| n512, n511, n510 | Mobile VSAT | 4, 5 | ≤ -122 |

## 10.4	Maximum input level

### 10.4.1	General

The maximum input level is defined as the maximum mean power, for which the throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

The maximum input level is defined as a directional requirement. The requirement is verified in beam locked mode in the direction where peak gain is achieved.

### 10.4.2	Minimum requirement for Mobile VSAT

For mobile VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A (with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 10.4.2-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.4.2-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in transmission bandwidth configuration | dBm | -109.6 for type 4 and type 5 (NOTE 2, 3, 4) |  |  |  |
| NOTE 1: The transmitter shall be set to [4 dB] below the PUMAX,f,c as defined in sub-clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 2: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: QPSK, R=1/3 variant with one sided dynamic OCNG Pattern as described in Annex A.5.1.1. NOTE 3: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 16QAM, R=1/2 variant with one sided dynamic OCNG Pattern as described in Annex A.5.1.1. NOTE 4: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 64QAM, R=1/2 variant with one sided dynamic OCNG Pattern as described in Annex A.5.1.1. |  |  |  |  |  |

### 10.4.3	Minimum requirement for Fixed VSAT

For fixed VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A (with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 10.4.3.-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.4.3-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in transmission bandwidth configuration | dBm | -101 for type 1, type 2 and type 3 (NOTE 2, 3, 4) |  |  |  |
| NOTE 1: The transmitter shall be set to [4 dB] below the PUMAX,f,c as defined in sub-clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 2: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: QPSK, R=1/3 variant with one sided dynamic OCNG Pattern as described in Annex A.5.1.1. NOTE 3: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 16QAM, R=1/2 variant with one sided dynamic OCNG Pattern as described in Annex A.5.1.1. NOTE 4: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 64QAM, R=1/2 variant with one sided dynamic OCNG Pattern as described in Annex A.5.1.1. |  |  |  |  |  |

## 10.5	Adjacent channel selectivity

### 10.5.1	Minimum requirement for Mobile VSAT

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The requirement applies at the RIB when the AoA of the incident wave of the wanted signal and the interfering signal are both from the same direction.

For mobile VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2.1.2 and A.3.2.1.3 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 10.5.1-1 and Table 10.5.1-2. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.5.1-1: Adjacent channel selectivity

| Operating band | Units | Adjacent channel selectivity / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n512, n511, n510 | dB | 25 | 25 | 25 | 25 |

Table 10.5.1-2: Adjacent channel selectivity test parameters

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | EISREFSENS_50M + 6 dB + 10log10(NRB x SCS x 12 / factor) NOTE 5 |  |  |  |
| PInterferer for band n512, n511, n510 | dBm | EISREFSENS_50M + 28.7 + 10log10(NRB x SCS x 12 / factor) NOTE 5 |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 [15] with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 [15] and set-up according to Annex C. NOTE 2: EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: The transmitter shall be set to same as the PUMAX,f,c as defined in clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 5: SCS is in kHz, the “factor” represents the normalized factor to scale wanted signal and interference level for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |  |  |  |

### 10.5.2	Minimum requirement for Fixed VSAT

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The requirement applies at the RIB when the AoA of the incident wave of the wanted signal and the interfering signal are both from the same direction.

For fixed VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2.1.2 and A.3.2.1.3 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 10.5.2-1 and Table 10.5.2-2. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.5.2-1: Adjacent channel selectivity

| Operating band | Units | Adjacent channel selectivity / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n512, n511, n510 | dB | 25 | 25 | 25 | 25 |

Table 10.5.2-2: Adjacent channel selectivity test parameters

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | EISREFSENS_50M + 6 dB + 10log10(NRB x SCS x 12 / factor) NOTE 5 |  |  |  |
| PInterferer for band n512, n511, n510 | dBm | EISREFSENS_50M + 28.7 + 10log10(NRB x SCS x 12 / factor) NOTE 5 |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 [15] with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 [15] and set-up according to Annex C. NOTE 2: EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: The transmitter shall be set to same as the PUMAX,f,c as defined in clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 5: SCS is in kHz, the “factor” represents the normalized factor to scale wanted signal and interference level for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |  |  |  |

## 10.6	Blocking characteristics

### 10.6.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

The requirement applies at the RIB when the AoA of the incident wave of the wanted signal and the interfering signal are both from the direction where peak gain is achieved.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

### 10.6.2	Minimum requirement for Mobile VSAT

In-band blocking is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an interferer at a given frequency offset from the centre frequency of the assigned channel.

For mobile VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2.1.2 and A.3.2.1.3 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 10.6.2-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.6.2-1: In band blocking requirements

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | EISREFSENS_50M + 6 dB + 10log10(NRB x SCS x 12 / factor) |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n512, n511 | dBm | EISREFSENS_50M + 28.7 + 10log10(NRB x SCS x 12 / factor) |  |  |  |
| FInterferer (offset) | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 [15] with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 [15] and set-up according to Annex C. NOTE 2: EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG pattern as described in Annex A.5.1.1 and set-up according to Annex C. NOTE 4: Void NOTE 5: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. NOTE 7: The transmitter shall be set to [4 dB] below the PUMAX,f,c as defined in clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 8: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |  |  |  |

### 10.6.3	Minimum requirement for Fixed VSAT

In-band blocking is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an interferer at a given frequency offset from the centre frequency of the assigned channel.

For fixed VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 10.6.3-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.6.3-1: In band blocking requirements

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | EISREFSENS_50M + 6 dB + 10log10(NRB x SCS x 12 / factor) |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n512, n511, n510 | dBm | EISREFSENS_50M + 28.7 + 10log10(NRB x SCS x 12 / factor) |  |  |  |
| FInterferer (offset) | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 [15] with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 [15] and set-up according to Annex C. NOTE 2: EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG pattern as described in Annex A.5.1.1 and set-up according to Annex C. NOTE 4: Void NOTE 5: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. NOTE 7: The transmitter shall be set to [4 dB] below the PUMAX,f,c as defined in clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 8: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |  |  |  |

## 10.7	Spurious emissions

The requirement is not applicable in this version of the specification.

## 10.8	Receiver antenna off-axis performance

The following regional requirements are applicable to NTN VSAT operating in band n512 towards geostationary satellite orbit.

The receiver antenna off-axis gain of each co-polarized components in any direction φ degrees from the antenna main beam shall not exceed the levels specified in Table 10.8-1.

Table 10.8-1: Off-axis Co-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φmin ≤ φ ≤ 48° | 32 – 25log10(φ) |
| 48° ≤ φ ≤ 85° | -10 |
| 85° ≤ φ ≤ 180° | 0 |
| NOTE: φmin = 1o or 100λ/D (degrees) whichever is the greater, for D/λ≥ 50. φmin = 2o or 114(D/ λ)-1.09 (degrees) whichever is the greater, for D/λ 50. where D is the nominal diameter of the antenna |  |

The receiver antenna off-axis gain of each cross-polarized components in any direction φ degrees from the antenna main beam shall not exceed the levels specified in Table 10.8-2.

Table 10.8-2: Off-axis Cross-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φr ≤ φ ≤ 7° | 23 – 20log10(φ) |
| NOTE: φr = 1o or 100λ/D (degrees) whichever is the greater where D is the nominal diameter of the antenna |  |