3GPP TSG-RAN WG4 Meeting #115																			R4-2505754

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Further D2R LLS SNR simulation results

Agenda item:	7.26.3.1

Document for:	Discussion

- Introduction

This contribution provides further D2R SNR simulation results based on certain assumptions. We also discussed some missing simulation assumptions.

- Discussion

In our simulation, the following assumptions are used. The highlighted parameters are the options we chose from the WF [1].

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 900 MHz |
| SCS | 15 kHz as baseline |
| Block structure | Depend on RAN1 |
| Channel model | AWGN |
| Number of Tx/Rx chains for Ambient IoT device | 1 |
| Number of BS antenna elements | 1 |
| Reference data rate | 1 kbps (M) |
| Message size | Option 1： {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size Option 2： 96 bits |
| BLER target | 10% |
| Sampling frequency（SFO） | Option 1: Randomly select a value from the range of [104 ppm~105 ppm] for device 1 Option 2: Use Residual SFO based on companies report |
| [OOK/BPSK chip rate] | 7.5 kcps for 2SB 15 kHz Transmission bandwidth |
| Receiver bandwidth | Option1: the same as transmission bandwidth. Option 2：18 kHz (consider SFO and BS filter) for 2SB 15 kHz transmission bandwidth Option 3: first sidelobes as receiver bandwidth Companies can report |
| Waveform (CW) | unmodulated single tone |
| Modulation | Option1：OOK Option 2：BPSK Companies can report |
| Line code | Manchester encoding |
| FEC | 1/3 Convolutional Code |
| ADC bit width | Ideal |
| D2R receiver | coherent receiver |
| Repetition | None |
| Preamble configuration | 32 bit M-sequence |
| Midamble configuration | 32 bit M-sequence |

We have some the following simulation results using the above assumptions,

| preamble/midamble configuration | Message size | BLER target | SNR (dB) |
| --- | --- | --- | --- |
| Preamble, without midamble | 20bits | 10% | 3.6 |
| Preamble, without midamble | 96bits | 10% | Can’t be decoded |
| Preamble, without midamble | 400bits | 10% | Can’t be decoded |
| Preamble + 1 midamble | 20bits | 10% | -3 |
| Preamble + 1 midamble | 96bits | 10% | -2.8 |
| Preamble + 1 midamble | 400bits | 10% | -2.2 |

One thing should be noted for our simulation results that we used the 32 bit M-sequence. Due to the limited time between the two meetings, we didn’t have time to update to the base sequence of 2^5 -1 m-sequence, but our estimation is that the SNR results are similar.

For the REFSENS test, usually using a relatively small signal, so one of the low SNR threshold of the configurations can be chosen as the physical channel configuration.

Proposal: Use only one message size and one preamble/midamble configuration to define the reader REFSENS. The configuration with low SNR threshold can be considered.

- Conclusion

This contribution provides further D2R SNR simulation results with some preamble/midamble configurations. We have the proposal for message size as following.

Proposal: Use only one message size and one preamble/midamble configuration to define the reader REFSENS. The configuration with low SNR threshold can be considered.

- References

[1] R4-2505097, “WF on A-IoT BS and CW requirements”, Huawei, RAN4#114b