3GPP TSG-RAN WG4 Meeting # 115	R4-2505768

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on event triggered L1 measurement reporting for mobility

Agenda Item:	7.29.3.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 Mobility.

- Event-triggered L1 reporting

- Discussion

Reporting capability

- Issue 2-4: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Yes (Xiaomi, CMCC, CATT, MTK, ZTE, OPPO, CTC, E///) - Option 1a: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO) - Option 1b: Number of supported L1 events is to be added on top of the currently supported L3 events. Details FFS. (Nokia) - Option 2: Open to let RAN1 discuss the number of events supported by UE at first. (MTK) - Option 2a: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo)

There are some discussions about the UE capability of supported number of events. Generally, we are fine to define capability for supported number of events for L1 based reporting for LTM. Since CSI-RS based LTM is also considered in L1 event triggered reporting, the relation between L3 reporting and L1 reporting, and relation between CSI-RS based L1 measurement/reporting and SSB based L1 measurement/reporting will have impact on the capability definition. it’s also FFS how to count frequency layer. Therefore, we prefer to discuss the detail of UE capability for in later stage when design is clearer.

Proposal 1: RAN4 to define UE capability for Event Triggering and Reporting Criteria for LTM.

Measurement delay

- Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Reuse the framework from L3 event triggered reporting. Discuss the details further.

If the measurement periods for different beams are the same, the legacy requirement can be re-used. However, if the RS periodicity is different, we prefer that the maximum measurement period between current beam and candidate beam will be used as measurement delay, with the following reasons:

- 1. Simplicity: maximum measurement period provides a straightforward and simple description of measurement period, making it easier to implement and understand. This simplicity is crucial for ensuring consistency across different systems and vendors.

- 2. Consistency with Existing Standards: Many similar solutions in the current specification use the maximum of periodicities. This approach ensures that the new solution is consistent with existing practices, reducing the risk of confusion and ensuring a smoother integration.

- 3. Reduced Complexity: other option introduces additional complexity by requiring different TL1_measurement periods for different cells. This can lead to more intricate and error-prone implementations, especially in dynamic environments where beam switching is frequent.

- 4. Ease of Definition: By using the maximum of the L1 measurement periods, we avoid the need to define and manage separate requirements for each component. This simplifies the specification and reduces the burden on system designers and implementers.

Proposal 2: For multiple beams with different measurement period, the maximum measurement period will be considered as measurement delay to ensure simplicity and consistency with existing standards.

- Issue 2-1: applicability of L1-RSRP reporting delay Candidate options: - Option 1: (vivo) - RAN4 starts the discussion on L1-RSRP reporting delay with the follow assumptions: - No indicate TCI change during the evaluation period. - No configuration change w.r.t measurement RS during the evaluation period - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period - Option 2 (Apple) - Seems no need to capture assumptions listed under option-1 into core requirements.

There is similar discussion in event triggered reporting in MIMO. The RAN1 agreement is as below, it states that if measured current beam RS is updated based on indicated TCI state, the counting for all new beams will be reset. This behavior should be aligned in mobility scenarios to ensure unified handling of TCI updates.

Agreement Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. - FFS: Further details on the update (if necessary).

Observation 1: In MIMO, the measured current beam RS is updated based on indicated TCI state, in such case, the UE needs to reset the counting for all new beams.

Therefore, we suggest to keep consistency between event triggered L1 reporting for MIMO and mobility. There is no need to capture such assumption of option 1.

Proposal 3: Suggest to keep consistency between event triggered L1 reporting for MIMO and mobility. The assumptions listed in Option 1 (e.g., no TCI change during evaluation) need not be explicitly captured in core requirements.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: RAN4 to define UE capability for Event Triggering and Reporting Criteria for LTM.

Proposal 2: For multiple beams with different measurement period, the maximum measurement period will be considered as measurement delay to ensure simplicity and consistency with existing standards.

Observation 1: In MIMO, the measured current beam RS is updated based on indicated TCI state, in such case, the UE needs to reset the counting for all new beams.

Proposal 3: Suggest to keep consistency between event triggered L1 reporting for MIMO and mobility. The assumptions listed in Option 1 (e.g., no TCI change during evaluation) need not be explicitly captured in core requirements.

- Reference