3GPP TSG-RAN WG4 Meeting # 115	 R4-2505769

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on CSI-RS based L1 measurement for mobility

Agenda Item:	7.29.3.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 Mobility.

- Definition of intra-frequency and inter-frequency - Pre-requisite of CSI-RS based L1 measurement

- Discussion

- Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 Agreement: - Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting. - Further discuss whether step 2 can be skipped for serving cell or not.

SSB-based L1 measurements serve dual critical functions in FR2-1 systems:

- Supporting mobility management

- Enabling fundamental beam management operations

Their importance is highlighted in the following aspects:

Critical Role in Beam Management:

- SSB measurements are essential for initial beam acquisition, beam tracking, and candidate beam detection, especially in multi-antenna (MIMO) operations.

Serving Cell vs. Neighbor Cell Considerations:

- For neighbor cells, skipping SSB measurements (if supported by UE capability) may be feasible to reduce power consumption, provided CSI-RS suffices for LTM.

- However, for the serving cell, SSB measurements remain indispensable for PHY-layer procedures, ensuring stable connectivity and efficient beam operation.

Given their foundational role in beam management and network reliability, SSB-based L1 measurements must be retained for the serving cell in FR2-1.Therefore, we propose that SSB skipping can’t be applied for serving cell but can be supported for neighbour cell. This method can preserve system robustness while allowing flexibility for neighbor cell measurements where applicable.

Proposal 1: For the serving cell, SSB-based L1 measurements shall not be skipped in FR2-1, even if the UE supports capability reporting to skip them for L1-RSRP for neighbour cell (e.g., for LTM purposes).

In last meeting, there is some discussion about how to define intra-frequency and inter-frequency measurement for CSI-RS L1-RSRP.

- Issue 4-4: CSI-RS based intra-frequency/inter-frequency measurement definition Candidate options: - Option 1: Apple, CMCC A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: MediaTek, ZTE, Ericsson, Nokia, Qualcomm A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP. - Option 3: Huawei If L1 CSI-RS RSRP measurement with gap is not supported, either option of intra-frequency CSI-RS based L1 RSRP measurement definition has no big difference. If CSI-RS measurement with gap is supported, to define measurement requirements, the “CSI-RS frequency layer” concept will be touched. Then the intra-frequency definition based on center frequency is more preferred. - Option 4: vivo - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

In last meeting, it’s agreed that don’t consider L1 measurement with gap and RTD>CP.

- Issue 4-6: measurement GAP and RTD assumption Agreement: - From RAN4 perspective, - Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. - Not to define RAN4 requirements for RTD > CP.

Generally, we are fine with both options. We understand the intention of option 2. Since we don’t consider L1 measurement with gap and RTD>CP, there is no big difference between option 1 and option 2. We prefer the unified framework as legacy. The detail requirement will anyway be defined in each scenario, the result is similar.

Proposal 2: Prefer to define intra – frequency by unified framework as legacy, support option 1.

- Issue 4-9: requirement applicability Candidate options: - Option 1: Qualcomm RAN4 requirements apply only when all CSI-RS resources from candidate cells on the same layer are configured with the same SCS and CP type. - Option 2: MediaTek For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR1, legacy measurement period, measurement restriction and scheduling restriction requirements are all applicable. For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR2, define measurement restriction when two CSI-RS resources from difference cells overlap in time domain. For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR2, legacy measurement period and scheduling restriction requirements are both applicable. - Option 2: vivo RRM requirements for CSI-RS based LTM are only applicable if the CSI-RS of candidate cell is configured with QCL-D to one of the SSBs for L1 measurements.

For neighbor cell L1-RSRP measurement, if SSB based measurement is skipped, UE will use CSI-RS for RX beam sweeping. Then the measurement period will be scaled by 8.

For measurement restriction, the legacy CSI-RS based L1-RSRP measurement can be re-used as baseline. Besides, overlapped CSI-RS from two cells needs to be considered.

- For FR2, when the CSI-RS for L1-RSRP measurement on one CC is in the same OFDM symbol as CSI-RS from neighbor cell L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both CSI-RSs. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

For scheduling restriction, for serving cell, legacy CSI-RS based scheduling restriction can be re-used. For neighbor cell, CSI-RS based scheduling restriction needs to be defined.

Proposal 3: For neighbor cell CSI-RS based L1-RSRP measurement in FR2, if SSB based measurement is skipped, the measurement period for CSI-RS based L1-RSRP will be scaled by 8.

Proposal 4: For CSI-RS based L1-RSRP measurement for LTM in FR2, define measurement restriction for overlapped CSI-RS from two cells for both serving cell and neighbor cell.

Proposal 5: For CSI-RS based L1-RSRP measurement for LTM, for serving cell, legacy scheduling restriction can be re-used. For neighbor cell, CSI-RS based scheduling restriction needs to be defined.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: For the serving cell, SSB-based L1 measurements shall not be skipped in FR2-1, even if the UE supports capability reporting to skip them for L1-RSRP for neighbour cell (e.g., for LTM purposes).

Proposal 2: Prefer to define intra – frequency by unified framework as legacy, support option 1.

Proposal 3: For neighbor cell CSI-RS based L1-RSRP measurement in FR2, if SSB based measurement is skipped, the measurement period for CSI-RS based L1-RSRP will be scaled by 8.

Proposal 4: For CSI-RS based L1-RSRP measurement for LTM in FR2, define measurement restriction for overlapped CSI-RS from two cells for both serving cell and neighbor cell.

Proposal 5: For CSI-RS based L1-RSRP measurement for LTM, for serving cell, legacy scheduling restriction can be re-used. For neighbor cell, CSI-RS based scheduling restriction needs to be defined.

- Reference