3GPP TSG-RAN WG4 Meeting # 115	R4-2505770

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on UE-initiated/event-driven beam management on Rel-19 MIMO phase 5

Agenda Item:	7.24.4.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the RRM requirement of UE-initiated/event-driven beam management for Rel-19 MIMO phase 5.

- Discussion

2.1 Reporting delay

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)? Agreement: - For T_RS (TSSB or TCSI-RS) in L1_measurement periods - T_RS (TSSB or TCSI-RS): - the value of current beam and new beams are the same. - For M in L1_measurement periods: - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam - If timeRestrictionForChannelMeasurement is not configured, - Further discuss on: - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used - Option 2: No requirements even the NW can configure it. - Option 3: FFS - For P (sharing factor) in L1_measurement periods: - For SSB based or CSI-RS based, - The definition is the same as legacy - The value can be the same or different for the current beam and new beam

For M in L1 measurement periods, if timeRestrictionForChannelMeasurement is not configured, we suggest to re-use the same values of legacy.

Proposal 1：For M in L1 measurement periods, reuse the same values of legacy, measurement period is based on 3 sample.

2.2 Measurement time

Issue 1-2: Components of Event triggered measurement reporting delay? Agreement: - The event triggered measurement reporting delay includes the Tfirst UL channel - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH Further discuss the two options and select one option in the next meeting: - The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK) - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

We support Option 1 for the following reasons:

- 1. Simplicity: Option 1 provides a straightforward and simple description of the TL1_measurement period, making it easier to implement and understand. This simplicity is crucial for ensuring consistency across different systems and vendors.

- 2. Consistency with Existing Standards: Many similar solutions in the current specification use the maximum of periodicities. This approach ensures that the new solution is consistent with existing practices, reducing the risk of confusion and ensuring a smoother integration.

- 3. Reduced Complexity: Option 2 introduces additional complexity by requiring different TL1_measurement periods for different beam measurements. This can lead to more intricate and error-prone implementations, especially in dynamic environments where beam switching is frequent.

- 4. Ease of Definition: By using the maximum of the L1 measurement periods, we avoid the need to define and manage separate requirements for each component. This simplifies the specification and reduces the burden on system designers and implementers.

Proposal 2: If TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

2.3 Whether to consider M>1

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19? <Way forward > - Option 1: (Qualcomm, Xiaomi, Apple, LGE) - Do not define requirements for eventInstanceCount-r19 > 1 - Option 1a: (Apple): To extend the requirements for case when eventInstanceCount-r19 > 1 is configured, add note longer delay is expected if eventInstanceCount-r19 > 1 is configured. - Option 2: (Samsung, CMCC, Huawei, ZTE, MediaTek) - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19 - Option 3: (Nokia, Ericsson) - Wait for further RAN1 conclusion - For how to specify the requirements in option 2 above: - Option 2a: - Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where, - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS. - C: event instances threshold is configured by NW. - Ttime_window: time window is configured by NW. - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH. - Other options are not precluded

In the context of event-triggered L1-RSRP reporting, the 3GPP RAN1 has introduced a new reporting quantity: the Condition Met Indicator (CMI). This 1-bit indicator is designed to provide an efficient and reliable mechanism for the UE to inform the gNodeB about whether a specific beam has met the event-triggering conditions within a defined time window. The interpretation of the CMI is as follows:

Agreement On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7.

Since RAN1 has introduced CMI for event triggered reporting for M>1, we support to specify requirements to cover both cases of no time window based and based with configurable eventInstanceCount-r19

Proposal 3: RAN4 to Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19.

2.4 Reduction of beam application delay

Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception. - Other alternatives are not precluded.

RAN1 agrees to further study the reduction of beam application delay, the majority options will skip extra SSB reception for synchronization. This part may have impact on the MAC CE based TCI activation delay if target TCI state is not in the active TCI state list. RAN4 may need to consider the impact according to RAN1 progress.

Proposal 4: Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

2.5 Indicated TCI states is changed

Issue 1-7: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time? <Way forward > - Option 1: (Samsung) - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. - Option 2: - For such case, No RAN4 requirements. - Option 3: (Apple) - Add the applicability rule of event triggered L1 measurement: - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE. - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

The RAN1 agreement is as below, it states that if measured current beam RS is updated based on indicated TCI state, the counting for all new beams will be reset.

Agreement Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. - FFS: Further details on the update (if necessary).

Observation: The measured current beam RS is updated based on indicated TCI state, in such case, the UE needs to reset the counting for all new beams.

Proposal 5: If indicated TCI states is changed during the L1 measurement time, the L1 evaluation time should restart.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1：For M in L1 measurement periods, reuse the same values of legacy, measurement period is based on 3 sample.

Proposal 2: If TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

Proposal 3: RAN4 to Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19.

Proposal 4: Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

Proposal 5: If indicated TCI states is changed during the L1 measurement time, the L1 evaluation time should restart.

- Reference