3GPP TSG-RAN WG4 Meeting # 115	 R4-2505771

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on other RRM requirement on Rel-19 MIMO phase 5

Agenda Item:	7.24.4.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 MIMO phase 5.

- UL TCI state activation in Asymmetric DL sTRP/UL mTRP - Two TAs in Asymmetric DL sTRP/UL mTRP

- Discussion

2.1 Asymmetric DL sTRP/UL mTRP

Issue 2-1: For Scenario#2, how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - Wait for RAN1 reply to the LS on whether the existing agreement for a single downlink timing reference is extended to Scenario#2.

The RAN1 reply LS is as below:

Agreement: - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG Above applies for the case UE is configured with SSB-MTC-additionalPCI

Observation 1: From RAN1 reply LS, for the scenario that the UE is not configured with PL offset in joint/UL TCI state(s), UE maintains two downlink reference timings.

Similar as discussion in legacy two TA reference timing, there are two steps to decide the timing:

- Reference cell

- DL reference point of the reference cell

Reference cell

In legacy two TA in mDCI, the reference cell is defined as below:

For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG.

The reference cell will be SpCell or SCells which will be configured with DL data transmission, i.e. PDCCH/PDSCH.

While for scenario 2, the reference cell in the same TAG may not be configured with PDCCH/PDSCH transmission. It’s possible that the reference cell will only transmit SSB.

For scenario 2, there are two types of DL reference cell:

- Type 1: Normal Cell(s) with PDCCH/PDSCH, i.e. Cell in the same TAG, configured with PDCCH/PDSCH

- Type 2: UL-TRP(s) with only SSB, i.e. Cell in the same TAG, configured with only SSB

Then the reference cell candidates can be:

- Case 1: Include only Type 2.

- Case 2: Include both type 1 and type 2 cells.

For case 1, if there are only UL-only TRPs where SSB is transmitted, then UL-only TRPs can be used as reference cell. If there are multiple UL-only TRPs, UE can choose any one with the same TAG.

For case 2, for simplicity, it’s up to UE implementation to choose either type 1 or type 2 cell.

Proposal 1: For scenario 2, if there are only UL-only TRPs with SSB, any one of UL-only TRP in the same TAG can be chosen as reference cell.

Proposal 2: For Scenario 2, if there are two types of cells as candidate reference cells in the same TAG (i.e., cells with DL data and UL-only TRPs with only SSB), the UE can choose either type as the reference cell based on its implementation.

Reference Point

In legacy, the reference point is defined as below:

For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAGas the uplink signal.

the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAGas the uplink signal.

For scenario 2, if the refence cell is UL-only TRP. The DL reference signal can only be SSB. DL TCI state can be configured for PDCCH/PDSCH/CSI-RS/DM-RS. SSB can be used as the source RS in the TCI state but there is no DL TCI configured for SSB.

We suggest to modify the wording as:

the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with DLorJointTCIState or UL-TCIState having the same TAGas the uplink signal.

This ensures that the definition can include many scenarios, such as cells with legacy DL transmission and cells with only SSB, where SSB is included in UL TCI state.

Proposal 3: For scenario 2, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with DLorJointTCIState or UL-TCIState having the same TAG as the uplink signal.

RAN1 update the reference timing for PRACH for scenario 2.

    - the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

If there is only UL-TRP with SSB in the same TAG, there is no DL TCI state for SSB. Then the DL reference timing is not clear.

Observation 2: For PRACH, there maybe no DL TCI state if there is only UL-only TRP with SSB in the same TAG.

We suggest to modify DL TCI state to DLorJointTCIState or UL-TCIState which can include both DL state and UL state case. This modification allows for a more flexible and inclusive definition of the reference timing, ensuring that the UE can derive a valid timing reference even when there is no DL TCI state configured for SSB.

Proposal 4: For PRACH, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) associated with DLorJointTCIState or UL-TCIState having the same TAG as the uplink signal.

Issue 2-6: Whether to define UL TCI state activation requirement for Scenario#2? Way forward: discuss the following options in the next meeting for Scenario#2 - Option 1: No RRM impacts on TCI state switching requirement. - Option 2: Legacy SSB based UL TCI state activation requirement can be re-used.

For scenario 2, the UL TCI state activation requirement can re-use the legacy SSB based UL TCI state activation requirement since SSB will be transmitted.

Proposal 5: For scenario 2, the UL TCI state activation requirement can re-use the legacy SSB based UL TCI state activation requirement.

- Conclusion

In this contribution, we provide the following proposals：

Observation 1: From RAN1 reply LS, for the scenario that the UE is not configured with PL offset in joint/UL TCI state(s), UE maintains two downlink reference timings.

Proposal 1: For scenario 2, if there are only UL-only TRPs with SSB, any one of UL-only TRP in the same TAG can be chosen as reference cell.

Proposal 2: For Scenario 2, if there are two types of cells as candidate reference cells in the same TAG (i.e., cells with DL data and UL-only TRPs with only SSB), the UE can choose either type as the reference cell based on its implementation.

Proposal 3: For scenario 2, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with DLorJointTCIState or UL-TCIState having the same TAG as the uplink signal.

Observation 2: For PRACH, there maybe no DL TCI state if there is only UL-only TRP with SSB in the same TAG.

Proposal 4: For PRACH, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) associated with DLorJointTCIState or UL-TCIState having the same TAG as the uplink signal.

Proposal 5: For scenario 2, the UL TCI state activation requirement can re-use the legacy SSB based UL TCI state activation requirement.

- Reference