# 3GPP TSG-RAN WG4 Meeting #115	R4-250xxxx

# Malta, MT, 19-23 May, 2025

Agenda Item:	7.5.3

Source: 	Xiaomi

Title: 						Methods for reducing the number of UE Rx chains

Document for:	Discussion

- Introduction

Rel-19 agreeded a study item to study the feasibility and requirements impact on using shared Rx chain to support mutilple fragemented carriers in single band under intra-band NC CA.

In previous RAN4 meeting, the reference archirecture and target scope has been clarified and agreed as following:

#### Sub-topic 1-2: Study scope clarification Agreement: - Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz as illustrated below: #### Merge issue 2-0/2-1 discussion: Agreement: The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823)

In previous RAN4 meeting, following agreements were reached for evaulation of RF requirements impact:

RAN4#114bis #### Issue 2-1: How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers - Consider the following alternatives - Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication - Alternative 1 has no modification of CA configurations in RAN4 - Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers. - Alternative 1a has no modification of CA configurations in RAN4 - Alternative 2: use the new notation for CA configurations with single Rx RF Chain - Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers #### Issue 2-2: Discussion on semi-statically switch hardware resources procedure and fallback considerations - Further discussions on - CA fallback with/without UE architecture switching - Consider the fall back - from high order CA to lower order CA - from CA to single carrier mode - Decided by network - UE architecture switching: - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA - FFS on whether it is decided by UE itself or decided by network - FFS on whether the additional indication from UE side is needed #### Issue 2-3: Triggering condition to enable ”One Rx RF chain” RAN4 agree there are two aspects regarding the triggering condition enable/disable the one Rx RF chain mode: - Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed - Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified - Note: detailed conditions to enable/disable can be further discussed and decided separately

In this contribution, views on Methods for reducing the number of UE Rx chains were provided.

- Discussion

- Issue 2-1: Signaling related

On signalling aspect, our preference we shall take this feature based on existing band combination structure with addtional necessary indications.

When UE access to the network, network will inquire necessary information from UE and if UE supported fragemented carriers feature and network have such fragemented operation deployment scenario which may further inquire the supporting indications.

Proposal 1 : Supporting alternative 1a use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers or Alternative 2a use the new bandwidth class conditioned on the frequency separation between two carriers

On Rx sharing capability indication, UE capability on supporting the max separation between fragemented carriers can be different which is related to both RF and BB capacity.  We sugguest to introduce Rx sharing capability indication based on max frequency seperation  across carriers and number of CCs.

Proposal 2: Introduce ” Rx sharing capability indication” based on max frequency seperation  across carriers and number of CCs.

NR intra-band non-contiguous DL CA frequency separation classes with single Rx chain mode

| NR NC DL CA frequency separation class | Maximum allowed frequency separation | Maximum allowed number of CCs |
| --- | --- | --- |
| A | 100 MHz | 2 |
| B | 200MHz | 2 |
| C | 200 MHz | 4 |

- Number of Rx and MIMO layers

Refer to UE capability in terms of MIMO layers (no dedicated signaling on Rx antenna ports) in TR 38.823/TS 38.306

RAN2 capability signaling on PDSCH MIMO layer IE is specified as per CC per band per band combination basis with most flexibility.

Observation 1: RAN2 PDSCH MIMO layer capability IE is specified as per CC per band per band combination basis.

This SI target to enable single Rx RF chain to support multiple fragmented carriers with benefits of supporting high order MIMO layers or high order CA combinations. Reducing number of MIMO layers per CC can be another way to support more fragmented carriers with limited number of Rx chains.

The story of intra-band non-collocated CA is different which target to enable intra-band non-collocated CA by reduced MIMO layers per CC under the limitation Rx chains.

Theoretically, supporting more fragmented carriers can be achieved by shared RF chain and/or reduced MIMO layers/RX chain per carrier. However, this is out of the scope of this SI for reducing MIMO layers to achieve more fragmented carriers or high CA orders and actually the existing mechanism/UE capability signaling already allow such operation.

Under single RF Rx chain operation condition with limited Rx RF chain; UE may increase MIMO layers on supported carriers or extend supporting high order band combinations with more carriers pending on RF capacity and baseband processing capacity.

Proposal 3: UE shall allow to indicate separate MIMO layer capacity under single Rx chain operation condition and fallback to normal condition for same band combination.

- Issue 2-2: Discussion on semi-statically switch hardware resources procedure and fallback considerations

In our view, both network and UE need to be aligned for the state whether UE stayed in “single Rx chain mode” or normal mode for specific intra-band NC CA configuration and corresponding high CA orders. The supported number of MIMO layers and CA configuration with high orders under “single Rx chain mode” and normal mode can be different.

For UE UE architecture switching “Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA”, this shall be controlled by network otherwise UE and network may have mismatch with unexpected system performance degradation.

Proposal 4: Network and UE shall be aligned on UE state on “single Rx mode” or normal mode.

Proposal 5: CA fallback with/without UE architecture switching shall be controlled by network with necessary interaction between NW and UE.

Proposal 6: UE architecture switching shall be controlled by network with necessary interaction between NW and UE.

- Issue 2-3 Triggering condition to enable ”One RF Rx chain”

Based on initial evaluation on Rx requirement impact due to Tx leakage and in-gap interference under single Rx chain mode, the receiver performance impact is diverse pending on configuration (DL-UL separation, Wgap length, CHBW) ,UE implementation performance on AGC adjustment and ADC design and the in-gap power level and PSD between wanted carrier(s) and interference.

On how to decide to enable ”one RF Rx chain“ or fall-back to normal mode, this can be left to UE implemention as well as UE can guranteen the Rx performance.  Once UE detect ”single Rx chain operation” not suitable, then UE can indiate to NW and fall-back to single CC by defualt, NW can reconfigure UE with single CC or normal CA mode.

Proposal 7: UE shall allow to switch between “normal CA operation” and “one RF chain operation” by indication to NW.

- UE self-judgment with 1bit indication to network

There are several ways to enable triggering conditions, those options can be summarized in below table for pros and cos.

| Options | Spec impact | Signaling overhead | UE implementation complexity | Performance impact |
| --- | --- | --- | --- | --- |
| Option 1: Existing channel quality reporting and procedures | Minimal | No additional signaling | No additional reporting | Not possible to monitor Tx leakage and in gap interference before UE switched to single RX mode. Ping-pong effect between different states with frequently switching on/off |
| Option 2: UE measurement reporting for in gap interference | Big impact with NW configuration and UE reporting (RAN1, RAN2 also need to be involved) | Signaling on measurement configuration and reporting | UE needs to follow NW configuration for measurement and reporting | NW can manage interference Measurement gap and interruption required before UE enter into Single RX mode for reporting |
| Option 3: UE self-judgment with 1bit indication to network | Small impact with 1bits indication FFS whether measurement gap interruption required for monitoring interference before UE enter into CA mode | Limited signaling overhead on indication | UE self-judgment, no measurement reporting required | NW can aware whether UE states whether suitable for single Rx mode Measurement gap and interruption required before UE enter into Single RX mode for reporting |

With above summary table, we believe option 3 is workable solution considering signaling overhead, spec impact and performance.  One drawback of option 2 and option 3, UE needs interruption for in gap interference monitoring before configured by network with single Rx mode CA (UE needs switching and interruption to do interference measurement and switching back to normal mode). To avoid frequent interruption due to UE measurement with self-judgment, we can apply single shot measurement/interruption before UE indication to Network.  The timeline can be refereed to below figure.

Proposal 8: Whether allow interruption with measurement gap and indication to NW for UE self-assessment before configured as “single Rx mode” can be controlled by Network as optional feature.

Figure 1: Work flow on UE switching between normal mode and single Rx mode

- Conclusion

In this contribution, views on Methods for reducing the number of UE Rx chains were provided.

Proposal 1 : Supporting alternative 1a use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers.

Proposal 2: Introduce ” Rx sharing capability indication” based on max frequency seperation  across carriers and number of CCs.

NR intra-band non-contiguous DL CA frequency separation classes with single Rx chain mode e.g.

| NR NC DL CA frequency separation class | Maximum allowed frequency separation | Maximum allowed number of CCs |
| --- | --- | --- |
| A | 100 MHz | 2 |
| B | 200MHz | 2 |
| C | 200 MHz | 4 |

Observation 1: RAN2 PDSCH MIMO layer capability IE is specified as per CC per band per band combination basis.

Proposal 3: UE shall allow to indicate separate MIMO layer capacity under single Rx chain operation condition and fallback to normal condition under same band combination.

Proposal 4: Network and UE shall be aligned on UE state on “single Rx mode” or normal mode.

Proposal 5: CA fallback with/without UE architecture switching shall be controlled by network with necessary interaction between NW and UE.

Proposal 6: UE architecture switching shall be controlled by network with necessary interaction between NW and UE.

Observation 2: The pros and cons of different methods for Triggering condition to enable” One Rx RF chain “ is summarized in below table in terms of spec impact, signaling overhead, UE implementation complexity and performance impact.

| Options | Spec impact | Signaling overhead | UE implementation complexity | Performance impact |
| --- | --- | --- | --- | --- |
| Option 1: Existing channel quality reporting and procedures | Minimal | No additional signaling | No additional reporting | Not possible to monitor Tx leakage and in gap interference before UE switched to single RX mode. Ping-pong effect between different states with frequently switching on/off |
| Option 2: UE measurement reporting for in gap interference | Big impact with NW configuration and UE reporting (RAN1, RAN2 also need to be involved) | Signaling on measurement configuration and reporting | UE needs to follow NW configuration for measurement and reporting | NW can manage interference Measurement gap and interruption required before UE enter into Single RX mode for reporting Reporting overhead |
| Option 3: UE self-judgment with 1bit indication to network | Small impact with 1bit indication FFS whether measurement gap interruption required for monitoring interference before UE enter into CA mode | Limited signaling overhead on indication | UE self-judgment, no measurement reporting required | NW can aware whether UE state is suitable for single Rx mode Measurement gap and interruption required before UE enter into Single RX mode for reporting |

Proposal 7: UE shall allow to switch between “normal CA operation” and “one RF chain operation” by indication to NW as the workflow described in figure below

- UE self-judgment with 1bit indication to network

Figure 1: Work flow on UE switching between normal mode and single Rx mode

Proposal 8: Whether allow interruption and indication to NW for UE self-assessment before configured as “single Rx mode” can be controlled by Network as optional feature

- Single shot measurement gap /interruption can be considered to minimize impact

Reference

[1] R4-2420337, WF on NR FR1 DL fragmented carriers study, MTK

[2] R4-2502873, WF on NR FR1 DL fragmented carriers study, MTK

[3] R4-2505109, WF on fragmented DL, MTK

-------------------------------draft TP ------------------------------

TP for TR38.755

---------------Start TP---------------------------

8	Study of means for a UE to inform the network of appropriate CA configuration it can support

8.X	Indication of one Rx RF chain mode for DL fragmented CA

8.X.X	Indication of fragmented CA proposal from company B (Xiaomi)

On Rx sharing capability indication, UE capability on supporting the max separation between fragemented carriers can be different which is related to both RF and BB capacity.  We sugguest to introduce Rx sharing capability indication based on max frequency seperation  across carriers and number of CCs.

Table 8.X.X-1: NR intra-band non-contiguous DL CA frequency separation classes with single Rx chain mode

| NR NC DL CA frequency separation class | Maximum allowed frequency separation | Maximum allowed number of CCs |
| --- | --- | --- |
| I | 100 MHz | 2 |
| II | 200MHz | 2 |
| III | 200 MHz | 4 |

8.Y	Triggering condition to enable ”One Rx RF chain”

8.Y.X Triggering condition proposal from company B (Xiaomi)

Based on initial evaluation on Rx requirement impact due to Tx leakage and in-gap interference under single Rx RF chain mode, the receiver performance impact is diverse pending on configuration (DL-UL separation, Wgap length, CHBW), UE implementation performance on ADC design and PSD between wanted carrier(s) and in-gap interference.

On how to decide to enable ”one RF Rx chain“ or fall-back to normal mode, this can be left to UE implemention as well as UE can guranteen the Rx performance.  Once UE detect ”single Rx chain operation” is not suitable, then UE can indiate to NW and NW can reconfigure UE with single CC or normal CA mode.

There are several ways to enable triggering condition, those options were summarized in below table.

Table 8.Y.X-1: Options for triggering condition to enable ”One Rx RF chain”

| Options | Spec impact | Signaling overhead | UE implementation complexity | Performance impact |
| --- | --- | --- | --- | --- |
| Option 1: Existing channel quality reporting and procedures | Minimal | No additional signaling | No additional reporting | Not possible to monitor Tx leakage and in gap interference before UE switched to single RX mode. Ping-pong effect between different states with frequently switching on/off |
| Option 2: UE measurement reporting for in gap interference | Big impact with NW configuration and UE reporting (RAN1, RAN2 also need to be involved) | Signaling on measurement configuration and reporting | UE needs to follow NW configuration for measurement and reporting | NW can manage interference Measurement gap and interruption required before UE enter into Single RX mode for reporting |
| Option 3: UE self-assessment with 1bit indication to network | Small impact with 1bits indication FFS whether measurement gap interruption required for monitoring interference before UE enter into CA mode | Limited signaling overhead on indication | UE self-judgment, no measurement reporting required | NW can aware whether UE state is suitable for single Rx mode Measurement gap and interruption required before UE enter into Single RX mode for reporting |

With above summary, option 3 is preferred solution considering signaling overhead, specification impact and performance impact.  There is one issue for option 2 and option 3 that UE needs interruption for in gap interference monitoring before configured by network with single Rx RF mode CA (UE needs switching and interruption to do interference measurement/self-assessment and switching back to normal mode). To avoid frequent interruption due to UE measurement with self-assessment, single shot measurement/interruption can be applied before UE indication to Network.  The workflow can be refereed to below figure.

Figure 8.Y.X-1: Work flow on UE switching between normal mode and single Rx mode

------------------End of TP---------------------------