# 3GPP TSG-RAN WG4 Meeting #115   	R4-2505795

# Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on (e)RedCap RRM requirements in NTN for NR Phase 3

Agenda Item:	7.31.3.1

Document for:	Discussion

# 1Introduction

In the last RAN4#114 meeting, further discussion on RRM requirements for Rel-19 NR NTN phase3 took place and a WF was approved as detailed in [1]. In this contribution, we would like to provide our further discussion on (e)RedCap RRM requirements in NTN for NR Phase 3 and provide our proposals.

- Discussion

##### Issue 1-2-1-1: Feature 25-1(parallelSMTC) Online agreement Agreement: - For Feature 25-1 (parallelSMTC), not differentiate 1Rx and 2Rx RedCap UEs for this UE capability. - RedCap UE is mandatory to support parallel measurements on 2 SMTC-s for a single frequency carrier - ‘Parallel’ means the SMTCs are TDM with proximity distance. ##### Issue 1-2-1-1a: The proximity distance between two SMTCs <Way Forward>: - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms. - Y=5 ms

In NTN Rel-17, a NTN UE shall at the least be capable of 2 SMTCs , and up to 4 SMTCs based on UE capability, with same periodicity but different offsets to measure neighbour satellites and the serving satellites.  RAN4 has agreed in last meeting to define the proximity distance between two SMTCs for (e)RedCap UEs in NTN. The main remaining issue is on the magnitude of the proximity distance. The proximity distance between the two SMTCs account for the time UE needed for processing the measurement, and for legacy R17 NR NTN UE the number is 4ms. Considering the reduced complexity of (e)RedCap, we are fine to relax the value of proximity distance for NTN (e)RedCap UE.

Proposal 1: When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

- the two SMTC occasions are fully or partially overlapping in time domain, or

- the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms.

  - Y=5 ms

#### Issue 3-1-1: General HD-FDD related impact Online agreement Agreement: - The legacy requirements and applicable conditions can be reused as baseline: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - [Minimum requirement for L1 indication for RLM and LR] - Scheduling availability of UE performing intra/inter measurements RAN4 to further discuss: - The legacy requirements and applicable conditions can be reused as baseline for defining requirements of RedCap UE in NTN scenario, and discuss whether there is spec impact for the following requirements by HD-FDD: - FFS: Scheduling availability of UE during L1-RSRP measurement - Other requirement is not precluded - Candidate options for the potential spec impact for each of the above requirements (if needed): - Option a: the requirement is extended, e.g., the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Option b: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Option c: The requirement applies when there is at least one RS within each measuremen interval. - FFS the same or different option for DRX and non-DRX

For RedCap UE operating in HD-FDD mode, overlapping between DL reception and UL transmission may occur since it cannot perform both operations in parallel. During discussion in R17 RedCap, the HD-FDD impact was considered and reflected in the related requirements. We support that to reuse the principle for RedCap UE in TN network as baseline. Based on last meeting progress, RAN4 need to further check the HD-FDD impact on “Minimum requirement for L1 indication for RLM and LR” and “Scheduling availability of UE during L1-RSRP measurement”.

The legacy requirement related to HD-FDD handling for RLM and BFD is copied below:

8.1B.6 Minimum requirement for L1 indication When DRX is not used for HD-FDD RedCap UEs, TIndication_interval,RedCap is max(10 ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in clause 8.1B.2 if the RLM-RS resource is SSB, or TCSI-RS specified in clause 8.1B.3 if the RLM-RS resource is CSI-RS, under the following condition - For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period. In case DRX is used for HD-FDD RedCap UEs, TIndication_interval,RedCap is Max(10ms, 1.5 × DRX_cycle_length, 1.5 × TRLM-RS,M)) if DRX_cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX_cycle_length is greater than 320ms, under the following condition - For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period. 8.5B.4 Minimum requirement for L1 indication for Redcap When the radio link quality on all the RS resources in set is worse than Qout_LR_Redcap, layer 1 of the UE shall send a beam failure instance indication to the higher layers The beam failure instance evaluation for the RS resources in set shall be performed as specified in clause 6 in TS 38.213 [3]. Two successive indications from layer 1 shall be separated by at least TIndication_interval_BFD_Redcap. When DRX is not used, TIndication_interval_BFD_Redcap is max(2ms, TSSB-RS,M) ) or max(2ms, TCSI-RS,M), where TSSB-RS,M and TCSI-RS,M is the shortest periodicity of all RS resources in set for the accessed cell, corresponding to either the shortest periodicity of the SSB in the set or CSI-RS resource in the set . When DRX is used, for SSB based link quality measurement, - TIndication_interval_BFD_Redcap = Max(1.5 × DRX_cycle_length, 1.5 × TSSB-RS,M), if DRX_cycle_length ≤ 320 ms, - TIndication_interval_BFD_Redcap = DRX_cycle_length, if DRX_cycle_length > 320 ms. When DRX is used, for CSI-RS based link quality measurement, - TIndication_interval_BFD_Redcap = Max(1.5 × DRX_cycle_length, 1.5 × TCSI-RS,M), if DRX_cycle_length ≤ 320 ms, - TIndication_interval_BFD_Redcap = DRX_cycle_length, if DRX_cycle_length > 320 ms. For HD-FDD UE, the above conditions and requirements TIndication_interval_BFD_Redcap apply given that for each BFD-RS configuration, at least one BFD-RS sample must fall with DL occasion within an indication period

We understand the applicability condition that “For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period” is to make sure UE have chance to take a measurement sample during one L1 indication interval to guarantee the performance of UE evaluation for the downlink radio link quality. We think this applicability condition could be reused for NTN scenario.

Proposal 2: The legacy requirements and applicable conditions can be reused as baseline for minimum requirement for L1 indication for RLM and BFD.

During last meeting discussion, companies mentioned the HD-FDD handling should be re-visited for scheduling availability of UE during L1-RSRP measurement. For RedCap UE in TN network, RAN4 agreed in Rel-17 that “CBD evaluation is always prioritized over the UL transmission of HD-FDD for RedCap”, and the below captured requirements reflect the agreement. However, Based on RAN1 agreements, there is a NTN specific collision handling rules that Network is allowed to indicate UL overriding DL. So, there would exist a misalignment between RAN1 and RAN4 on the collision handling rule if we follow legacy way.

9.5B.6.1 Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1 For FD-FDD and TDD RedCap UEs, there are no scheduling restrictions due to L1-RSRP measurement performed on SSB and CSI-RS configured as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1. For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1. 9.5B.6.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement. - For FD-FDD and TDD RedCap UEs, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement. - For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1.

From our perspective, it would be good to introduce the similar applicability condition as RLM and BFD for CBD. However, there is no such requirement as L1 indication interval for CBD. If RAN4 to align the HD-FDD handling between RLM, BFD and CDB, many effort would be needed to figure out the “measurement interval” for CBD. So, from our perspective, we support to go with the strategy as option b.

Proposal 3: RAN4 to clarify that the requirements of CBD evaluation period apply provided that no CBD resource is dropped due to collision with UL transmission for HD-FDD RedCap UE.

- Conclusion

Proposal 1: When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

- the two SMTC occasions are fully or partially overlapping in time domain, or

- the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms.

  - Y=5 ms

Proposal 2: The legacy requirements and applicable conditions can be reused as baseline for minimum requirement for L1 indication for RLM and BFD.

Proposal 3: RAN4 to clarify that the requirements of CBD evaluation period apply provided that no CBD resource is dropped due to collision with UL transmission for HD-FDD RedCap UE.

- Reference

- R4-2504912, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT