3GPP TSG-RAN WG4 Meeting #115	R4-2505803

# Malta, 19th – 23rd May 2025

Agenda item:			7.8.4

Source:	Nokia

Title:	On BS RF requirements due to introduction of 7 MHz channel bandwidth

Document for:	Approval

# Introduction

During RAN4#114bis, a WF on 7 MHz channel bandwidth has been approved with a number of agreements on BS RF requirements [1]. This document addresses remaining open issues (in-channel selectivity and narrowband Rx IM requirements) due to introduction of 7 MHz channel bandwidth.

# Discussion

This document provides a proposal on in-channel selectivity requirements (requirements to be added to all BS classes with Wide Area BS class example shown below) due to the introduction of 7 MHz channel bandwidth. It should be noted that 14 RBs are proposed for the interfering signal which corresponds to 40% of the maximum transmission bandwidth configuration for 7 MHz channel bandwidth (the same assumption has been made for 3 and 5 MHz channel bandwidths). For the wanted signal, existing FRC (G-FR1-A1-7 with 15 allocated RBs) is proposed, according to previous agreement made in RAN4. It should be also noted there is the following statement in TS 38.104:

“Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1.”

Table 7.8.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-20 | -102.8 | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 7 | 15 | G-FR1-A1-7 | -100.6 | -79.9 | DFT-s-OFDM NR signal, 15 kHz SCS, 14 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -92.3 | -71.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -101.3 | -81.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -98.8 | -78.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -92.6 | -71.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -98.2 | -78.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -92.7 | -71.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 7.8.2-1a: Wide Area BS in-channel selectivity for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 3 |  |  | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 |  |  | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 7 |  |  | -79.9 | DFT-s-OFDM NR signal, 15 kHz SCS, 14 RBs |
| 10, 15, 20, 25, 30, 35 | FRC A14-1 in Annex A.14 in TS 36.104 [13] | -124.3 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 |  |  | -71.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 3 |  |  | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 |  |  | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 7 |  |  | -79.9 | DFT-s-OFDM NR signal, 15 kHz SCS, 14 RBs |
| 10, 15, 20, 25, 30, 35 | FRC A14-2 in Annex A.14 in TS 36.104 [13] | -130.2 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 |  |  | -71.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| NOTE: Interfering signal is placed in one side of the Fc, while the NB-IoT PRB is placed on the other side. Both interfering signal and NB-IoT PRB are placed at the middle of the available PRB locations. The wanted NB-IoT tone is placed at the centre of this NB-IoT PRB. |  |  |  |  |

Furthermore, we would like to come back to the decision on interfering CW signal offset (±355kHz) for the narrowband Rx IM requirement. This offset was calculated on the basis of the following definition in TR 38.817-02, Clause 7.7, assuming RB size of 360kHz:

“RBinterferer = RB size with the highest SCS defined for the modulated interfering signal channel bandwidth (in kHz).”

However, we propose to use ±400kHz offset assuming RB size of 180kHz since for 7MHz channel bandwidth only 15kHz SCS is supported and there is the following statement in TS 38.104/38.141-1:

“The subcarrier spacing for the modulated interfering signal shall in general be the same as the subcarrier spacing for the wanted signal, except for the case of wanted signal subcarrier spacing 60 kHz and BS channel bandwidth <=20MHz, for which the subcarrier spacing of the interfering signal should be 30 kHz.”

# Conclusion

This document provides proposals on in-channel selectivity and narrowband Rx IM requirements due to introduction of 7 MHz channel bandwidth. It is proposed to take this into account for final RAN4 agreement.

# Reference

[1] R4-2505098; WF on requirements for 7MHz channel bandwidth; Ericsson