3GPP TSG-RAN WG4 Meeting #115	R4-2505807

# Malta, 19th – 23rd May 2025

Agenda item:			10.1

Source:	Nokia

Title:	On RAN task to work on framework simplification for co-location/co-existence requirements

Document for:	Approval

# Introduction

During RAN4#114bis, the WF on framework simplification for co-existence and co-location requirements was agreed in [1]. This document addresses how co-existence and co-location tables can be further updated to address issues identified in this WF.

# Discussion

Taking into account issues identified in the WF [1], it is proposed to further update (modifications shown by the track changes) the following co-existence and co-location tables:

- Co-existence requirement

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | System type to co-exist with | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| Frequency range of downlink operating band | GSM850, CDMA850 or GSM900 | -57 | 100kHz | Note 1 |
|  | DCS1800 or PCS1900 | -47 | 100kHz |  |
|  | Other (Note 5) | -52 | 1MHz |  |
| Frequency range of uplink operating band | GSM850, CDMA850, GSM900, DCS1800 or PCS1900 | -61 | 100kHz | Note 1, Note 2, Note 3, Note 4 |
|  | Other | -49 | 1MHz |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 or n67; the co-existence requirements in this table does not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for the excluded frequency range may be covered by local or regional requirements. NOTE 2: For a BS operating in Band n28, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies 4 MHz above the Band n28 downlink operating band.. NOTE 3: For a BS operating in Band n29, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band. NOTE 4: For a BS operating in Band n67, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. NOTE 5: Applies to other operating bands except Band 107 and 108. |  |  |  |  |

- Co-location requirement

The measurement bandwidth for the requirements in table 6.6.5.2.4-1 is 100 kHz.

Table 6.6.5.2.4-1: BS spurious emissions basic limits for BS co-located with another BS

| Frequency range for | Type of co-located BS | Basic limits |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| co-location requirement |  | WA BS (dBm) | MR BS (dBm) | LA BS (dBm) |  |
| Frequency range of co-located uplink operating band | GSM850, CDMA850 or GSM900 | -98 | -91 | -70 | Note 1 |
|  | DCS1800 or PCS1900 | -98 | -91 | -80 |  |
|  | n51, n91, n93 | N/A | N/A | -88 |  |
|  | n53 | N/A | -91 | -88 |  |
|  | n100, n101 | -96 | N/A | N/A |  |
|  | n96, n102 | N/A | -90 | -87 |  |
|  | n104 | -95 | -90 | -87 |  |
|  | Other (Note 2) | -96 | -91 | -88 |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.6.5.2.4-1 do not apply for the frequency range extending ΔfOBUE immediately outside the BS transmit frequency range of a downlink operating band (see table 5.2-1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [15]. NOTE 2: Applies to other operating bands except Band 47. |  |  |  |  |  |

# Conclusion

This document addresses how co-existence and co-location tables can be further updated, to address issues identified in the WF [1]. It is proposed to take these changes into account for final specifications modifications.

Furthermore, it is proposed to inform RAN on the status of their task (see proposed draft LS in Clause 4), according to their proposed schedule in [2].

# Draft LS to RAN

Title:	Reply LS on framework simplification for co-location/co-existence requirements

Response to:	-

Release:	Release 19

Work Item:	-

Source:	RAN4

To:	RAN

Cc:

Contact Person:

- Name:		TBD

- E-mail Address:	TBD

Attachments: -

- Overall Description:

RAN4 would like to provide the status of RAN task regarding framework simplification for co-location/co-existence requirements. RAN4 had dedicated agenda in Q1 and Q2 2025 and agreed to maintain (and simplify) tables for co-existence and co-location requirements in each specification. The following set of CRs have been agreed in RAN4:

| Specification |  | RAN4 Tdoc |
| --- | --- | --- |
| 38.104 | NR BS Core |  |
| 38.141-1 | NR BS Conf - Conducted |  |
| 38-141-2 | NR BS Conf - OTA |  |
| 38.106 | NR Repeater - Core |  |
| 38.115-1 | NR Repeater -Conf Cond. |  |
| 38.174 | IAB Core |  |
| 38.176-1 | IAB Conf Conducted |  |
| 38.176-2 | IAB Conf OTA |  |
| 37.105 | AAS BS Core |  |
| 37.145-1 | AAS BS Conf Conducted |  |
| 37.145-2 | AAS BS Conf OTA |  |
| 37.104 | MSR BS Core |  |
| 37.141 | MSR BS Conf |  |
| 36.104 | E-UTRA BS Core |  |
| 36.141 | E-UTRA BS Conf |  |
| 36.106 | E-UTRA Repeater Core |  |
| 36.143 | E-UTRA Repeater Conf |  |

2. Actions:

To RAN:

RAN4 respectfully asks RAN to take the above information into account and approve set of RAN4 agreed CRs (mentioned above) during RAN#108.

3. Date of Next TSG-RAN4 Meetings:

RAN WG4 Meeting #116    		August 25 – August 29, 2025	         Bengaluru, India (TBC), IN

# Reference

[1] R4-2504741; Way forward for [114bis][331] BSRF_Simplify_CoLo_Coex, Ericsson

[2] RP-243288, Moderator's summary of discussion about R19-RAN4-BSRF, RAN4 Chair