3GPP TSG RAN WG4 Meeting #115		R4-2505810

Malta, MT, May 19th-23rd, 2025

Agenda item:	7.14.4.1

Source:	Telecom Italia S.p.A., Vodafone, Deutsch Telekom, Orange

Title:	Proposals for Rel-19 TRP TRS requirements working procedure

WI/SI:	TRP_TRS_MIMO_OTA_Ph3

Release:	Rel-19

Document for:	Approval

# 1	Introduction

During RAN4#114-bis meeting, discussion on Rel-19 TRP TRS requirements related to the WI [1] was held which outcomes and agreements have been captured in the WF [2]. This contribution proposes some changes to the working procedure for TRP TRS Performance Test Campaign to define requirements in order to address some of the open points from the discussion at RAN4#114-bis meeting.

# 2	1Tx FR1 TRP TRS requirements

Proposal 1: Adopt the following changes to the working procedure to define the TRP TRS requirements:

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: n3, n5, n7, and n8 [2Tx at bands n41;]

  - Use scenarios:

    - For 1Tx, consider both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR;

    - For 2Tx, only consider Browsing mode (Hand phantom only) requirements in Rel-19;

  - Hand Phantom: Wide Grip Hand

  - Operation mode: SA with 1Tx  as phase 1. SA with 2Tx (non-coherent UL-MIMO) as phase 2 after 2Tx lab alignment activity.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: Size 1(width >72mm and ≤92mm)

  - DUT capability: support for Bands those listed in the latest WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: 2023-2025

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. For bands support both PC2 and PC3, focus on PC2 measurements, but both PC2 and PC3 requirements are needed. For PC3 bands, only PC3 requirements.

- Devices provisioning:

  - Any 3GPP member can work with test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

  - Logistical aspects for devices provisioning to the labs are TBD

- Test results submitting:

  - UE information disclosure: the following rules should be considered

    - UE model information based on template [Rel-18 R4-2408104 as starting point, update is needed] can be shared to MCC. RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure.

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - laboratories use the same template to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

      - Information of the devices that are going to be measured shall be shared with the RAN4 Secretary as soon as available (i.e., before the measurement activity on such devices starts)

      - The RAN4 Secretary updates the summary of statistical information  and publishes it to 3GPP RAN4 (i.e., living document) in order to monitor the achievement of the thresholds in a timely manner and take the proper actions if these are not met

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - Statistics of UE information can be shared to RAN4 reflector after TBD meeting by RAN4 secretary after post-processing

    - Consider following aspects as thresholds related to the devices pool for Rel-19

      - Minimum number of devices for each band, each device size, each power class: 60

      - Minimum number of device models: 50

      - Minimum number of devices' vendors: 6

      - Percentage of devices from 2023 to 2025: 100%

      - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

  - Laboratories use the same worksheet template in [Rel-18 R4-2315844 as starting point, update is needed] to submit the measurement results for Rel-19 3GPP TRP TRS performance data pool.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - The allowed maximum number of submitted devices from each lab is [15] (depends on how many test labs will join the activity) with a minimum of 3 devices.

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to share on the RAN4 reflector (for example - how many devices have been measured and on which bands)

  - All the measurement results shall be submitted by RAN4#117 meeting (i.e., not later than the TDoc submission deadline) in order to provide enough time to the companies to analyse the provided results and let RAN4 have an initial discussion on the requirements’ values during the meeting.

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): 60

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-19.

  - Method of limits derivation: per-band Data driven approach, working procedure in Rel-18 on how to specify the requirement is baseline.

  - Inclusion in the data set of measurement results related to faulty devices shall be avoided:

    - All the measurements results shall be submitted by RAN4#117 meeting

    - Method to identify faulty devices is FFS; some options are reported below:

      - [Define a threshold under which the device is considered a faulty sample

      - Define a delta with respect to the maximum measured value under which the device is considered a faulty sample

      - Analysis of the DCF curve to identify deviations of the lower tail and therefore a faulty sample of the device

      - Other options are not precluded]

    - Each identified faulty device can be replaced with another sample of the same model and measured by the test lab by RAN4#118 meeting

    - If the new measurement results cannot be provided by RAN4#118 meeting, the values related to the faulty device will be removed from the data set

  - The value at [80%] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - For a band supporting both PC2 and PC3, specify PC3 requirement based on finalized PC2 requirements, for TDD bands 3dB offset, for FDD bands TBD.

  - Definition of PDA hand related TRP/TRS requirements (Size2) in legacy band of Rel-18 is based on offset, as follows:

    - To adopt the same offset defined by ETSI for the bands reported in ETSI EN 301-908-13 V13.4.1 (to be released), i.e., bands n1, n3, n7, n8, n20, n28, n38, and n40

    - To adopt the offset of 0 dB for all the other bands

- Test lab procedures for 1Tx

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - For UE support PC2 at one band, PC3 testing is not needed.

  - For new labs of 1Tx requirements work, those not declared aligned in Rel-17 and Rel-18 also need to complete 1Tx lab alignment on at least n78 (high band) and n28 (low band).

  - Only legacy grid, i.e., 15degree TRP and 30 degree TRS, is used for this measurements campaign

- Test lab procedures for 2Tx

  - FFS

# 3	Conclusions

In the following, the observations and proposals of the contribution are summarized:

Proposal 1: Adopt the following changes to the working procedure to define the TRP TRS requirements:

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: n3, n5, n7, and n8 [2Tx at bands n41;]

  - Use scenarios:

    - For 1Tx, consider both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR;

    - For 2Tx, only consider Browsing mode (Hand phantom only) requirements in Rel-19;

  - Hand Phantom: Wide Grip Hand

  - Operation mode: SA with 1Tx  as phase 1. SA with 2Tx (non-coherent UL-MIMO) as phase 2 after 2Tx lab alignment activity.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: Size 1(width >72mm and ≤92mm)

  - DUT capability: support for Bands those listed in the latest WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: 2023-2025

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. For bands support both PC2 and PC3, focus on PC2 measurements, but both PC2 and PC3 requirements are needed. For PC3 bands, only PC3 requirements.

- Devices provisioning:

  - Any 3GPP member can work with test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

  - Logistical aspects for devices provisioning to the labs are TBD

- Test results submitting:

  - UE information disclosure: the following rules should be considered

    - UE model information based on template [Rel-18 R4-2408104 as starting point, update is needed] can be shared to MCC. RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure.

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - laboratories use the same template to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

      - Information of the devices that are going to be measured shall be shared with the RAN4 Secretary as soon as available (i.e., before the measurement activity on such devices starts)

      - The RAN4 Secretary updates the summary of statistical information  and publishes it to 3GPP RAN4 (i.e., living document) in order to monitor the achievement of the thresholds in a timely manner and take the proper actions if these are not met

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - Statistics of UE information can be shared to RAN4 reflector after TBD meeting by RAN4 secretary after post-processing

    - Consider following aspects as thresholds related to the devices pool for Rel-19

      - Minimum number of devices for each band, each device size, each power class: 60

      - Minimum number of device models: 50

      - Minimum number of devices' vendors: 6

      - Percentage of devices from 2023 to 2025: 100%

      - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

  - Laboratories use the same worksheet template in [Rel-18 R4-2315844 as starting point, update is needed] to submit the measurement results for Rel-19 3GPP TRP TRS performance data pool.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - The allowed maximum number of submitted devices from each lab is [15] (depends on how many test labs will join the activity) with a minimum of 3 devices.

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to share on the RAN4 reflector (for example - how many devices have been measured and on which bands)

  - All the measurement results shall be submitted by RAN4#117 meeting (i.e., not later than the TDoc submission deadline) in order to provide enough time to the companies to analyse the provided results and let RAN4 have an initial discussion on the requirements’ values during the meeting.

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): 60

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-19.

  - Method of limits derivation: per-band Data driven approach, working procedure in Rel-18 on how to specify the requirement is baseline.

  - Inclusion in the data set of measurement results related to faulty devices shall be avoided:

    - All the measurements results shall be submitted by RAN4#117 meeting

    - Method to identify faulty devices is FFS; some options are reported below:

      - [Define a threshold under which the device is considered a faulty sample

      - Define a delta with respect to the maximum measured value under which the device is considered a faulty sample

      - Analysis of the DCF curve to identify deviations of the lower tail and therefore a faulty sample of the device

      - Other options are not precluded]

    - Each identified faulty device can be replaced with another sample of the same model and measured by the test lab by RAN4#118 meeting

    - If the new measurement results cannot be provided by RAN4#118 meeting, the values related to the faulty device will be removed from the data set

  - The value at [80%] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - For a band supporting both PC2 and PC3, specify PC3 requirement based on finalized PC2 requirements, for TDD bands 3dB offset, for FDD bands TBD.

  - Definition of PDA hand related TRP/TRS requirements (Size2) in legacy band of Rel-18 is based on offset, as follows:

    - To adopt the same offset defined by ETSI for the bands reported in ETSI EN 301-908-13 V13.4.1 (to be released), i.e., bands n1, n3, n7, n8, n20, n28, n38, and n40

    - To adopt the offset of 0 dB for all the other bands

- Test lab procedures for 1Tx

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - For UE support PC2 at one band, PC3 testing is not needed.

  - For new labs of 1Tx requirements work, those not declared aligned in Rel-17 and Rel-18 also need to complete 1Tx lab alignment on at least n78 (high band) and n28 (low band).

  - Only legacy grid, i.e., 15degree TRP and 30 degree TRS, is used for this measurements campaign

- Test lab procedures for 2Tx

  - FFS

# 4	References

- RP-243277, “Revised WID: WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3”, vivo, December 2024

- R4-2504763, “WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3”, vivo, April 2025