3GPP TSG-RAN WG4 Meeting #115		R4-2505814

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.24.4.2

Source:		Samsung

Title:	Discussion on RRM requirements of other aspects in Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

In last RAN4#114bis meetings, the WF was approved in [1]. There are still open issues in RRM requirements for CJTC reporting and asymmetric DL sTRP/UL mTRP scenarios. In this contribution, we continue to discuss the RRM requirements for asymmetric DL sTRP/UL mTRP scenarios in Rel-19 MIMO phase 5.

- Discussion

CJTC Reporting

In RAN4#114bis meeting, the latest agreement is approved in [1] as:

Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration? Agreement: - Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified)

In last meeting, we support to add the core requirements including measurement period even the number of samples may be 1. Companies provided simulation results and propose to choose number of samples from 1, 2, 3.

By averaging more samples, of course there can be noise suppression benefit. By increasing the SNR condition, this benefit gain will not so obvious

According to the observation of simulation results, there is no huge gain if change the number of samples from 1 to 3.

In legacy Rel-15 L1-RSRP measurement, although there can be different sets of multiple samples in measurement periods. However, the accuracy requirement is based on 1 sample.

Proposal 1: Define the measurement period based on 1 sample. The SNR condition and accuracy can be decided in performance phase.

For the scheduling restriction, to achieve accurate estimation, scheduling restriction is needed.

Agreement(RAN1#116bis) For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for delay offset reporting Dn,offset, i.e. (AD, MD), are NW-configured via higher-layer (RRC) signalling from the following candidate values: - AD ={0.5CP, 0.75CP, CP, 1.5CP, 2CP, , , } where CP and denote the length of the cyclic prefix according to the current specifications (for normal CP) within a slot and the SCS, respectively - MD ={32, 64} In addition, the inside/outside range for the 1-bit indicator dn is equal to [0, CP]. Agreement(RAN1#118) For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset, i.e. AD, at least support the following values: {0.5CP, CP}

When the UE performed based on measurement on CSI-RS recourses, they are assumed from different TRPs with delay offset and frequency offset. If the delay offset is larger than CP, ISI will degrade the measurement performance. RAN1 introduced the 1-bit indicator to report whether is larger than the length of CP.

To achieve accurate measurement, UE is not expected to receive other DL signals for interfering.

Proposal 2: The scheduling restriction is specified as:

For the UE is configured with CJTC reporting, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for CQI on the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting, and on one data symbol before and one data symbol after the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting.

Proposal 3: If CJTC measurement collides with other L1 measurement, follow legacy principle, UE is required to measure one of but not both of different measurements when measurement resources are at the same time.

Asymmetric DL sTRP/UL mTRP

For Scenario#2, RAN4 sent LS to RAN1 in RAN4#113 meeting. The secarios#2 is defined in RAN1 reply LS.

Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s).

In last RAN1#120bis meeting, a reply LS has been approved in [2] in RAN1.

Question: Based on RAN1 LS (R1-2410870), RAN4 discussed timing requirement for two TA enhancement for asymmetric DL sTRP/ UL mTRP. RAN4 would like to check with RAN1 whether RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). Answer: The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). And RAN1 made the following new agreement for further clarification: Agreement: - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG - Above applies for the case UE is configured with SSB-MTC-additionalPCI.

According to the reply LS and latest RAN1 progress, our understanding is two TAGs will be used for inter-cell scenarios.

The mechanism of PRACH and two TAs extension for inter-cell are the same as the mechanism in Rel-18 m-DCI scenarios.

For the TRP with DL/UL, the reference point can use similar definition in Rel-18 two TA from DLorJointTCIState list. For the TRP with only SSB, it can only be used by SSB.

Proposal 4: For Asymmetric DL sTRP/UL mTRP Scenario#2, RAN4 to define two DL reference points for uplink timing requirements.

- For the reference point for PRACH transmission triggered by PDCCH order, reuse the same requirements in Rel-18 two TA for inter-cell operation.

- For inter-cell operation, for the reference point for PUCCH/PUSCH/SRS

  - For first TAG, reuse the similar requirements in Rel-18 two TA for serving cell by DLorJointTCIState

  - For second TAG, use the SSB timing from additional PCI cell.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for for “CJTC reporting” and “asymmetric DL sTRP/UL mTRP scenarios” in Rel-19 MIMO phase 5:

Proposal 1: Define the measurement period based on 1 sample. The SNR condition and accuracy can be decided in performance phase.

Proposal 2: The scheduling restriction is specified as:

For the UE is configured with CJTC reporting, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for CQI on the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting, and on one data symbol before and one data symbol after the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting.

Proposal 3: If CJTC measurement collides with other L1 measurement, follow legacy principle, UE is required to measure one of but not both of different measurements when measurement resources are at the same time.

Proposal 4: For Asymmetric DL sTRP/UL mTRP Scenario#2, RAN4 to define two DL reference points for uplink timing requirements.

- For the reference point for PRACH transmission triggered by PDCCH order, reuse the same requirements in Rel-18 two TA for inter-cell operation.

- For inter-cell operation, for the reference point for PUCCH/PUSCH/SRS

  - For first TAG, reuse the same requirements in Rel-18 two TA for serving cell

  - For second TAG, use the SSB timing from additional PCI cell.

- Reference

[1] R4-2504903, WF on RRM requirements for NR_MIMO_Ph5_Part2, MediaTek

[2] R4-2505308, Reply to RAN4 LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP, RAN1