3GPP TSG-RAN4 Meeting #115	R4-2505815

# Malta, MT, 19th - 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on event trigger Reporting based on L1-RSRP measurements for serving cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the work split in WF R4-2504902, event trigger Reporting based on L1-RSRP measurements are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RRM requirements for event trigger Reporting based on L1-RSRP measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec is not completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.x1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------Start Change #1-------------------------

9.x1	Event trigger Reporting based on L1-RSRP measurements

9.x1.1	Introduction

When configured by the network with CSI-ReportConfig with eventType-r19 and with dl-OrJointTCI-StateList, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for event trigger reporting based on L1-RSRP measurement. The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements within the active BWP.

The UE shall be able to measure all CSI-RS resources and/or SSB resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet within the CSI-ResourceConfig settings configured for L1-RSRP for the active BWP, provided that the number of resources, including the number of SSB resources of the cell with PCI different from serving cell configured for L1-RSRP measurements in clause xxx, does not exceed the UE capability indicated by beamManagementSSB-CSI-RS.

The UE shall report the measurement quantity (reportQuantity) and send aperiodic reports, according to the reportConfigType according to the CSI reporting configuration(s) (CSI-ReportConfig) for the active BWP.

9.x1.2	Requirements applicability

The requirements in this clause apply, provided:

-	The CSI-RS or SSB or CSI-RS and SSB resources configured for L1-RSRP measurements are measurable.

An SSB resource configured for L1-RSRP shall be considered measurable when for each relevant SSB the following conditions are met:

-	L1-RSRP related side conditions given in clauses 10.1.20.1 for FR2, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.4.1 for a corresponding band.

A CSI-RS resource configured for L1-RSRP shall be considered measurable when for each relevant CSI-RS the following conditions are met:

-	L1-RSRP related side conditions given in clauses 10.1.20.2 for FR2, for a corresponding band,

-	CSI-RS_RP and CSI-RS Ês/Iot according to Annex B.2.4.2 for a corresponding band.

A CSI-RS and SSB resource configured for L1-RSRP shall be considered measurable when the measurable resource conditions are met for both CSI-RS resource and SSB resource.

Requirements are defined for periodic, semi-persistent and aperiodic resources.

In case the SSB resources configured for L1-RSRP measurements outside SMTCs are overlapped with the PDSCH/PDCCH that is associated to a PCI different from serving cell, L1-RSRP measurement performance degradation is expected.

9.x1.3	Measurement Reporting Requirements

The UE shall send L1-RSRP reports only for report configurations configured for the active BWP.

The UE shall report the L1-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1 dB step size according to xxx.

9.x1.3.3	Event Triggered Reporting

The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE transmits first PUCCH over the air interface.

The reporting delay is defined as: TL1_RSRP_Measurement_Period + Tfirst_UL_channel

FFS on TL1_RSRP_Measurement_Period, Tfirst_UL_channel

9.x1.4	L1-RSRP measurement requirements

9.x1.4.1	SSB based L1-RSRP Reporting

The requirements in this section apply for SSB from serving cell for both beams of indicated TCI states and if reference signal configured by the newBeamResourceSet-r19 is SSB. The UE shall be able to complete the measurement within the period defined in Table:

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*N)*TDRX |
| Note: TDRX is the DRX cycle length. |  |

TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, UE shall perform L1-RSRP based on TSSB_current and TSSB_new.

TSSB_current is the periodicity of the SSB-Index of the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state.

TSSB_new is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

The definition of M, P, N refer to clause 9.5.4.

FFS on “event1” and “event7”

9.x1.4.2	CSI-RS based L1-RSRP Reporting

The requirements in this section apply for CSI-RS from serving cell for both beams of indicated TCI states and if reference signal configured by the newBeamResourceSet-r19 is CSI-RS. The UE shall be able to complete the measurement within the period defined in Table:

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320 ms | max(ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320 ms | ceil(M*P*N)*TDRX |
| NOTE 1: TDRX is the DRX cycle length. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

TCSI-RS is the periodicity of the CSI-RS configured for L1-RSRP measurement.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, UE shall perform L1-RSRP based on TCSI-RS_current and TCSI-RS_new.

TCSI-RS_current is the periodicity of the CSI-RS of the reference signal in the indicated TCI state.

TCSI-RS_new is the periodicity of the CSI-RS of the reference signal configured by the newBeamResourceSet-r19.

The definition of M, P, N refer to clause 9.5.4.

FFS on “event1” and “event7”

9.x1.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement

The requirements in clause 9.5.5 applied.

9.x1.6	Scheduling availability of UE during L1-RSRP measurement

The requirements in clause 9.5.6 applied.

----------------------------------------End Change #1-------------------------