3GPP TSG-RAN WG4 Meeting #115		R4-2505817

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.24.5

Source:		Samsung

Title:	Discussion on RRM performance requirements of Rel-19 MIMO Phase5

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of performance part of this WI are described in [1] as:

- Specify necessary performance requirements for the enhancements listed above.

From this meeting, RAN4 will start the discussion on RRM performance part of Rel-19 MIMO Phase 5. In this contribution, we provide our view of the RRM performance part of this WI.

- Discussion

For the work plan for RRM performance part, we think the initial version we submitted in [2] can be referred.

According to the Work plan, from this meeting, RAN4 will start the discussion on RRM performance part and the final CRs will be expected to be agreed finally according to the agreements.

In previous meeting, RAN4 discussed and achieved some consensus on RRM core requirements of Rel-19 MIMO.

For each objective, in the discussion for core requirements, RAN4 agreed:

- No RRM impacts by CSI enhancement to support up to 128 ports.

- No RRM impacts by 3 Tx support.

They should cover both core and performance requirements.

Then, the discussion on performance requirements should focus on other 3 objectives including UEIBM, CJTC reporting, enhancement for asymmetric DL sTRP/UL mTRP scenarios.

Observation 1: The performance requirements focus on other 3 objectives including UEIBM, CJTC reporting, enhancement for asymmetric DL sTRP/UL mTRP scenarios.

The performance requirements usually can be divided into two parts: Measurement accuracy and Test cases.

- UE-initiated/event-driven beam management

Measurement accuracy

For measurement accuracy for UEIBM, we have already achieved the agreements:

- The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.

Observation 2: The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.

For the test case of L1-RSRP measurement accuracy, we think it can be combined with the reporting delay together in the same test. No dedicated TC are needed.

Test cases

According to the core requirements, RAN4 to add new core requirements for both SSB and CSI-RS based event triggered L1-RSRP measurement and event triggered reporting delay.

RAN4 to discuss the test coverage of UEIBM, at least the following should be tested:

- sTRP serving cell SSB based L1-RSRP

- sTRP serving cell CSI-RS based L1-RSRP

- sTRP additional PCI cell SSB based L1-RSRP

For the test coverage of different events, prioritize to test event-2. We are open to choose limited TCs for event-1. For event-7, we prefer not to add test case for it.

For the test coverage of modeA and modeB:

RAN1 introduced both ModeA and ModeB as:

Mode A (dynamically scheduling UCI by gNB)

Mode B (UCI in pre-configured resource(s) for second UL channel)

Mode A is for UE request an uplink resource for beam report while mode B uses the pre-configured resource for beam report. In the UE feature list discussion, mode A is the basic UE capability. Mode B depends on UE capability. However, mode B is quite typical used due to it can save the scheduling info. We support to cover both mode A and mode B.

In core requirements discussion, it is still open whether to define such core requirements if eventInstanceCount-r19 > 1. We can wait for decision for the core requirements. But even there is no core requirements if eventInstanceCount-r19 > 1, RAN4 can design TCs to verify it.

Proposal 1: RAN4 to add new test cases to cover:

- sTRP serving cell SSB based L1-RSRP

- sTRP serving cell CSI-RS based L1-RSRP

- sTRP additional PCI cell SSB based L1-RSRP

Prioritized for the tests are for event-2. We are open to choose limited TCs for event-1. No TCs are added for event-7.

TCs are defined to cover both Mode A and Mode B.

FFS on whether to define TC if eventInstanceCount-r19 > 1. We support to define such TC no matter there is core requirements or not.

- CJTC reporting

In last RAN4#114bis meeting, it is agreed in [4]:

Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration? Agreement: - Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified)

Firstly, for the performance part, the report mapping tables are needed to reflect the new measurements defined in RAN1 for CJTC-Dd, CJTC-F, CJTC-P.

Proposal 2: RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P.

When RAN4 introduced the core requirements for delay and frequency offset, UE shall measure and report the measurement during measurement period and pass accuracy test.

Proposal 3: RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F.

For the TCs design for CJTC reporting, RAN4 can add TCs to verify the measurement period and accuracy in the same test.

Proposal 4: RAN4 to add TCs to test measurement period and accuracy in the same test. The TCs covers CJTC-Dd and CJTC-F.

- Asymmetric DL sTRP/UL mTRP scenarios

In this objective, two TAs are enhanced for the UL-TRPs for core requirements. For the TCI state switching, the legacy requirements are reused, no need to add new test cases.

Proposal 5: RAN4 to add TCs for two TA core requirements verification. No need to add TCs for TCI state switching.

- Conclusion

In this contribution, we analyze the potential impacts on RRM performance requirements for Rel-19 MIMO and provide our proposals:

Observation 1: The performance requirements focus on other 3 objectives including UEIBM, CJTC reporting, enhancement for asymmetric DL sTRP/UL mTRP scenarios.

Observation 2: The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.

Proposal 1: RAN4 to add new test cases to cover:

- sTRP serving cell SSB based L1-RSRP

- sTRP serving cell CSI-RS based L1-RSRP

- sTRP additional PCI cell SSB based L1-RSRP

Prioritized for the tests are for event-2. We are open to choose limited TCs for event-1. No TCs are added for event-7.

TCs are defined to cover both Mode A and Mode B.

FFS on whether to define TC if eventInstanceCount-r19 > 1. We support to define such TC no matter there is core requirements or not.

Proposal 2: RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P.

Proposal 3: RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F.

Proposal 4: RAN4 to add TCs to test measurement period and accuracy in the same test. The TCs covers CJTC-Dd and CJTC-F.

Proposal 5: RAN4 to add TCs for two TA core requirements verification. No need to add TCs for TCI state switching.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2412134, Work plan for Rel-19 MIMO Phase 5, Samsung

[3] R4-2504902, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

[4] R4-2504903, WF on RRM requirements for NR_MIMO_Ph5_Part2, MediaTek