3GPP TSG-RAN WG4 Meeting #115		R4-2505818

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.25.4.1

Source:		Samsung

Title:	Views on L1 CLI measurement requirements

Document for:		Discussion

- Introduction

In last RAN4#114bis meetings, the WF was approved in [1]. In this contribution, we continue to discuss the open issues of RRM requirements for L1 CLI measurement.

- Discussion

In last RAN4#114bis meeting, the potential issues for further discussion are captured in WF [1] as below:

Potential issues for further discussion Topic 1: UE-to-UE CLI handling Sub-topic 1-1: General issues Issue 1-1-5: Requirement applicability Sub-topic 1-2: L1-SRS-RSRP Issue 1-2-4: Side condition Issue 1-2-5: Measurement accuracy Sub-topic 1-3: L1-CLI-RSSI Issue 1-3-4: Side conditions Issue 1-3-5: Measurement accuracy Issue 1-3-6: SCS for performing measurement

Measurement period for L1-SRS-RSRP

In RAN4#114bis, RAN4 achieved the agreement as below if RAN1 has the similar NW configuration as L1-RSRP, and sent LS to RAN1.

- Agreement - The number of samples (M) for measurement period: - For periodic and semi-persistent SRS resource, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise - For aperiodic SRS resources M=1

In last RAN1#120bis meeting, RAN1 approve the reply LS in [2], and confirm that there will be the same NW configuration.

Question 1: RAN4 respectfully asks RAN1 to clarify whether the NW configuration of timeRestrictionForChannelMeasurement or a similar configuration will apply for L1-SRS-RSRP and/or L1-CLI-RSSI measurement. Answer: RAN1 would like to inform RAN4 that the NW configuration of timeRestrictionForChannelMeasurement will apply for L1-SRS-RSRP and L1-CLI-RSSI measurement.

Proposal 1: For the measurement period of L1-SRS-RSRP, RAN4 to agree below according to RAN1 reply LS:

  - The number of samples (M) for measurement period:

    - For periodic and semi-persistent SRS resource, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

    - For aperiodic SRS resources M=1

Scheduling Restriction for L1-SRS-RSRP

In last RAN4#114bis meeting, the agreement in [1] is listed as:

- Agreement (online session on Monday with update) - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for SRS resource and Y symbols before, where Y is re-using same number as R16 requirements. - For the symbols after the SRS resource, not define scheduling restriction on symbols after symbols for SRS resource in TS 38.133, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211. - For DL, - if UE is able to support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies. - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements. - FFS whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

In last RAN1#120bis meeting, RAN1 approve the reply LS in [2]:

Question 2: RAN4 kindly asks whether RAN1 will define optional UE capability for FDM-ed DL reception and SRS measurement. Answer: RAN1 would like to inform RAN4 that a new optional UE capability will be defined for FDM-ed DL reception and SRS measurement.

There is still FFS on the issue of whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

According to RAN1 reply LS, the UE will support FDM-ed DL reception and SRS measurement by the new UE capability. No need to add restriction after SRS Resouce.

Proposal 2: For L1-SRS-RSRP measurement, For the UE support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies.

- The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements.

- Not define scheduling restriction on symbols after symbols for SRS resource

Measurement Restriction for L1-SRS-RSRP

In last RAN4#114bis meeting, the agreement in [1] is listed as:

- Agreements - Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options - Option 1: Prioritize L1-SRS-RSRP measurement - Option 2: Prioritize L1-CLI-RSSI measurement - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI

When the measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, because the reference timing of the different measurement are different. The UE cannot measure them both at the same time. It can be avoided by UE cannot be configured for two types of measurement at the same time. But this is too restricted for NW implementation. In existing RRM requirement for other measurement, UE can be configured but if UE cannot perform the different measurements at the same time, UE is required to measure one of but not both of them. The principle can be used here.

Proposal 3: For Measurement Restriction for L1-SRS-RSRP, UE is required to measure one of but not both of different measurements when measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping.

Measurement period for L1-CLI-RSSI

In RAN4#114bis, RAN4 achieved the agreement as below:

- Agreements (online session on Monday) - For DUD case with method #1 and resource type#2 - In RAN4 understanding, if UE can support simultaneous PDSCH reception on the two DL subbnads for DUD, it can support simultaneous RSSI measurement on the two DL subbands based on 1 sample. - RAN4 to decide the number of samples for measurement period based on RAN1 progress on UE capability. - For method #3, the measurement period is based on 1 sample.

RAN1 is still in the discussion of UE feature list and UE capability as in R1-2503593:

| 60-2 | [Transmission and] reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG |  | YES | n/a | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

No conclusion yet. By our understanding and internal check, we think RAN1 will specify such UE capability in UE feature list in RAN1. Therefore, the context in first bullet still valid from RAN4 understanding. Let’s check the final consensus in RAN1.

Scheduling Restriction for L1-CLI-RSSI

In RAN4#114bis, RAN4 achieved the agreement as below:

- Agreements (adhoc session on Tuesday with addition) - Method #1: - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for RSSI resource. - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource - Method #3: - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for RSSI resource. - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource. - Note: the common understanding in RAN4 if the RSSI in UL suband is too strong compare to the wanted signals in DL suband(s), UE may fail to decode the PDCCH or PDSCH. - FFS whether restricted symbol(s) is needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband. - Generic assumption for defining RRM requirements, not intend to add it in the core requirement. - For all Method #1, Method #2 and Method #3, - Note: The RF BW is wide enough to cover DL active BWP, including PRBs from both DL subband(s) and UL subband that fall within the DL active BWP.

There is still FFS for method #3 for whether restricted symbol(s) is needed in highlighted above. As we mentioned in last meeting, the method #3 are using the same timing as in method #1. We think it should follow the same principle as Method #1 which has been agreed in last meeting.

Proposal 4: For scheduling restriction of Method #3 of L1-CLI-RSSI, follow the agreement as Method#1:

    - For DL,

      - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource

      - Restricted symbol(s) is not needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband

Measurement Restriction for L1-CLI-RSSI

In RAN4#114bis, RAN4 achieved the agreement as below:

#### Issue 1-3-3: Measurement restriction - Agreements (adhoc session on Tuesday) - Collision between L1-CLI-RSSI measurements with different methods - Capture in RAN4 specs that UE is not expected to perform CLI measurement using Method #1 and Methods#3 in the same OFDM symbol. - Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements - For Method #1& Method 3: - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1.

There is FFS for FR1 as above.

As we discussed in last meeting, for FR2, because UE cannot receive, the measurement restriction is needed. For FR1, UE can receive the signals among all REs at the same time especially when SCS are the same for L1 measurement.

Proposal 5: For measurement Restriction for L1-CLI-RSSI on FR1, if the measurement resource collides with legacy L1 measurement including RLM, BFD, CBD or L1-RSRP, if SCS are the same, the measurement can be performed without restriction.

L1-CLI-RSSI SCS for performing measurement

Issue 1-3-6: SCS for performing measurement

In Rel-16 L3 CLI,

RAN2 spec: RSSI-ResourceConfigCLI-r16 ::= SEQUENCE { rssi-ResourceId-r16 RSSI-ResourceId-r16, rssi-SCS-r16 SubcarrierSpacing, RAN4 spec: When configured by the network, the UE shall be able to perform CLI-RSSI measurement of configured rssi-ResourceConfigCLI. The subcarrier spacing for CLI-RSSI measurement resource configuration can be same or different from the subcarrier spacing of active BWP.

However, in Rel-19, the SCS of L1-CLI-RSSI measurement resource are from the BWP:

RAN1#120 Agreement - UE determines the subcarrier spacing of the CLI-RSSI measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - UE determines the subcarrier spacing of the SRS-RSRP measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - Note: No RAN1 spec impact is expected.

Proposal 6: The SCS of L1-CLI-RSSI are the same as the SCS of the DL BWP configured by bwp-Id. The SCS can be different from the SCS of SSB.

- Conclusion

In this contribution, we provide our proposals for L1 CLI measurement:

Proposal 1: For the measurement period of L1-SRS-RSRP, RAN4 to agree below according to RAN1 reply LS:

  - The number of samples (M) for measurement period:

    - For periodic and semi-persistent SRS resource, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

    - For aperiodic SRS resources M=1

Proposal 2: For L1-SRS-RSRP measurement, for the UE support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies.

- The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements.

- Not define scheduling restriction on symbols after symbols for SRS resource

Proposal 3: For Measurement Restriction for L1-SRS-RSRP, UE is required to measure one of but not both of different measurements when measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping.

Proposal 4: For scheduling restriction of Method #3 of L1-CLI-RSSI, follow the agreement as Method#1:

    - For DL,

      - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource

      - Restricted symbol(s) is not needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband

Proposal 5: For measurement Restriction for L1-CLI-RSSI on FR1, if the measurement resource collides with legacy L1 measurement including RLM, BFD, CBD or L1-RSRP, if SCS are the same, the measurement can be performed without restriction.

Proposal 6: The SCS of L1-CLI-RSSI are the same as the SCS of the DL BWP configured by bwp-Id. The SCS can be different from the SCS of SSB.

- Reference

[1] R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei

[2] R4-2505307, Reply LS on L1 measurement, RAN1