3GPP TSG-RAN WG4 Meeting #115		R4-2505822

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.10.4.5

Source:		Samsung

Title:	Discussion on RRM performance requirements of Rel-19 NR-NTN in less than 5MHz

Document for:		Discussion

- Introduction

In last RAN4#114bis meeting, RAN4 discussed the RRM performance part of less than 5MHz for NTN initially. The latest agreements and open issues have been included in [1].

In this contribution, we continue the discussion on the open issues in [1] and provide our view of the RRM performance part of less than 5MHz for NTN.

- Discussion

Test cases

In [1], for this WI, for the procedure without new core requirements, RAN4 agree not to define the new test cases with only cell re-selection for FFS as below:

### Issue 2-2-2: Test cases to verify the core requirements reusing NTN Agreement: - Not define the new test cases for the following requirements RRM requirements; NTN requirements in TS38.133 v18.5.0. RRC connection mobility control; 6.2C RRC Connection Mobility Control for Satellite Access. UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy; 7.1C, 7.2C, 7.3C. - Further discuss whether to define the new test case for the following requirement IDLE/inactive mode mobility; 4.2C Cell Re-selection for NR UE for Satellite Access.

For cell Re-selection requirements, as we mentioned in last meeting, although there are no new core requirements. However, the new test case is needed.

In the core requirements, RAN4 to decide no extension for Tdetect in core requirements. However, UE still need to read MIB which will need more samples due to punctured PBCH.

That is the same reason for extension the Tdetect and TSI-NR in TN less than 5MHz system as below:

Tdetect, NR_Intra + 40 ms See table 4.2.2.3-1 in clause 4.2.2.3 Tevaluate, NR_ intra See table 4.2.2.3-1 in clause 4.2.2.3 TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case. This gives a total of 33.36 s, allow 34 s for the cell re-selection delay to a newly detectable cell and 7.76 s for the cell re-selection delay to an already detected cell in the test case, which we allow 8 s.

Proposal 1: Define new test case for cell reselection for less than 5MHz NTN for extension both Tdetect and TSI-NR in the test.

For the procedure with new core requirements for satellite access, RAN4 agree to define the new test cases as below:

### Issue 2-2-3 Test cases to verify the new core requirements Agreement: - Define the new test cases for the core requirements for NTN less than 5MHz introduced in Rel19.e.g. - HO-1: Intra-frequency HO, [unknown cell] - CHO-1: [Inter-frequency CHO], time based with L3 measurement - SatSwitch-1: Hard switch - RLM-1: RLM OOS for 15RB - RLM-2: RLM IS for 15RB - BFR-1: BFR - MR-1: Intra-frequency with SSB index reading - [MR-2: Inter-frequency without SSB index reading] - FFS the detailed configuration for each test case

For the HO test, according to the core requirements, the extension of T∆ are for both known cell and unknown cell.

- T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

In legacy NTN test, the Intra-frequency HO with only known cell has been verified. We are fine to a different function: unknown cell. Use T1 and T2 for cell 1 and cell 2 for configuration.

Proposal 2: For HO intra-frequency HO, design the TC for unknown cell. In the test config, two successive time periods T1 and T2 are used. At the start of T1, UE does not have the timing information for unknown cell.

For the CHO test, according to the core requirements, it covers both intra-frequency and inter-frequency measurement. In addition, the extension of T∆ are the same. To minimize the test effort, the group don’t want to test every configuration. If only one test is chosen, we slightly prefer the intra-frequency CHO because we think it is more typical deployments in NTN deployment network.

Proposal 3: For CHO test, the extension of T∆ are the same for intra-f/inter-frequency. For the test, we suggest to define the TC for intra-frequency CHO.

For the L3 measurement, it is still in bracket of MR-2: Inter-frequency without SSB index reading.

If based on without SSB index reading, we don’t see the necessity of this test. If the inter-frequency test is considered, we also suggest to define the TC with SSB index reading.

Proposal 4: For inter-frequency test, RAN4 to define the TC with SSB index reading.

- Conclusion

In this contribution, we continue to discuss the RRM performance requirements for less than 5MHz for NR-NTN and provide our proposals:

Proposal 1: Define new test case for cell reselection for less than 5MHz NTN for extension both Tdetect and TSI-NR in the test.

Proposal 2: For HO intra-frequency HO, design the TC for unknown cell. In the test config, two successive time periods T1 and T2 are used. At the start of T1, UE does not have the timing information for unknown cell.

Proposal 3: For CHO test, the extension of T∆ are the same for intra-f/inter-frequency. For the test, we suggest to define the TC for intra-frequency CHO.

Proposal 4: For inter-frequency test, RAN4 to define the TC with SSB index reading.

- Reference

[1] R4-2504896, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Xiaomi