3GPP TSG-RAN WG4 Meeting #115								    R4-2505835

Malta, MT, 19th – 23rd May, 2025

Agenda Item: 5.27.1

Source: MediaTek Inc.

Title: Discussion on remaining issue of Tx switching for 3Tx UEs in Rel-19 TEI

Document for: Discussion

# 1	Introduction

In RAN4#114b meeting, a WF [1] was agreed on further enhancing Tx switching in TEI-19, and an LS [2] was sent to RAN1. In the WF [1], there are 3 scenarios listed.

Issue 1: Introduction of Tx switching enhancements in TEL-19. Background The following scenarios of further Tx switching enhancement for 2 configured UL bands were discussed in RAN4#114b meeting: - Scenario 1. Switching between the following 2 cases for 3Tx UEs configured with uplinkTxSwitchingOption set to 'dualUL’ - Case 1-1: Two antenna ports on Band A and one antenna port on Band B - Case 1-2: One antenna port on Band A and two antenna ports on Band B - Scenario 1a. Switching between the following 2 cases for 3Tx UEs configured with uplinkTxSwitchingOption set to ‘switchedUL’ - Case 2-1: Three antenna ports on Band A and zero antenna port on Band B - Case 2-2: Zero antenna port on Band A and three antenna ports on Band B - Scenario 2. Switching between the following 2 cases for 4Tx UEs configured with uplinkTxSwitchingOption set to ‘switchedUL’ - Case 3-1: Four antenna ports on Band A and zero antenna port on Band B - Case 3-2: Zero antenna port on Band A and Four antenna ports on Band B Agreement: - Support scenario 1 in Rel-19 TEI - Focus on scenario 1 in RAN4 #115 meeting: - Inform RAN1 that RAN4 is considering whether to support scenario 1a and scenario 2 in Rel-19 TEI and check the possibility to support scenario 1a/2 from RAN1 perspective in addition to scenario 1. - FFS whether need to further discuss scenario 1a and scenario 2 in RAN4 which is pending on RAN1 decision on supporting above scenarios in May meeting.

RAN4 agreed to focus on Scenario 1 in RAN4#115 meeting, while leaving other scenarios to RAN1 to decide whether to be introduced in TEI-19. Therefore, in this paper, we focus on the open issues of scenario 1 in the WF [1].

# 2	Open issues

In WF [1], 3 open issues are captured. In this section we discuss this 1-by-1.

Issue 2: Applicable duplexing mode for the 2 configured UL bands (focus on scenario 1 in RAN4#115 meeting) Wayforward: - FFS any down selection among {FDD+TDD, FDD+FDD, TDD+TDD} is needed. - Companies are encouraged to provide the justifications, such as commercial demand or problem identified in scheduling.

In our understanding, there are potential interests on FDD+FDD (e.g., CA_n25A-n71A, CA_n66A-n71A), FDD+TDD (e.g., CA_n25A-n41A, CA_n66_n77) and TDD+TDD (e.g., CA_n41A-n77A). Therefore, we suggest not precluding any cases in the scope.

Proposal 1: For scenario 1, all cases (FDD+FDD, FDD+TDD, TDD+TDD) are considered in the scope.

Issue 3: Switch period (focus on scenario 1 in RAN4#115 meeting) Wayforward: - FFS whether and which existing values can be re-used, or any new values need be introduced. - The final conclusions will be captured in the Rel-19 RAN4 feature list to RAN1 and RAN2

For 3Tx UE, Tx switching between Case 1-1 and Case 1-2 (as defined in WF[1]) involves only one Tx chain per band. Therefore, the set {n35us, n140us, n210us} in current UE capability (e.g., UplinkTxSwitch in Rel-17) is sufficient. How to indicate the UE capability is related to the next issue, i.e., whether we agree to reuse existing UE capability or introduce new UE capability for band combo report for 3Tx switching case.

Proposal 2: For scenario 1, the existing switch periods {n35us, n140us, n210us} can be reused.

Issue 4: UE band combo report (focus on scenario 1 in RAN4#115 meeting) Wayforward: - FFS whether and which existing UE capabilities can be re-used, or any new UE capabilities need be introduced. - The final conclusions will be captured in the Rel-19 feature list to RAN1 and RAN2

As we discussed in [3], we can take Rel-17 UE capability as a starting point, as copied and pasted below.

BandCombination-UplinkTxSwitch-v1720 ::= SEQUENCE { bandCombination-v1720 BandCombination-v1720 OPTIONAL, uplinkTxSwitching-OptionSupport2T2T-r17 ENUMERATED {switchedUL, dualUL, both} OPTIONAL } ULTxSwitchingBandPair-v1700 ::= SEQUENCE { uplinkTxSwitchingPeriod2T2T-r17 ENUMERATED {n35us, n140us, n210us} OPTIONAL } UplinkTxSwitchingBandParameters-v1700 ::= SEQUENCE { bandIndex-r17 INTEGER(1..maxSimultaneousBands), uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 ENUMERATED {nonCoherent, fullCoherent} OPTIONAL }

Firstly, although it seems feasible to directly re-use Rel-17 UE capability, to avoid confusion it is still beneficial to introduce separate UE capabilities for Rel-19. Therefore, after some more internal discussions, we decided to suggest RAN4 introducing separate Rel-19 UE capabilities rather than reusing Rel-17 ones.

Proposal 3: For scenario 1, introduce new Rel-19 UE capabilities for Tx switching within 2 configured UL bands for 3Tx UEs.

On the switching option (i.e., switchedUL or dualUL), it is already clearly agreed in [1] and [2] that only dualUL is supported. Therefore, the corresponding UE capability becomes redundant and does not need to be reported to network.

Proposal 4: For scenario 1, the UE capability report for switching option (i.e., switchedUL or dualUL) is redundant and does not need to be reported.

Apart from the switching option, all the other UE capabilities can be kept, e.g., for switching period and for coherent UL. Possible new UE capabilites could be like below which is modified from Rel-17 with changes highlighted. Note that the final decision should be left to RAN2 to decide.

BandCombination-UplinkTxSwitch-v19xx ::= SEQUENCE { bandCombination-v19xx BandCombination-v19xx OPTIONAL, uplinkTxSwitching-OptionSupport2T2T-r17 ENUMERATED {switchedUL, dualUL, both} OPTIONAL } ULTxSwitchingBandPair-v19xx ::= SEQUENCE { Uplink3TxSwitchingPeriod2TPerBand2T-r19 ENUMERATED {n35us, n140us, n210us} OPTIONAL } UplinkTxSwitchingBandParameters-v19xx ::= SEQUENCE { bandIndex-r19 INTEGER(1..maxSimultaneousBands), uplinkTxSwitching2T2T-PUSCH-TransCoherence-r19 ENUMERATED {nonCoherent, fullCoherent} OPTIONAL }

Regarding the UE feature list, we provided a draft version which was revised from Rel-17 feature list [4]. In our understanding, this is the approach which can minimize the required discussions in both RAN4 and RAN2, because the Rel-17 conclusions can be re-used as much as possible.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD diff. | Need of FR1/FR2 diff. | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59-1 | Switch period for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | Indicate the supported switching period for dynamic UL Tx switching between two bands for 3Tx UE with up to two transmit antenna connectors in each band |  | Yes | N/A | UE does not support dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | UE signals supported switching period per pair of UL bands per UL band combination | No need | FR1 only | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} UE is mandatory to support dualUL when supporting this feature | Optional with capability signalling |
| 59-2 | Application of DL interruptions due to dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | Capability to indicate that for the band where DL interruption is needed due to dynamic UL Tx switching between 2 band for 3Tx UE with up to 2Tx per band | 59-1 | Yes | N/A | UE not reporting this capability means DL interruption is not required | UE capability is defined as per band per band combination for each band pair supporting UL Tx switching | No need | FR1 only | Support mixture of FDD/TDD | The same capability for Rel-16 DL interruption due to Tx switching is reused. | Optional with capability signalling |
| 59-3 | UL-MIMO coherence capability for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | Capability to indicate whether UL-MIMO coherence is supported when dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band is conducted. | 59-1 | Yes | N/A | Rel-15 per band capability pusch-TransCoherence is applicable | Per band per BC | No need | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |

Proposal 5: For scenario 1, RAN4 to introduce the UE features 59-1, 59-2 and 59-3 for the switching period, DL interruption and UL-MIMO coherence for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band, respectively.

# 3	Summary

In this paper, we discussed the open issues based on the previous agreed WF for scenario 1 for further Tx switching enhancement in Rel-19 TEI. We have the following proposals.

Proposal 1: For scenario 1, all cases (FDD+FDD, FDD+TDD, TDD+TDD) are considered in the scope.

Proposal 2: For scenario 1, the existing switch periods {n35us, n140us, n210us} can be reused.

Proposal 3: For scenario 1, introduce new Rel-19 UE capabilities for Tx switching within 2 configured UL bands for 3Tx UEs.

Proposal 4: For scenario 1, the UE capability report for switching option (i.e., switchedUL or dualUL) is redundant and does not need to be reported.

Proposal 5: For scenario 1, RAN4 to introduce the UE features 59-1, 59-2 and 59-3 for the switching period, DL interruption and UL-MIMO coherence for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band, respectively.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD diff. | Need of FR1/FR2 diff. | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59-1 | Switch period for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | Indicate the supported switching period for dynamic UL Tx switching between two bands for 3Tx UE with up to two transmit antenna connectors in each band |  | Yes | N/A | UE does not support dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | UE signals supported switching period per pair of UL bands per UL band combination | No need | FR1 only | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} UE is mandatory to support dualUL when supporting this feature | Optional with capability signalling |
| 59-2 | Application of DL interruptions due to dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | Capability to indicate that for the band where DL interruption is needed due to dynamic UL Tx switching between 2 band for 3Tx UE with up to 2Tx per band | 59-1 | Yes | N/A | UE not reporting this capability means DL interruption is not required | UE capability is defined as per band per band combination for each band pair supporting UL Tx switching | No need | FR1 only | Support mixture of FDD/TDD | The same capability for Rel-16 DL interruption due to Tx switching is reused. | Optional with capability signalling |
| 59-3 | UL-MIMO coherence capability for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band | Capability to indicate whether UL-MIMO coherence is supported when dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band is conducted. | 59-1 | Yes | N/A | Rel-15 per band capability pusch-TransCoherence is applicable | Per band per BC | No need | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |

# 4	References

- [1] R4-2505220, WF on Tx switching for TEI19, MediaTek inc.

- [2] R4-2505221, LS on UL Tx switching for TEI19, RAN4

- [3] R4-2504078, Introducing Tx switching for 3Tx UEs in Rel-19 TEI, MediaTek Inc., Orange, T-Mobile USA, Vodafone, Verizon

- [4] R4-2211189, Rel-17 RAN4 UE feature list for NR, CMCC

- [5] R4-2505836, CR 38.101-1 on time mask of Tx switching for 3Tx UEs [TxSwich_R19], MediaTek inc.

- [6] R4-2505837, CR 38.133 on interruption of Tx switching for 3Tx UEs [TxSwich_R19], MediaTek inc.