3GPP TSG-RAN WG4 Meeting #115							         		     R4-2505841

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.7.3	UE RF requirements and UE capability [NR_LBCA_Sw-Core]

Title: 	On CA_n5-n8 switching between 1UL2DL and 1DL2UL

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Background

In R4#114b, a number of companies provided input [1-7] for the answer to the RAN2 LS on the specific CA_n5-n8 2UL case where DL is only feasible in band n5. The contribution [6], rightly pointed at a RAN4 issue due to the switching between n5+n8DL/n5UL and n5DL/n5+n8UL operations. In this contribution, we provide our position on this switching aspect.

# Discussion

## 2.1 Switching for CA_n5-n8 2UL

In release 18, RAN4 introduced CA_n5-n8 1UL with a specific restriction to UL in band n5 only due to band n5 DL overlap with n8 UL.

In release 19, RAN4 introduced CA_n5-n8 2UL with a specific restriction to DL in band n5 only due to band n5 DL overlap with n8 UL. This being the first case where less bands are supported in DL than UL, RAN4 consulted with RAN2 on whether the current signaling could support such an operation.

RAN4 also agreed that CA_n5-n8 2DL/2UL with a specific narrower filter should not be specified and that such operation would require the definition of new bands.

However, RAN4 omitted to clarify the necessary switching between the R18 2DL1UL mode and the R19 1DL/2UL mode as pointed in [7].

We believe that this CA_n5-n8 issue is very similar to the switching between an FDD band and an SDL band covered in the Release 19 LBLB switching WI and should ideally be covered under this WI rather than in basket block approval.

One aspect is that unlike the LBLB switching that may require LO retuning, in CA_n5-n5 2UL the UE must have two full receivers and two full transmitters as both 2DL and 2UL are supported and thus it is likely to have enough PLLs to avoid LO retuning interruption and only require interruption for PA ON/OFF and switching.

Proposal:

- CA_n5-n8 2DL/2UL based on specific filters is not in scope for R19 and should be based on new band definitions if introduced in later releases.

- CA_n5-n8 n5DL/2UL to 2DL/n5UL switching period should be specified and discussed with the LBLB switching WI.

- At minimum, ON/OFF PA switching should be specified and use the same interruption time as LBLB switching

- FFS if LO retuning interruption time is needed for CA_n5-n8.

# Conclusion

In this contribution, we have discussed the switching aspect related to the 2DL/n5UL to n5DL/2UL CA_n5-n8 operation and have the following proposal.

Proposal:

- CA_n5-n8 2DL/2UL based on specific filters is not in scope for R19 and should be based on new band definitions if introduced in later releases.

- CA_n5-n8 n5DL/2UL to 2DL/n5UL switching period should be specified and discussed with the LBLB switching WI.

- At minimum, ON/OFF PA switching should be specified and use the same interruption time as LBLB switching

- FFS if LO retuning interruption time is needed for CA_n5-n8.

# References

[1] R4-2504124 On LS response to RAN1 on UE capability for non-simultaneous RxTx FDD-FDD inter-band CA, Nokia, R4#114b

[2] R4-2503339 Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, MediaTek Korea Inc., R4#114b

[3] R4-2503482 draft reply LS to RAN2 on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, CATT, R4#114b

[4] R4-2503692 Discussion and replied LS on UL CA_n5-n8, Huawei, HiSilicon, R4#114b

[5] R4-2503973 Reply LS on UE capability for FDD-FDD inter-band CA simultaneous RxTx, ZTE Corporation, Sanechips, R4#114b

[6] R4-2504116 CA_n5A-n8A capabilities, Qualcomm France, R4#114b

[7] R4-2504117 Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, Qualcomm France, R4#114b