3GPP TSG-RAN4 Meeting #115	R4-2505846

# Malta, Malta, 19th - 23rd May, 2025

Agenda item:		7.14.3.3

Source:				Keysight Technologies UK Ltd

Title:					TP to TR 38.762: on performance metrics

Document for:		Approval

# Introduction

This contribution is incorporating discussions/proposals from [1] to update [2].

# Discussion

Proposal 1: approve the TP to correct TMT definition and to define a single performance metric

# References

- R4-2505844, On TMT and CTMT metric definitions, Keysight Technologies, 3GPP TSG-RAN4 Meeting #115, May 2025

- TR 38.762, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) dynamic test methodology for FR1 UEs

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

# 5	Performance metrics

Let the CDF of the Total Measured Throughput (CTMT) be the figure of merit for the MIMO OTA tests with link adaptation and dynamic channel models. The TP shall be recorded every 200 ms during the emulation of the route separately for each of the M DUT orientations and for each of the L rotations per orientation, defined in Clause B.2. The Total Measured Throughput (TMT) is defined as the combination/concatenation of all measured TP values for each channel model, i.e.,

TMTUMi=TP1,1(t1),..., TP1,1(tN), ...,TPm,l(t1),..., TPm,l(tN),..., TPM,L(t1),..., TPM,L(tN)UMiTMTUMi=TP1(t1),..., TP1(tN), ...,TPm(t1),..., TPm(tN),..., TPM(t1),..., TPM(tN)UMi

and

TMTUMa=TP1,1(t1),..., TP1,1(tN), ...,TPm,l(t1),..., TPm,l(tN),..., TPM,L(t1),..., TPM,L(tN)UMa

TMTUMa=TP1(t1),..., TP1(tN), ...,TPm(t1),..., TPm(tN),..., TPM(t1),..., TPM(tN)UMa

with m = 1 to M=4 and l = 1 to L=4.

Subsequently, the TMTs from both channel models are combined into a single one

TMTUMi&UMa=TMTUMi, TMTUMa

Three separate CTMT values shall be determined from the TMT values, i.e., the 10%, 50%, and 90%-ile CDF

-	CTMTUMi&UMa,10%=CDF10%TMTUMi&UMa

-	CTMTUMi&UMa,50%=CDF50%TMTUMi&UMa

-	CTMTUMi&UMa,90%=CDF90%(TMTUMi&UMa)

CTMTUMa,10%=CDF10%TMTUMa

CTMTUMa,50%=CDF50%TMTUMa

CTMTUMa,90%=CDF90%(TMTUMa)

The final CTMT metric is the weighted average of the three CTMTs, i.e.,

CTMTUMi&UMa=w10CTMTUMi&UMa,10%+w50CTMTUMi&UMa,50%+w90CTMTUMi&UMa,90%3

with the weights w10, w50, w90 FFS.

<<< END OF CHANGES >>>