3GPP TSG-RAN4 Meeting #115	R4-2505849

# Malta, Malta, 19th - 23rd May, 2025

Agenda item:		7.14.3.2

Source:				Keysight Technologies UK Ltd

Title:					On Partial Radiated Surfaces

Document for:		Approval

# Introduction

This contribution is reviewing partial radiated surfaces for Scenario 2 IoT-NTN and NR-NTN handheld UEs.

# Discussion

Partial radiated metrics have been introduced in CTIA previously, i.e., near-horizon partial radiated power (NHPRP), near-horizon partial isotropic sensitivity (NHPIS), upper hemipshere isotropic sensitivity (UHIS), or partial isotropic GNSS sensitivity (PIGS) defined in Section 3 of [1] and visualized in Figure 1. In these figures, the range of partial integration is shown in yellow, the grid points within that range are shown in green and those outside the range in red. The TX-based grids are shown with a grid point spacing of Dq=Df=15° while the RX-based grids are shown with a grid point spacing of Dq=Df=30°.

Figure 1: Visualization of CTIA’s integration surfaces for partial radiated metrics: from top left to bottom right: NHPRP45, NHPIS45, UHIS, PIGS.

Partial sphere metrics are currently being considered for IoT-NTN and NT-NTN devices, specifically Scenario 2 handheld UEs, e.g., as summarized in the RAN4#114-bis WF [2]

### Sub-topic 2-1 UE type, usage scenarios and performance metric for NTN (NR-NTN and IoT-NTN) Issue 2-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2 Agreements: o For IoT-NTN and NR-NTN, typical usage scenarios are: § Scenario 1: both head+hand mode and hand only mode. § Scenario 2: hand only mode targeted for specific UE orientation usage § Scenario 3: hand only mode targeted for arbitrary UE orientation usage o UE shall declare support for only one scenario per band. The declaration details of each scenario should be further discussed. o For IoT-NTN and NR-NTN handheld UEs declared Scenario 2, the performance metrics are partial sphere metric. FFS details o For IoT-NTN and NR-NTN handheld UEs declared Scenario 3, the performance metrics are full sphere TRP and TRS

or [3], [4].

In [3], specifics about the partial spherical aspects were proposed, i.e., to allow rather arbitrary partial surfaces.

Proposal 3: For handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining an OEM declaration of the initial DUT position and the range of elevation and azimuth angles relative to the initial position over which the performance metric is calculated. Angular granularity of this declaration can be further discussed.

Various examples of such arbitrary partial surfaces are visualized in Figure 2 where a partial surface around a declared direction (yellow arrow) with full (left two images) f ranges or partial f range (right) is visualized with the yellow cone as the boundary of the cone with fixed ±45° angular width and the green circles highlighting the grid points within the partial surface.

Figure 2: Visualization of various arbitrary partial surfaces.

The concerns with this type of approach are two-fold. The integration of EIRPs/EISs of custom (declared) surfaces to partial radiated powers/sensitivities can become very complex and could likely require validations/alignments between different vendors, especially considering the various quadratures, e.g., Clenshaw-Curtis and sin(q) considered in 3GPP. On the other hand, allowing completely arbitrary partial surfaces, e.g., with different q and f ranges, would likely require different performance metrics which typically has been undesirable in 3GPP for OTA requirements.

Observation 1: Arbitrary partial surfaces could be complex for system vendors to integrate and would require different performance requirements

It is therefore proposed to define just one partial surface/metric for TX and RX testing. Ideally, these surfaces should be centered around q=0°, i.e., the top pole, to reduce complexity. Additionally, the angular width of the cone should be a multiple of the classical grid step size Dq=Df of the coarsest grid step size of the classical grids, i.e., Dq=Df=30°. Two suitable choices are presented in Figure 3 for a partial surface with angular width of ±60° and in Figure 4 for a partial surface with angular width of ±90°.

Figure 3: Visualization of partial surface with ±60° angular width for classical TRP grids (top) and TRS grids (bottom).

Figure 4: Visualization of partial surface with ±90° angular width for classical TRP grids (top) and TRS grids (bottom).

Proposal 1: For scenario 2 IoT-NTN and NR-NTN handheld UEs, define just one partial surface for TX and RX testing, centered around q=0° (top pole) and with an angular width of either ±60° or ±90°.

The choice of postprocessing of EIRPs/EISs within the partial surface, e.g., CDF with target %-ile, surface integration, and/or peak EIRP/EIS, is up to OEMs/chipset vendors.

Observation 2: The choice of postprocessing of EIRPs/EISs within the partial surface, e.g., CDF with target %-ile, surface integration, and/or peak EIRP/EIS, is up to OEMs/chipset vendors

Given the desire to assess the directive nature of NTN OTA performance over a limited sector, it is not clear whether the NTN OTA test cases need to take into account some sort of beam steering or active antenna system with the ability to change antenna patterns. If so, the test procedures might have to be adjusted to take a dwell time into account.

Proposal 2: OEMs/Chipset vendors to provide feedback whether NTN OTA test cases should take additional dwell times into account to accommodate active antenna systems

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: Arbitrary partial surfaces could be complex for system vendors to integrate and would require different performance requirements

Observation 2: The choice of postprocessing of EIRPs/EISs within the partial surface, e.g., CDF with target %-ile, surface integration, and/or peak EIRP/EIS, is up to OEMs/chipset vendors

Proposal 1: For scenario 2 IoT-NTN and NR-NTN handheld UEs, define just one partial surface for TX and RX testing, centered around q=0° (top pole) and with an angular width of either ±60° or ±90°.

Proposal 2: OEMs/Chipset vendors to provide feedback whether NTN OTA test cases should take additional dwell times into account to accommodate active antenna systems

# References

- CTIA 01.90, Test Plan for Wireless Device Over-the-Air Performance, Informative Reference Material

- R4-2504763, WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3, vivo, 3GPP TSG-RAN WG4 Meeting #114bis, April 2025

- R4-2504578, On NTN TRP TRS test methodology, Apple, 3GPP RAN4 Meeting #114bis, April 2025

- R4-2504728, Draft CR to 38.870 on NTN OTA performance metric and test method, Vivo, Samsung, Apple, 3GPP TSG-RAN4 Meeting #114bis, April 2025

- OTA191205, Use of Clenshaw-Curtis Quadrature for Partial Integrals, Keysight Technologies
This is a CTIA Certification Program Working Group Document available to CTIA Certification Program Working Group members

# Annex: Mathematical Expression of Metrics

### TX Metrics:

The upper-hemisphere partial radiated power metric, see Figure 4, can be expressed mathematically as

UHPRP≅12a6cutTX,6+i=05cutTX,iwhere

cutTX,i=1Mj=0M-1EiRPθθi,ϕj+EiRPϕθi,ϕj⋅Wθi

with

a6=0.5

for both Clenshaw-Curtis or sin(q) quadratures. The TX cuts, cutTX,i, are further visualized in Figure 5.

The partial radiated power metric with EIRP integrated from q=0° to q=60° in the upper hemisphere, see Figure 3, can be expressed mathematically as

UPRP±60≅12a4cutTX,4+i=03cutTX,i

with

a4=0.5

for the Clenshaw-Curtis quadrature or

a4=θ=52.5∘θ=60∘Wθdθθ=52.5∘θ=67.5∘Wθdθ=θ=52.5∘θ=60∘sinθ∆θdθθ=52.5∘θ=67.5∘sinθ∆θdθ=cosθ|θ=52.5∘θ=60∘cosθ|θ=52.5∘θ=67.5∘≈0.48108

for the sin(q) quadrature [5].

Figure 5: Visualization of TX cuts, cutTX,i.

### RX Metrics:

The upper-hemisphere partial radiated isotropic sensitivity metric, see Figure 4, can be expressed mathematically as

UHPIS=21cutRX,0+cutRX,1+cutRX,2+b3cutRX,3where

cutRX,i=1Mj=0M-11EISθ(θi,ϕj)+1EISϕ(θi,ϕj)⋅W(θi)

with

b3=0.5

for both Clenshaw-Curtis or sin(q) quadratures. The RX cuts, cutRX,i, are further visualized in Figure 6.

The partial radiated isotropic sensitivity metric with EIS integrated from q=0° to q=60° in the upper hemisphere, see Figure 3, can be expressed mathematically as

UPIS±60=21cutRX,0+cutRX,1+b2cutRX,2

with

b2=0.5

for the Clenshaw-Curtis quadrature or

b2=θ=45∘θ=60∘Wθdθθ=45∘θ=75∘Wθdθ=θ=45∘θ=60∘sinθ∆θdθθ=45∘θ=75∘sinθ∆θdθ=cosθ|θ=45∘θ=60∘cosθ|θ=45∘θ=75∘≈0.46200

for the sin(q) quadrature [5].

Figure 6: Visualization of RX cuts, cutRX,i.