# 3GPP TSG-RAN WG4 Meeting #115                               	R4-2505858

# St. Julian’s, Malta, May 19 – May 23, 2025

Agenda item:	7.28.7

Source:	Apple

Title:	On Demodulation Requirements for LP-WUS

Document for:	Discussion

# Introduction

5G systems are designed and developed targeting for both mobile telephony and vertical use cases. Besides latency, reliability, and availability, UE energy efficiency is also critical to 5G. Currently, 5G devices may have to be recharged per week or day, depending on individual’s usage time. In general, 5G devices consume tens of milliwatts in RRC idle/inactive state and hundreds of milliwatts in RRC connected state. Designs to prolong battery life is a necessity for improving energy efficiency as well as for better user experience.

The Rel-18 study item for "Study on low-power wake up signal and receiver for NR" includes investigations to the followings.

- The low-power wake-up signal and receiver, including power saving benefit, coverage, system overhead impact, network energy impact and other related aspects.

- The receiver architecture for low-power wake-up receiver and provide analysis for power consumption, noise figure and etc.

- L1 design and procedure changes needed to support the low-power wake-up signal and evaluations for the link performances.

- Higher layer protocol changes needed to support the low-power wake-up signals

- Related RAN4 impacts

In RRC IDLE/INACTIVE modes, it’s observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied. In RRC CONNECTED mode, it’s observed that moderate UE power saving gain (up to more than 10%) is obtained with marginal impact to capacity by using LP-WUS/WUR to trigger UE MR PDCCH monitoring compared with existing UE power saving techniques, across different types of XR traffic and system load scenarios.

# Discussion

RAN4 RRM and RF sesions have continued monitoring the progress of this WI on RAN1 and RAN2 sessions. RAN1 has mostly completed the signal design and UE operation on both Idle/Inactive and Connected Mode. RAN2 continues on

    - LP-WUS configuration, LP-SS configuration and the Entry/Exit condition for LP-WUS monitoring and RRM relaxation related aspects. RAN4 RRM is focused on RRM Measurements relaxation aspects based on LP-SS, while RAN4 RF is focused on determining REFSENS and SNR for LP-WUS detection.

Observation 1: LP-WUS detection is revelant for both Idle/Inactive and Connected Mode.

For the LP-WUR, there are two types of receivers considered: an OOK receiver and a OFDM receiver. From LP-WUS design point of view, RAN1 has specified OOK (both OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK on-duration symbols, and with the possibility of this OFDM sequence to carry information the information as well.

Observation 2: For LP-WUS detection, the LP-WUR receiver can be of OOK type and of OFDM type.

The OOK receiver type performs envelop detection to decode the informaton carried over the OOK ON and OFF symbols. This receiver has lower power consumption but also lower link-level performance. It’s not sensitive to frequency error and it’s moderately sensitive to timing errors. See Figure 1.

Figure 1: OOK Receiver - homodyne/zero-IF architecture with baseband envelope-detection based LP-WUR

The OFDM receiver has higher complexity since it may perform time-domain or frequency-domain correlation with known sequences, decoding the information carried in the overlay sequences. This receiver has higher power consumption but also a better link-level performance. It can be sensitive to both time and frequency errors, but may reuse PSS/SSS for synchronization. See Figure 2.

Figure 2: OFDM Receiver - IQ based LP-WUR

In terms of the OOK waveform, RAN1 has designed based on OOK with Manchester Encoding as the baseline waveform for LP-WUS and LP-SS. The OOK-based receiver can be used to detect the OOK symbols, while the OFDM receiver can detect also the OFDM overlaid sequence. DFT-s-OFDM is used to generate OOK waveform to allow the reuse of the gNB hardware for generating this waveform while multiplexing this newly generated signal with legacy NR signals.

Observation 3: New OOK modulation and new OFDM overlaid sequence detection has been specified for LP-WUS detection, which are novel in the context of TS 38.101-4.

In terms of modulation, M = 1, 2, or 4 is the number of OOK symbols within an OFDM symbol.

In terms of information payload, a LP-WUS supports up to 5 information bits, which can then be mapped up to 32 codepoints, which are used to identify groups of UEs that are targeted to start PDCCH/Paging monitoring on a given Paging Occasion (PO), and this codepoint based wake-up indication is adopted for LP-WUS for both idle/inactive and connected mode.

In terms of coding, for # of information bits >=3 bits, Reed-Muller (RM) code is used with rate matching to match the number of OFDM symbols available. For more details, see TS 38.212 Section 5.3.3 where RM code is specified for Uplink Control Information (UCI). For 1 and 2 information bits, the procedure is given in the same Section 5.3.3.

Observation 4: For OOK envelope detection, LP-WUS coding aspects have both an information payload dimension and a code rate dimension.

In terms of the so-called Overlaid OFDM Sequences, these are specified to be on top of the OOK ON symbols, i.e. the ON duration of the OOK pattern transmitted for the LP-WUS. These overlaid sequences may or may not carry actual information. The receiver can perform sequence correlation in either time or frequency domain, and it is expected that an OFDM receiver can detect early the LP-WUS compared to an OOK receiver, depending on the number of candidate overlay sequences for each OOK ON symbol.

When using overlaid OFDM sequences, several details are important. First, the length of the overlaid OFDM sequences is given by 12 * X / M for all M values, where X = 11 is the number of PRBs for the LP-WUS, meaning the sequence length is 132, 66 and 33 for M = 1, 2 and 4, respectively. In addition, overlaid OFDM sequences use Zaddoff-Chu (ZC) sequences with cyclic extension. BZC is given by the largest prime number such that BZC < LZC, with the corresponding allowed root values and cyclic extensions, which are in turn obtained from RRC signalling. In addition, the maximum number of candidate overlaid sequences per OOK ON chip for one cell is 16 for M=1; 8 for M=2; and 4 for M=4

If the overlaid sequence does not carry information, a single overlaid sequence is on each OOK ‘ON’ symbol. Otherwise, the raw information bits are used to determine the overlaid OFDM sequences for each OOK ON chip. This is

- Assume L is the number of candidate overlaid OFDM sequences for one OOK ON chip.

- Assume N is the number of raw information bits. If N / log2 L is not an integer, bit 0 as MSB is used for padding.

- The raw information bits are divided into K segments from MSB to LSB, where K = ceil(N / log2 L).

In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially. See Figure 3 below.

Figure 3: Overlay sequence to bit mapping in OOK waveform

Observation 5: For OFDM detection, LP-WUS can be transmitted in several ways based on the configured M-factor, which determines a maximum number of candidate overlaid sequences per OOK ON chip.

In general, it can be observed that several new aspects are introduced with this new LP-WUS waveform. Hence, RAN4 needs to discuss if and how to determine the relevant demodulation requirements based on the multiple novel aspects discussed above.

Proposal 1: RAN4 to discuss potential new demodulation requirements for the detection rate of LP-WUS, considering the novel degrees of freedom of this new waveform.

However, another important aspect is the testability of this new signal. Similar to PBCH, the LP-WUS does not have a direct mechanism to determine its detection rate. The only way we can foresee is to indirectly detect LP-WUS by properly configure a PDCCH test. However, the problem here is that to know if a certain PDCCH decoding instance was triggered by the UE detecting LP-WUS or whether the MR monitored the configured PDCCH windows directly. Hence, this very aspect can undermine the purpose of defining LP-WUS demodulation requirements in the first place.

Proposal 2: RAN4 to discuss the testability aspect of LP-WUS and whether this can justify the purpose of defining LP-WUS demodulation requirements in the first place.

Finally, another aspect that has being brought during this work item is the LP-WUS false alarm rate (FAR), but we prefer to leave this up to UE implementation. First, FAR is not a new concept, since it already happens in PDCCH detection, and it’s not a phenomenon that can be tested by the network or by TE vendor. Second, it’s up to UE implementation to minimize FAR in order to maximize the power saving gains of this feature.

Proposal 3: RAN4 not to consider LP-WUS false alarm rate as part of demodulation requirements discussion.

# Conclusion

In this contribution, we continue the discussion on the simulation assumption.

Observation 1: LP-WUS detection is revelant for both Idle/Inactive and Connected Mode.

Observation 2: For LP-WUS detection, the LP-WUR receiver can be of OOK type and of OFDM type.

Observation 3: New OOK modulation and new OFDM overlaid sequence detection has been specified for LP-WUS detection, which are novel in the context of TS 38.101-4.

Observation 4: For OOK envelope detection, LP-WUS coding aspects have both an information payload dimension and a code rate dimension.

Observation 5: For OFDM detection, LP-WUS can be transmitted in several ways based on the configured M-factor, which determines a maximum number of candidate overlaid sequences per OOK ON chip.

Proposal 1: RAN4 to discuss potential new demodulation requirements for the detection rate of LP-WUS, considering the novel degrees of freedom of this new waveform.

Proposal 2: RAN4 to discuss the testability aspect of LP-WUS and whether this can justify the purpose of defining LP-WUS demodulation requirements in the first place.

Proposal 3: RAN4 not to consider LP-WUS false alarm rate as part of demodulation requirements discussion.

# References

[1] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo / NTT DOCOMO, RAN #105, Melbourne, Australia, September 9-12, 2024