3GPP TSG-RAN WG4 Meeting #115	 					     							R4-2505864

Malta, Malta, 19-23 May 2025

Agenda item:			7.29.2

Source:	Apple

Title:	On UE feature list for CSI-RS based L1 measurement

Document for:	Discussion

# Introduction

This contribution presents feature list for CSI-RS based L1 measurement.

# Discussion

- NR_Mob_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56. NR_Mob_Ph4 | 56-1 | Number of CSI-RS resources for L1-RSRP measurement within a slot | - The max number of CSI-RS resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for L1-RSRP measurement | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the number of CSI-RS resources for L1 measurement within a slot. | Per BC | No | Yes | N/A | Candidate value: {1,2,3,4,5,6,7,8,16,32, 48,64} Note: It is also counted in FG 2-24 | Optional with capability signaling |
|  | 56-2 | Number of total CSI-RS resources to be measured | - The max number of total CSI-RS resources of serving cells and neighboring cells across all frequency layers for L1 measurement. Both LTM candidate cell(s) and serving cells for CSI-RS based L1-RSRP measurement are counted, regardless of whether CSI-RS based L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the total number of CSI-RS resources of serving cells and neighboring cells across all frequency layers for L1 measurement. | Per BC | No | No | N/A | Candidate values: {2,4,8,12,16,32,64} Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Optional with capability signaling |
|  | 56-3 | Skipping SSB based L1 measurement in FR2-1 | - Support of skipping SSB based L1 measurement for FR2-1 for CSI-RS based L1 measurement | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | N/A | The network cannot have optimized measurement configuration | Per UE | no | FR2-1 only | N/A | - For a UE not supporting the capability, SSB based L1 measurement is necessary for FR2-1. - The CSI-RS resources for L1 measurement should be at least Type-D QCL’ed with the SSB for L1 measurement. - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped: - The CSI-RS resources for L1 measurement should be Type-D QCL’ed with the SSB for L3 measurement. - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor. | Optional with capability signalling |