3GPP TSG-RAN4 Meeting#115	R4-2505885

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.5.3

Source:	Apple

Title:	On methods for reducing the number of UE Rx chains for fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

At the last RAN4 meeting, a WF was agreed that captures the agreements and open issues [2]. In this contribution, we share our views on the open issues:

- How to indicate UE capability for supporting “one Rx RF chain” for DL fragmented carriers

- Discussion on semi-statically switch hardware resources procedure and fallback considerations

# 2	Discussion

How to indicate UE capability for supporting “one Rx RF chain” for DL fragmented carriers

On signaling aspects, we shared our pros and cons analysis with three alternatives in [3]. During the discussion at the last meeting, the following agreements were captured in the WF [2]:

#### Issue 2-1: How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers - Consider the following alternatives - Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication - Alternative 1 has no modification of CA configurations in RAN4 - Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers. - Alternative 1a has no modification of CA configurations in RAN4 - Alternative 2: use the new notation for CA configurations with single Rx RF Chain - Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

Our views on the two alternatives are provided below.

- Usually, the CA configurations supported by a UE are reported to the network when the UE camps on a cell and receives capability inquiry from the network. In the case of UE RF architecture switching (switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA), the UE may need to update its supported CA configuration. In this regard, our initial analysis is as follows:

  - For Alt. 1, the UE would report its supported CA configuration with 1 additional bit indicating whether Rx sharing capability is needed to support the CA configuration in RAN2 signaling. For example, if the UE supports CA_n7(2A)-n25(2A) with Rx sharing capability and supports CA_n7A-n25(2A) without, UE will report CA_n7(2A)-n25(2A) with the bit set to 1 and CA_n7A-n25(2A) with the bit set to 0. Such UE capability reporting is done when the UE camps on a cell, and there is no need for the UE to further update its CA configuration when there is RF architecture switching.

  - For Alt. 2, since new notation of CA configuration with single Rx RF chain is specified in RAN4 and reported by UE when the UE camps on a cell, there is no need for the UE to further update its CA configuration when there is RF architecture switching.

- For Alt. 1a, for futureproof-ness of the signaling design to allow UEs to support different max. bandwidth with a single Rx RF chain, it can be further discussed.

- While Alt. 2 can work, we do not prefer to consider Alt. 2a because the current CA bandwidth classes are defined for contiguous CA, not for non-contiguous CA. Since a new notation is being discussed, we should aim to find other ways to indicate the frequency separation between two CCs.

- Comparing Alt. 1 with Alt. 2, we slightly prefer Alt. 1 as it requires no changes to current RAN4 CA notation. Therefore, when specifying potential new requirements for this feature, the change to the specification is minimal.

- We also note that in our understanding UE indication of when Rx sharing can be enabled by the network is needed when some triggering condition is met.

Proposal 1: Further discussion is needed to understand all the details about each alternative. Meanwhile, we prefer Alt. 1 to Alt. 2.

Discussion on semi-statically switch hardware resources procedure and fallback considerations

There was further clarification made at the last meeting between CA fallback and UE architecture switching, as captured in the WF [2]:

#### Issue 2-2: Discussion on semi-statically switch hardware resources procedure and fallback considerations - Further discussions on - CA fallback with/without UE architecture switching - Consider the fall back - from high order CA to lower order CA - from CA to single carrier mode - Decided by network - UE architecture switching: - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA - FFS on whether it is decided by UE itself or decided by network - FFS on whether the additional indication from UE side is needed

For this SI, UE architecture switching should be informed to the network and the network, as a result, may change the CA configuration accordingly. For example, when the UE switches from 2 Rx RF chains to 1 Rx RF chain, the network would understand that there is a spared Rx RF chain at the UE, and if needed, can be used to support one or more CCs. Thus, there is a potential to configure a higher order CA configuration to the UE. On the other hand, when the UE switches from 1 Rx RF chain to 2 Rx RF chains, and if 1 Rx RF chain is used to support two non-contiguous CCs and all other Rx RF chains are used to support other CCs in the CA configuration, such switching indicates that the network should configure a CA fallback to the UE. As such, such architecture switching should be informed to the network by the UE and decided by the network together with potential CA fallback.

We acknowledge that in some cases, the UE can decide on its own to switch between 2 Rx RF chains and 1 Rx RF chain for benefits like power saving, which may or may not be transparent to the network depending on if some measurement is needed by the UE to determine if 1 Rx RF chain or 2 Rx RF chain works given the measured blocker level. If such measurement is need and configured by the network, it is not transparent to the network. However, such architecture switching may not involve CA configuration change, such as CA fallback. Whether such cases should be considered in the SI can be further discussed.

Observation 1: For this SI, UE architecture switching should be informed to the network and the network, as a result, may change the CA configuration accordingly.

Proposal 2: It is proposed to allow the UE to inform the network of potential UE architecture switching and the network then decides such switching together with CA configuration change including CA fallback accordingly.

Next, we share our views on CA fallback associated with UE architecture switching. It is assumed that the UE works in in the inter-band CA case, where single RX chains A1 and A2 are used to support CC1 and CC2 in band A, and single RX chains B1 and B2 are used to support CC3 in Band B. When the UE cannot support CC1 and CC2 with single RX chain, as shown below, there are two fallback cases, Case 1 and Case 2:

- Case 1: Release one CC, either CC1 or CC2, in Band A

- Case 2: Release CC3 in Band B

Fallback Case 1: Release one CC, either CC1 or CC2, in Band A

Fallback Case 2: Release CC3 in Band B

It may be more straightforward to consider Case 1, since if a UE can use single RX chain to support two CCs in a band, it is expected to support just one CC.

On the other hand, there could be other considerations from the network/UE side. For example, if CC1 and CC2 channel bandwidth is larger than CC3 channel bandwidth, to support higher DL throughput, it is beneficial to fall back to CC1 + CC2 CA.

Therefore, which CA fallback case to support is proposed to be left to UE capability and network configuration.

Proposal 3: Which CA fallback case to support in inter-band CA is proposed to be left to UE capability and network configuration. This can be further discussed in the WI phase.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: Further discussion is needed to understand all the details about each alternative. Meanwhile, we prefer Alt. 1 to Alt. 2.

Observation 1: For this SI, UE architecture switching should be informed to the network and the network, as a result, may change the CA configuration accordingly.

Proposal 2: It is proposed to allow the UE to inform the network of potential UE architecture switching and the network then decides such switching together with CA configuration change including CA fallback accordingly.

Proposal 3: Which CA fallback case to support in inter-band CA is proposed to be left to UE capability and network configuration.

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

R4-2505109, "WF on NR FR1 DL fragmented carriers study," Moderator (Mediatek Inc.), Apple, Qualcomm

R4-2504450, "On methods for reducing the number of UE Rx chains for fragmented carriers," Apple