3GPP TSG-RAN WG4 Meeting #115		R4-2505887

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.25.4.1

Source:	Apple

Title:   	On UE RRM requirements for UE-to-UE CLI handling

Document for:	Discussion

# 1	Introduction

In the latest R19 SBFD WI [1], there is RAN4 objective on UE CLI measurement and reporting, as shown below:

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

At the last RAN4 meeting, a WF was agreed [2]. In this contribution, we continue to share our views on the open issues.

# 2	Discussion

Time offset between DL timing and SRS arrival timing

In response to RAN4’s inquiry, RAN1 provided their views in [3] as copied below:

# 1 Overall description When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios. For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

Given the following points from the RAN1 reply LS:

- For intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded.

- From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

We think it is reasonable that RAN4 specify requirements for intra-cell case only. As a result, the cell phase synchronization error of 3us is not considered in the time offset between DL reference timing in the serving cell and SRS arrival time. More specifically, the time offset is no larger than Terror_SRS_RSRP, where Terror_SRS_RSRP is TC × NTA_offset + 1.67us for FR1 or TC × NTA_offset + 0.67us for FR2.

Proposal 1: Per RAN1 LS that from RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement, it is proposed that RAN4 focuses on requirements for Intra-cell UE-to-UE L1 CLI measurement scenario.

Scheduling restriction

Regarding scheduling restriction, the following agreements were captured in the WF [2]:

#### Issue 1-2-2: Scheduling restriction (for L1-SRS-RSRP) - Agreement (online session on Monday with update) - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for SRS resource and Y symbols before, where Y is re-using same number as R16 requirements. - For the symbols after the SRS resource, not define scheduling restriction on symbols after symbols for SRS resource in TS 38.133, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211. - For DL, - if UE is able to support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies. - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements. FFS whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

#### Issue 1-3-2: Scheduling restriction (for L1-CLI-RSSI) - Agreements (adhoc session on Tuesday with addition) - Method #1: - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for RSSI resource. - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource - Method #3: - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for RSSI resource. - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource. - Note: the common understanding in RAN4 if the RSSI in UL suband is too strong compare to the wanted signals in DL suband(s), UE may fail to decode the PDCCH or PDSCH. - FFS whether restricted symbol(s) is needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband. - Generic assumption for defining RRM requirements, not intend to add it in the core requirement. - For all Method #1, Method #2 and Method #3, - Note: The RF BW is wide enough to cover DL active BWP, including PRBs from both DL subband(s) and UL subband that fall within the DL active BWP.

There are two FFSs concerning whether restricted symbol(s) is needed for the symbol(s) after SRS or RSSI resource to consider the switching between UL subband and DL subband. Since CLI measurement is performed within the active DL BWP and the UE stays in the DL reception mode, no transient period is required for UE switching between UL subband and DL subband. As a result, no restricted symbol is needed.

Proposal 2: It is confirmed that no restricted symbol(s) is needed for the symbol(s) after SRS or RSSI resource due to the switching between UL subband and DL subband.

Measurement restriction

There is the following options in the WF [2]:

#### Issue 1-2-3: Measurement restriction (for L1-SRS-RSRP) - Agreements - Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options - Option 1: Prioritize L1-SRS-RSRP measurement - Option 2: Prioritize L1-CLI-RSSI measurement - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI

We agree that when measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI. This is because the measurement timing for the two measurement quantities are different, one follows the DL timing and the other following DL timing with a time offset. Furthermore, depending L1-CLI-RSSI is for method#1 or method#3, the UE may use different AGC settings for L1-SRS-RSRP and L1-CLI-RSSI measurement.

As to whether to prioritize L1-SRS-RSRP and L1-CLI-RSSI measurement, several factors need to be considered, including whether there is sudden power change that demands an immediate measurement for AGC setting, how CLI interference leakage into DL subband is compared to the wanted signals, despite the fact that CLI leakage power in DL subband is much lower than CLI itself in the UL subband. In different cases, the UE may need to prioritize different measurements. Therefore, Option 3 is a reasonable choice.

Proposal 3: When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI. It is left to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: Per RAN1 LS that from RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement, it is proposed that RAN4 focuses on requirements for Intra-cell UE-to-UE L1 CLI measurement scenario.

Proposal 2: It is confirmed that no restricted symbol(s) is needed for the symbol(s) after SRS or RSSI resource due to the switching between UL subband and DL subband.

Proposal 3: When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI. It is left to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

# 4	References

RP-241614, "Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)," Huawei

RP-2504904, "WF on RRM requirements for NR_duplex_evo," Huawei, HiSilicon

R1-2410890, "Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation," RAN1