3GPP TSG-RAN WG4 Meeting #115														R4-2505889

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.18.3.1

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#113, which includes open issues for further discussion. In this contribution, we continue the discussion for FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor.

# Discussion

In the WF, the following agreements and open issues were captured.

## Sub-topic 1-1 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF ### Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF Agreement in RRM session (Monday): - For the baseline scenario agreed in RAN4 #112, limit the serving cell to a FR2 serving cell. - Leave the decision to RAN2. Further discuss the details for the LS to RAN2, and the LS can only be approved if the content is agreeable. - Details of the LS can refer to R4-2504962. ### Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Further discuss this issue. ### Issue 1-1-3: Status report of fast beam sweeping Further discuss this issue. ## Sub-topic 1-2 Requirement of L3 measurement delay reduction by optimizing Rx BSF ### Issue 1-2-1: Applicability of L3 measurement delay reduction by optimizing Rx BSF Further discuss this issue. ### Issue 1-2-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Agreement in the ad-hoc: - Introduce the UE capability of reduced N, the values of which are {2, 4 and 6}. And UE chooses one value to report according to its capability. FFS the granularity of the UE capability in issue 1-2-4. ### Issue 1-2-3: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF Agreement in the ad-hoc: - For each scenario for PC3 UE, the described factors in the table should be replaced with capability value. Scenarios; Factors. 1; Mpss/sss_sync_w/o_gaps = [reduced N]*3 Mmeas_period_w/o_gaps = [reduced N]*3. 2; Mpss/sss_sync_with_gaps = [reduced N]*3 Mmeas_period_with_gaps = [reduced N]*3. 3; Mpss/sss_sync_inter = [reduced N]*3 Mmeas_period_inter = [reduced N]*3 MSSB_index_inter = [reduced N]*3. 4; Mpss/sss_sync_inter = [reduced N]*5 Mmeas_period_inter = [reduced N]*5 MSSB_index_inter = [reduced N]*3. 5; If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then N in the HO requirement will be replaced by [reduced N] when the target cell is in FR2-1 and the PC3 UE supports [Rel-19 Rx FBS].. 7; RRC Re-establishment/redirection Can be discussed directly in the draft CR.. ### Issue 1-2-4: Other aspects of UE capability design Further discuss this issue.

FBS exit condition

Here the exit condition is not the deactivation of FBS based on signal quality being above the threshold. Instead, it refers to the exceptional cases where FBS is not deactivated when considering the serving cell signal quality alone. The main reason is the need of UE power saving or overheating. For this purpose, UAI mechanism can be used. Either the existing UAI IE “multiRx-PreferenceFR2-r18” can be re-purposed to indicate not only L1 multi-RX operation, but also L3 measurement delay reduction, or a new IE such as “FastL3Measurement-r19” can be designed.

Proposal 1: To consider UE power consumption or overheating, UAI mechanism should be used. Either the existing UAI IE “multiRx-PreferenceFR2-r18” can be re-purposed to indicate not only L1 multi-RX operation, but also L3 measurement delay reduction, or a new IE such as “FastL3Measurement-r19” can be designed.

Furthermore, we want to discuss the issues and the proposed solution raised in [2].

Observation 1: Based on agreed triggering condition (serving cell’s signal quality), it is possible that UE will always activating multiple panels for L3 measurement, which is problematic for at least following case: - Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving. - Case 2: UE is still in the cell centre and the triggering conditions are met due to block. Observation 2: Rel-18 UAI indication is not sufficient as the decision is made by gNB, and gNB has no idea whether fast L3 measurement is urgent or not (e.g. case 1 and case 2 mentioned above). And gNB even don’t known whether fast measurement is triggered or not. Proposal 1: Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification.

First, we agree the two cases would lead to inefficient use of FBS and hence UE power consumption. However, we think the proposed solution that UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results is problematic since it is hard for the network/UE to predict with good confidence that the UE is currently in one of the two cases, i.e., Case 1 and Case 2. In addition, even after the UE reports qualified measurement results, the network may want to wait for more report to be sent in before making a handover/SCell change decision. Therefore, we propose to consider the two possible solutions:

- Option 1: No specific action is taken, assuming UE can use UAI to inform the network that it needs to fall back to Non-FBS mode (i.e., deactivate FBS operation) if power consumption/overheating becomes an issue.

- Option 2: After the UE sends a qualified measurement report, the UE can wait for a certain period. If no decision form the network is made, the UE can deactivate FBS. To implement it, a timer can be used. The timer starts when the UE sends a qualified measurement report (say A2 or A3), and when the timer expires, the UE exits from FBS operation. The value of the timer should be at least two or three times the L3 SSB-based measurement periods.

Proposal 2: To address the power consumption issues in the two cases (Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving; Case 2: UE is still in the cell centre and the triggering conditions are met due to block.), it is proposed to consider the following two options:

- Option 1: No specific action is taken, assuming UE can use UAI to inform the network that it needs to fall back to Non-FBS mode (i.e., deactivate FBS operation) if power consumption/overheating becomes an issue.

- Option 2: After the UE sends a qualified measurement report, the UE can start a timer, and when the timer expires, the UE exits from FBS operation.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: To consider UE power consumption or overheating, UAI mechanism should be used. Either the existing UAI IE “multiRx-PreferenceFR2-r18” can be re-purposed to indicate not only L1 multi-RX operation, but also L3 measurement delay reduction, or a new IE such as “FastL3Measurement-r19” can be designed.

Proposal 2: To address the power consumption issues in the two cases (Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving; Case 2: UE is still in the cell centre and the triggering conditions are met due to block.), it is proposed to consider the following two options:

- Option 1: No specific action is taken, assuming UE can use UAI to inform the network that it needs to fall back to Non-FBS mode (i.e., deactivate FBS operation) if power consumption/overheating becomes an issue.

- Option 2: After the UE sends a qualified measurement report, the UE can start a timer, and when the timer expires, the UE exits from FBS operation.

# References

- R4-2504900, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2501218, Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, Huawei, HiSilicon