3GPP TSG-RAN4 Meeting  #115	R4-2505893

Malta, MT, 19th – 23rd May, 2025

Agenda item:	4.2.2

Source:	Apple

Title:	Power control ambiguity issue for PC1.5 UEs

Document for:	Discussion

# 1	Introduction

In this contribution, we discuss the ambiguity issue in power control for PC1.5 UEs and provide solutions.

# 2	Discussion

In R17, power class (PC) 1.5 was introduced for UE, where the maximum output power is 29dBm. However, the UE needs to use two TXs, each equipped with a 26dBm PA to achieve the maximum output power. In Table 6.2.1-1: UE Power Class of 38.101-1 [1] copied below, there is Note 5 for PC1.5 saying it is “Achieved via dual Tx”.

Table 6.2.1-1: UE Power Class NR band; Class 1 (dBm); Tolerance (dB); Class 1.5 (dBm); Tolerance (dB); Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB). n34; 295; +2/-3; 26; +2/-3; 23; ±2. n38; 23; ±2. n39; 295; +2/-3; 26; +2/-3; 23; ±2. n40; 316; +2/-3; 295; +2/-3; 26; +2/-3; 23; ±2. n41; 316; +2/-3; 295; +2/-33; 26; +2/-33; 23; ±23. n47; 23; ±2. n48; 23; +2/-3. n50; 23; ±2. n51; 23; ±2. n53; 23; ±2. n54; 23; ±2. n65; 23; ±2. n66; 316; +2/-3; 26; +2/-3; 23; ±2. n70; 26; +2/-3; 23; ±2. n71; 316; +2/-3; 26; +2/-3; 23; +2/-2.5. n72; 316; +2/-3; 23; ±2. n74; 23; ±2. n77; 316; +2/-3; 295; +2/-3; 26; +2/-3; 23; +2/-3. n78; 316; +2/-3; 295; +2/-3; 26; +2/-3; 23; +2/-3. n79; 295; +2/-3; 26; +2/-3; 23; +2/-3. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: Achieved via dual Tx. NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 7: The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. NOTE 8: PC1 in Band n100 and n101 is allowed only for FRMCS cab-radio’s, i.e. user equipments installed on trains with external antenna on top of the train roof at approximately 4m over ground..

In the PCMAX formula in 38.101-1(copied below), PPowerClass = 29dBm for PC1.5. and PCMAX is used in RAN1 power control for PUSCH.

### 6.2.4 Configured transmitted power The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds: PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass + ΔPPowerBoost} where PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7]; PPowerClass is the maximum UE power specified in Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation, without taking into account the tolerance specified in the Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation;

For PUSCH, there are different codes specified for 1 layer transmission, as shown blow [2].

Because the UE cannot achieve the max. Output power with a single TX, when the network configures the UE with one port MIMO transmission, such as TPMI 0 or 1 in the above table, there is an ambiguity in calculating PCMAX. More specifically, if PCMAX is based on 29dBm, but the UE can only transmit as high as 26dBm. This would cause misunderstanding at the network and may lead to performance degradation.

To address this ambiguity issue, there are two options:

- Option 1: Improve Note 5 to make it clearer, e.g., “Achieved via dual Tx. If the network configures the UE with TPMI 0 or 1 for single layer transmission in Table 6.3.1.5-1 in 38.211, the UE maximum output power is 26dBm.” The detailed wording can be further discussed.

- Option 2: Enhance the UE PHR reporting. If the UE is configured with TPMI 0 or 1 for single layer transmission in Table 6.3.1.5-1 in 38.211, the UE sends a PHR report updating its PCMAX to 26dBm instead of 29dBm.

Proposal 1: To address the power control ambiguity issue for PC1.5 UEs, it is proposed to consider one of the following two options:

- Option 1: Improve Note 5 to make it more clear, e.g., “Achieved via dual Tx. If the network configures the UE with TPMI 0 or 1 for single layer transmission in Table 6.3.1.5-1 in 38.211, the UE maximum output power is 26dBm.” The detailed wording can be further discussed.

- Option 2: Enhance the UE PHR reporting. If the UE is configured with TPMI 0 or 1 for single layer transmission in Table 6.3.1.5-1 in 38.211, the UE sends a PHR report updating its PCMAX to 26dBm instead of 29dBm.

# 3	Conclusions

In this contribution, we make the following proposals and observations.

Proposal 1: To address the power control ambiguity issue for PC1.5 UEs, it is proposed to consider one of the following two options:

- Option 1: Improve Note 5 to make it clearer, e.g., “Achieved via dual Tx. If the network configures the UE with TPMI 0 or 1 for single layer transmission in Table 6.3.1.5-1 in 38.211, the UE maximum output power is 26dBm.” The detailed wording can be further discussed.

- Option 2: Enhance the UE PHR reporting. If the UE is configured with TPMI 0 or 1 for single layer transmission in Table 6.3.1.5-1 in 38.211, the UE sends a PHR report updating its PCMAX to 26dBm instead of 29dBm.

# 4	References

TS 38.101-1, "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

TS 38.211, "NR; Physical channels and modulation"