3GPP TSG-RAN WG4 Meeting #115						R4-2505894

Malta, MT, 19th – 23rd May, 2025

Agenda item:	4.5.1

Source:	Apple

Title:	(NR_newRAT-Core) On UE handling of TX timing larger than MTTD for CA and DC

Release:	Rel-15

Document for:	Discussion

# 1. Introduction

In Rel-15 gradual timing adjustment and MTTD requirements were introduced.  In RAN4#114 potential update to the gradual timing and MTTD requirements was discussed. In this contribution we share our views on defining behavior for handling TX timing larger than MTTD requirements.

# 2. Discussion

In RAN4#114 the issue TTD exceeding MTTD when UE applies gradual timing adjustment or receives new TA command was discussed.

On gradual timing adjustment and MTTD, the following proposals were discussed in RAN4#114:

- Proposals

  - Option 1 (Apple CR 2500254):

    - the UE shall stop gradual timing adjustment on SCell or PSCell If MTTD is exceeded due to gradual timing adjustment

  - Option 2 (MTK comments on NWM):

    - It seems fine without any changes, as the current spec means no requirements in the spec.

    - If it is agreed to introduce this requirement, prefer to LTE wording to avoid NBC changes, i.e. "UE may” stop transmission or stop adjustment would be more appropriate.

  - Option 3 (Nokia CR R4-2501887):

    - A UE may stop transmitting on the PSCell. The UE is allowed to transmit if UE meets all the transmit timing requirements; otherwise, the UE shall not transmit any uplink signal.

  - Option 4 (Ericsson CR R4-2501615 for Rel-18):

    - A UE shall stop transmitting on the PSCell if timing difference exceeds MTTD and the UE cannot continue to regulate its transmit timing.

The main points from the discussion in RAN4#114 were

- UE should be allowed to continue to adjust if it has the capability of handling TTD larger than MTTD

- UE behavior when TTD exceeds MTTD or its TTD limit

  - Option 1: UE stops adjustment

  - Option 2a: UE stops UL transmission on PSCell

  - Option 2b: UE stops all UL transmission

One proposal was that UE continues adjustment as long as it meets all the timing requirements. The issue with such a requirement that is that UE doesn’t know if its meeting or violating any timing requirements.

Defining a behavior based on the condition of meeting a certain requirement is not feasible.

The behavior defined should not restrict UEs that can handle TTD>MTTD.

In case UE can handle TTD > MTTD, RAN4 to discuss if we should introduce UE capability to let NW know of such capability. Alternatively, without explicit capability, leave it to UE implementation and operating mode for UE to determine it capability to handle TTD beyond MTTD. Since this is related to UL transmission, the UE can be allowed to continue adjustment if it doesn’t cause UE Tx EVM degradation.

- The UE shall be allowed to continue adjustment if it doesn’t cause UE TX EVM degradation.

On the UE behavior when UE stops UL transmission, the NW would not have any idea why the UE stopped UL transmission. This is different from stopping transmission after receiving TA command from the NW. In this case gradual timing adjustment is don’t at the UE, and NW doesn’t know when UE starts it. Stopping UL transmission could indicate link failure on the SCell or PSCell while this is not the case.

If UE stops transmission, NW might not know the reason and assume its link failure.

If the UE stops adjustment, but continues to transmit, the NW would send a new TA command and there would be no issue.

In case the consensus in RAN4 is to stop UL transmission, there needs to be some indication to the NW on why transmission was stopped. The indication can be sent on PCell or the other TAG where UL timing has not exceeded MTTD or the TTD limit UE can handle.

- In case UE stops transmission, UE shall send an indication to NW via signaling on the UL via PCell or other TAG.

Introducing the signaling would be the only way to let the NW know the reason for UE not transmission on the UL due to TTD exceeding MTTD with gradual timing adjustment. Introducing such signaling from previous release might not be very practical, and a more practical solution might be to define behavior to stop adjustment when TTD exceeds the limit.

Introducing signaling for this might not be feasible from earlier release.

A more practical solution could be to introduce UE behavior to stop adjustment when TTD exceeds the limit.

- In case TTD exceeds MTTD during gradual timing adjustment on any TAG, UE shall continue gradual timing adjustment unless it impacts UE TX EVM, otherwise UE shall stop gradual timing adjustment on that TAG.

On the issue of TTD exceeding MTTD when new TA command is received, the following was discussed in RAN4#114:

Issue: UE timing adjustment after receiving the TA command

RAN4 to further discuss:

As starting point, capture in the specification that the UE may stop transmission on SCell or PSCell If MTTD is exceeded following the TAC.

- FFS other details or any other conditions

- FFS the applicable release

We propose to confirm defining the UE behavior that UE shall stop transmission on SCell or PSCell if MTTD is exceeding following a TA command. In case UE has capability to handle TTD> MTTD, UE can be allowed to continue transmission if UE TX EVM is not degraded.

- In case TTD exceeds MTTD after receiving new TA command, UE may apply the new TA command if UE TX EVM is not degraded, otherwise UE shall stop transmission on SCell or PSCell.

In case UE supports asynchronous operation, it would be capable of handling larger TTD. In case UE only supports synchronous operation – for example interband CA, only synchronous DC operation, then the UE would have a limit on the TTD it can handle. The UE behavior for handling large TTD would only be applicable in this case. Hence, the UE behavior would be applicable for the following cases:

- 	Inter-band CA
-	Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support asynchronous operation (Rel-16)
-	Intra-band EN-DC if UE doesn’t support asynchronous operation
-	Inter-band NE-DC if UE doesn’t support asynchronous operation
-	Inter-band NR-DC if UE doesn’t support asynchronous operation

- UE shall stop transmission when TAC is received/ stop gradual timing adjustment if TTD exceeds MTTD in the following cases:
       Inter-band CA
       Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support       asynchronous operation 
       Intra-band EN-DC if UE doesn’t support asynchronous operation
       Inter-band NE-DC if UE doesn’t support asynchronous operation 
       Inter-band NR-DC if UE only supports synchronous operation

On applicable release to introduce this UE behavior – Timing requirements were introduced in Rel-15 which has been deployed for few years now. The clarification to UE behavior could be introduced in Rel-15 if agreeable, otherwise Rel-16 onwards.

- Further discuss if the clarification to UE behavior with TTD exceeds a limit is introduced from Rel-15 or Rel-16.

Our proposals are captured in our companion CR [1].

# 3. Conclusion

In this paper, we propose an update to the gradual timing requirements in CA and DC. Our observations and proposals are captured below:

- Defining a behavior based on the condition of meeting a certain requirement is not feasible.

The behavior defined should not restrict UEs that can handle TTD>MTTD.

- The UE shall be allowed to continue adjustment if it doesn’t cause UE TX EVM degradation.

If UE stops transmission, NW might not know the reason and assume its link failure.

- In case UE stops transmission, UE shall send an indication to NW via signaling on the UL via PCell or other TAG.

Introducing signaling for this might not be feasible from earlier release.

A more practical solution could be to introduce UE behavior to stop adjustment when TTD exceeds the limit.

- In case TTD exceeds MTTD during gradual timing adjustment on any TAG, UE shall continue gradual timing adjustment unless it impacts UE TX EVM, otherwise UE shall stop gradual timing adjustment on that TAG.

- In case TTD exceeds MTTD after receiving new TA command, UE may apply the new TA command if UE TX EVM is not degraded, otherwise UE shall stop transmission on SCell or PSCell.

- UE shall stop transmission when TAC is received/ stop gradual timing adjustment if TTD exceeds MTTD in the following cases:
       Inter-band CA
       Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support       asynchronous operation 
       Intra-band EN-DC if UE doesn’t support asynchronous operation
       Inter-band NE-DC if UE doesn’t support asynchronous operation 
       Inter-band NR-DC if UE only supports synchronous operation

- Further discuss if the clarification to UE behavior with TTD exceeds a limit is introduced from Rel-15 or Rel-16.

# Reference

- R4-2505895, “(NR_newRAT-Core) CR to 38.133 for gradual timing adjustment and MTTD-R15”, Apple.