3GPP TSG-RAN WG4 Meeting #115						R4-2505904

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.17.3

Source:	Apple

Title:	On UE demodulation performance requirements with 8RX

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#114bis the UE demodulation enhancements in Rel-19 were discussed in detail and way forward [1] was agreed. In this contribution we share our views on UE demodulation requirements with 8RX.

# 2. Discussion

## General

The following agreements were made on general aspects:

Definition of MMSE-IRC receiver capability for 8Rx UE Agreement: - Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] only up to 4 layers with two joint 4Rx MIMO detectors. 8Rx UE MMSE-IRC receiver coverage Agreement: - Down select the [limit] to one of 0.5dB or 1.0dB. Remaining proposals: - Option 1: Introduce one more rule on the top of the agreed criteria: After reviewing the simulation results for all the test cases by the agreed criteria, if 2 sets of requirements are required for at least one test case, define two sets of PDSCH requirements for all cases, which are applicable for both baseline and simplified receivers.

In RAN4#114, we had an open issue on receiver coverage

Issue 1-1-2: 8Rx UE MMSE-IRC receiver coverage

Agreement:

- Criteria for decision if one or two sets of PDSCH requirements are to be introduced:
- Secure the span is <=2.5 dB for each receiver type by removal of outliers
- If difference in the average of each receiver type for a specific configuration is > [TBD:limit] then define two sets of requirements for the specific configuration.

In [1] we agreed to introduce 2 sets of requirements for 8RX for PDSCH demod for inter-cell interference and intra-cell inter-user interference. But we didn’t agree on a limit. It would be good to have a limit documented for future reference.

It would be helpful to agree on a limit for future reference although we have agreed on 2 sets of requirements for PDSCH demod with ICI and MU-MIMO.

We propose to choose 1 dB as the limit to decide if 2 sets of requirements are introduced.

- Finalize limit as 1 dB to decide if 2 sets of requirements are introduced.

## Wideband CQI reporting requirements with inter-cell interference

On CQI reporting requirements in ICI the following agreements were made in [1]:

CQI calculation algorithm and Receiver coverage Agreement: - Companies are encouraged to provide suggestions on methodology for deciding CQI reporting requirements in next meeting. - Decision to have one or two sets of requirements is postponed to next meeting when more simulation results are available. Rank for serving cell Agreement: - Introduce CQI reporting requirements with inter-cell interference for Rank 2 only.

To determine if one or 2 sets of requirements are introduced for CQI reporting in ICI, we propose the following method:

- Choose the SINR for defining requirements based on TP gain with baseline receiver.

- At the chosen SINR, if the gamma value difference is > 0.2, then define 2 sets of requirements for simplified and baseline receiver.

- Adopt the following methodology and criteria to decide CQI reporting requirements:
- Choose the SINR for defining requirements based on TP gain with baseline receiver. 
- At the chosen SINR, if the gamma value difference is > 0.2, then define 2 sets of requirements for simplified and baseline receiver

## Applicability rule and release independence

On applicability rule and release independence the following agreements were made in [1]:

Release independency Agreement: - The new Rel-19 requirements shall at least be release independent from Rel-18.

On the open issues –

Test applicability to skip 2/4Rx requirements

Proposals

- Option 1:
For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not.

- Option 2:
To consider if UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

- Option 3:
For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

- Option 4: 
Introduce test applicability under inter-cell interference and under intra-cell inter user interference to skip a 2/4Rx requirement if at least the following applies for each considered 8Rx test:
- UE support both 8Rx and 2/4Rx
- The UE passes an 8Rx requirement with same rank and interference configuration as the 2/4Rx requirement.

- Option 5:
RAN4 shall have the following applicability rules:
- UE that passes Rank2 ICI test case with 8Rx shall skip Rank1 ICI requirements with 4Rx.
- UE that passes Rank2 MU-MIMO test case with 8Rx shall skip Rank1/2 MU-MIMO requirements with 4Rx.

For 8RX the requirements in ICI are with different rank and MCS compared to requirements with 2RX/4RX. Hence there is no corresponding test case/ requirements between 8RX and 2RX/4RX. We support option 1 above to reduce test burden.

- For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not.

Release independency

Agreement:

- The new Rel-19 requirements shall at least be release independent from Rel-18.

Remaining proposals:

- Option 1:
The new Rel-19 requirements shall also be release independent from Rel-17.

- Option 2:
The new Rel-19 requirements shall also be release independent from Rel-17. The requirements shall be mandatory for Rel-18 and optional from Rel-17.

Since R18 PDSCH demod requirements for 8RX are release independent since Rel-17, it is okay to introduce requirements in ICI and MU-MIMO release independent since Rel-17 as well, since no new UE capability is needed for ICI and MU-MIMO.

- The requirements for 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario can be release independent from Rel-17.

# 3. Conclusion

In this paper, we provide our views on UE demodulation requirements with 8RX. Our observations and proposals are captured below:

General

- It would be helpful to agree on a limit for future reference although we have agreed on 2 sets of requirements for PDSCH demod with ICI and MU-MIMO.

- Finalize limit as 1 dB to decide if 2 sets of requirements are introduced.

CQI reporting requirements with inter cell interference

- Adopt the following methodology and criteria to decide CQI reporting requirements:
- Choose the SINR for defining requirements based on TP gain with baseline receiver. 
- At the chosen SINR, if the gamma value difference is > 0.2, then define 2 sets of requirements for simplified and baseline receiver

Applicability rule and release independence

- For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not.

- The requirements for 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario can be release independent from Rel-17.

# Reference

- R4-2504694, “Way Forward for [114bis][322] NR_demod_Ph5_Part2_UE”, Nokia.