3GPP TSG-RAN WG4 Meeting #115						R4-2505910

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.24.4.2

Source:	Apple

Title:	On RRM Requirements for MIMO Phase 5

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#114bis the other RRM requirements impact for MIMO phase 5 was discussed and WF [1] was approved. In this contribution we discuss RRM requirements impact for objectives other than UE initiated beam management.

# 2. Discussion

UE Reporting enhancement for CJT calibration

For CJT calibration the following agreements were made in RAN4#114:

Issue 1-1: On the simulation assumption for delay and frequency offset for CJT calibration Agreement: - Update the report resolution to (MD = 256, MFO = 256). Moderator’s note: simulation results are collected in R4-2504955, companies can use the table structure in the excel doc to update their results. Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration? Agreement: - Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified)

In the past few meetings, for CJT calibration, the issue of deciding the number of samples has been discussed. Simulation parameters have been agreed for further evaluation. In RAN4#114bis the simulation results for delay and frequency offset were compiled in [2].

We capture the results in terms of fraction of ppm for frequency offset error and fraction of CP for delay offset error in tables below.

Table 1: Error for Frequency Offset

| SCS | SNR | Freq Offset: 350Hz; FO estimation error in AFO = 0.2 ppm and MFO = 256 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max(CDF5%, CDF95%) fraction of ppm |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MTK |  |  | Ericsson |  |  | Huawei |  |  | Nokia |  |  |
|  |  | #samples |  |  |  |  |  |  |  |  |  |  |  |
|  |  | #1 | #3 | #5 | #1 | #3 | #5 | #1 | #3 | #5 | #1 | #3 | #5 |
| 15KHz | -3 | 0.051 | 0.030 | 0.023 | 0.011 | 0.006 | 0.005 | 0.056 | 0.034 | 0.026 | 0.010 | 0.008 | 0.005 |
| 15KHz | 0 | 0.030 | 0.019 | 0.014 |  |  |  | 0.030 | 0.026 | 0.013 | 0.008 | 0.006 | 0.005 |
| 15KHz | 3 | 0.020 | 0.013 | 0.009 | 0.005 | 0.003 | 0.002 | 0.026 | 0.016 | 0.010 | 0.005 | 0.005 | 0.004 |
| 30KHz | -3 | 0.104 | 0.055 | 0.047 | 0.020 | 0.013 | 0.009 | 0.116 | 0.052 | 0.046 | 0.017 | 0.013 | 0.010 |
| 30KHz | 0 | 0.065 | 0.038 | 0.029 |  |  |  | 0.081 | 0.045 | 0.027 | 0.013 | 0.009 | 0.008 |
| 30KHz | 3 | 0.041 | 0.024 | 0.019 | 0.009 | 0.005 | 0.005 | 0.053 | 0.027 | 0.022 | 0.009 | 0.007 | 0.005 |

Table 2: Error for Time offset

| SCS | SNR | Tome offset: 0.5 CP; TO estimation error in AD = CP and MD = 256 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max(CDF5%, CDF95%) Fraction of CP |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MTK |  |  | Ericsson |  |  | Huawei |  |  | Nokia |  |  |  |
|  |  | #samples |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | #1 | #3 | #5 | #1 | #3 | #5 | #1 | #3 | #5 | #1 | #3 | #5 |  |
| 15KHz | -3 | 0.113 | 0.074 | 0.070 | 0.031 | 0.023 | 0.016 | 0.109 | 0.078 | 0.070 | 0.059 | 0.012 | 0.004 |  |
| 15KHz | 0 | 0.066 | 0.047 | 0.039 |  |  |  | 0.063 | 0.047 | 0.039 | 0.059 | 0.012 | 0.004 |  |
| 15KHz | 3 | 0.039 | 0.027 | 0.023 | 0.023 | 0.023 | 0.016 | 0.047 | 0.031 | 0.023 | 0.035 | 0.012 | 0.004 |  |
| 30KHz | -3 | 0.117 | 0.074 | 0.066 | 0.039 | 0.031 | 0.023 | 0.063 | 0.039 | 0.031 | 0.059 | 0.012 | 0.004 |  |
| 30KHz | 0 | 0.063 | 0.043 | 0.039 |  |  |  | 0.039 | 0.031 | 0.023 | 0.059 | 0.012 | 0.004 |  |
| 30KHz | 3 | 0.055 | 0.035 | 0.035 | 0.039 | 0.031 | 0.023 | 0.031 | 0.023 | 0.016 | 0.059 | 0.004 | 0.004 |  |

Based on the results in the tables above the improvement is in the order of 1/100ths of CP for time offset and 1/100ths of ppm for frequency offset. It is obvious that more averaging would give us better results in terms or accuracy. But it is hard to understand the impact of error when the resolution is so small.

The improvement with averaging is in the order of 1/100 ths of ppm for frequency offset and 1/100 ths of CP for time offset.

The measurement accuracy would improve with more samples, but the impact is unknown given the small resolution.

Based on the above observations, we propose that the core requirements are defined for CJT calibration with assumption of one-shot measurement reporting.

- Define core requirements for CJT calibration reporting enhancement for delay/frequency/phase offset reporting based on single shot measurement.

Another issue is the necessity of scheduling restriction for CJT calibration. The proposal was based on the UE indicating a 1 bit when the estimated delay offset plus delay spread exceeds CP length. The baseline assumption of CJT transmission is that the MTTD is within CP. In case a scheduling restriction is introduced to avoid any ISI to ensure error free measurement based on TRS, then the scheduling restriction is to avoid any transmission on other TRPs on symbols where TRS is transmitted and adjacent symbols. The necessity of scheduling restriction needs to be further discussed.

Baseline assumption for CJT is that MTTD is within CP.

- RAN4 to discuss the necessity of scheduling restriction for CJT calibration reporting.

Asymmetric DL sTRP/UL mTRP

For asymmetric DL sTRP/UL mTRP  the following agreements were made in RAN4#114bis:

Issue 2-1: For Scenario#2, how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - Wait for RAN1 reply to the LS on whether the existing agreement for a single downlink timing reference is extended to Scenario#2. Issue 2-4: For Scenario#1 in FR2, whether to update the RRM requirements for 2TA enhancement for FR2? Agreement: - RAN4 to define timing requirements for FR1 as well as FR2.

On the timing requirements for 2TA, RAN4 has been discussing based on scenario 1 and scenario 2 –

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s).

RAN4 has received reply LS [3], with the following information:

Answer: The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s).

And RAN1 made the following new agreement for further clarification:

Agreement: - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG - Above applies for the case UE is configured with SSB-MTC-additionalPCI

Based on reply LS from RAN1 for scenario #2, the UE shall maintain 2 DL timing reference which is similar to R18 MIMO 2 TA enhancement for multi-DCI mTRP operation.

The timing reference for scenario #2 can be defined as:

For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point.

For multi-TRP operation with two TAs if the UE is provided SSB-MTC-AdditionalPCI, the reference point for PRACH transmission triggered by PDCCH order,

-	if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

-	if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG.

- Define the following for scenario #2:
For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point.

- Define timing reference for PDCCH ordered PRACH for scenario #2 as:
For multi-TRP operation with two TAs if the UE is provided SSB-MTC-AdditionalPCI, the reference point for PRACH transmission triggered by PDCCH order,
-	if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.
-	if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG

Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Way forward:

- Discuss whether gradual timing adjustment applies to both TAGs or not.

- Discuss whether gradual timing adjustment applies to DL/UL TRP or also to UL TRP.

In Rel-18 MIMO evo, for timing requirements with 2 TA, the gradual timing adjustment requirements were applicable to each TAG. In case of asymmetrical DL sTRP/ UL mTRP, at least for scenario 1 there is only 1 DL/UL TRP with corresponding reference timing. In case the UE needs to apply gradual timing adjustment based on propagation delay change at the UE, it can only be applicable to the TRP it receives DL reference from. The same gradual timing adjustment might not be applicable for the other TRP, as UE cannot predict if the same propagation delay change applies to UL only TRP.

In scenario #1, with single timing reference UE can only apply gradual timing to the TRP it receives DL RS for timing reference.

In scenario #1 the propagation delay change necessitating a gradual timing adjustment on DL/UL TRP might not be applicable to UL only TRP.

Hence, the gradual timing adjustment requirements shall be applicable only to TRP with both UL and DL.

- For scenario #1 - The gradual timing adjustment requirements are only applicable to TRP with both UL and DL.

Issue 2-6: Whether to define UL TCI state activation requirement for Scenario#2?

Way forward: discuss the following options in the next meeting for Scenario#2

- Option 1: No RRM impacts on TCI state switching requirement.

- Option 2: Legacy SSB based UL TCI state activation requirement can be re-used.

Based on RAN1 LS [3] the operation is like multi-DCI multi-TRP with 2TA, and the existing TCI state requirements introduced in R18 MIMO Evo should be applicable for scenario #2.

- For scenario #2 - The existing requirements for unified TCI state with mTRP are applicable without any updates.

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from UE initiated beam management. Our observations and proposals are captured below:

UE Reporting enhancement for CJT calibration

- The improvement with averaging is in the order of 1/100 ths of ppm for frequency offset and 1/100 ths of CP for time offset.

The measurement accuracy would improve with more samples, but the impact is unknown given the small resolution.

- Define core requirements for CJT calibration reporting enhancement for delay/frequency/phase offset reporting based on single shot measurement.

Baseline assumption for CJT is that MTTD is within CP.

- RAN4 to discuss the necessity of scheduling restriction for CJT calibration reporting.

Asymmetric DL sTRP/UL mTRP

- Define the following for scenario #2:
For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point.

- Define timing reference for PDCCH ordered PRACH for scenario #2 as:
For multi-TRP operation with two TAs if the UE is provided SSB-MTC-AdditionalPCI, the reference point for PRACH transmission triggered by PDCCH order,
-	if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.
-	if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG

In scenario #1, with single timing reference UE can only apply gradual timing to the TRP it receives DL RS for timing reference.

In scenario #1 the propagation delay change necessitating a gradual timing adjustment on DL/UL TRP might not be applicable to UL only TRP.

- For scenario #1 - The gradual timing adjustment requirements are only applicable to TRP with both UL and DL.

- For scenario #2 - The existing requirements for unified TCI state with mTRP are applicable without any updates.

# Reference

- R4-2504903, “WF on RRM requirements for NR_MIMO_Ph5_Part2”, MediaTek.

- R4-2504955, “Collection of simulation results for CJT calibration in NR_MIMO_Ph5_Part2”, MediaTek.

- R4-2505308, “Reply to RAN4 LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP”, RAN1, OPPO