3GPP TSG-RAN WG4 Meeting #115						R4-2505911

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.24.5

Source:	Apple

Title:	On RRM Performance Requirements for MIMO Phase 5

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

The performance part for Rel-19 MIMO Phase 5 WI starts in RAN4#115. In previous meetings we have discussed core requirements for MIMO Phase 5. In this contribution we present our views on the performance requirements impact and test cases for this WI.

# 2. Discussion

In previous meetings RRM impact analysis was discussed and the following table represents the summary.

| Topic | Description | RRM impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | Yes |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | No |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | No |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | No |
|  | Aperiodic standalone CJT calibration reporting | Yes |
| 3TX | 3-antenna-port codebook-based UL transmission | No |
| Asymmetric DL sTRP/UL mTRP |  | Yes |

### UE Initiated/ Event Triggered Beam Reporting

For UE-initiated beam reporting, the following agreements were made:

- RAN4 will define Event triggered measurement reporting delay requirement

  - Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air.

  - Ending point is the point when the UE transmits a first PUCCH triggered by the event.

- RAN4 requirements are defined for Event 2; FFS on applicability to Event 1/7.

- RAN4 requirements shall be discussed for FR2

- Reporting delay is L1_Measurement_period + Time to first UL Channel

  - L1 Measurement time shall consider both current beam and new beam measurement

  - Measurement period for current and new beam shall be based on existing L1-RSRP measurement period for SSB, SSB from cell with additional PCI and CSI-RS

- The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.

- Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

Based on the agreements, we propose to introduce test case for reporting delay requirement for UEIBR in FR2.

Introduce test case to verify UE initiated beam reporting delay in FR2.

### CJT Calibration Reporting

For CJT calibration, we agreed to introduce the following requirements:

- Define core requirements for delay and frequency offset reporting for

  - Measurement period (FFS number of samples)

  - Scheduling restriction (if identified)

The number of samples for CJT reporting is FFS.

For CJT calibration reporting, delay, frequency and phase offset are reported. The following agreements are made for these quantities:

Time offset: AD(Max delay offset)∈CP2,CP;MD(resolution) ∈{32, 64, 128, 256}

Frequency offset: AFO(Max freq offset)∈0.1ppm, 0.2 ppm;MFO(resolution) ∈{16, 32,  256}

Phase offset: n, -uniformly quantized phase between 0 and 2; MΦresolution∈{16, 32}

RAN4 should define mapping tables for the time offset, frequency offset, phase offset reporting based on the agreements of range and resolution.

RAN4 to define mapping tables for time offset, frequency offset, phase offset reporting based on the respective of range and resolution for CJT calibration reporting.

In core requirements we have collected simulation results to determine if we need to define requirements with > 1 sample for measurement. Based on the results, the accuracy is diverse among companies.

- Based on the results collected for FO/ TO accuracy the results are very diverse.

RAN4 should discuss feasibility of defining accuracy requirements for delay and frequency offset reporting for CJT calibration.

RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part.

### Asymmetric DL sTRP/ UL mTRP

RAN4 has agreed to define timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP. The following scenarios are considered:

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

RAN4 to discuss defining test cases for timing requirements for 2TA for sceraio#1 and scenario#2,

RAN4 to discuss defining test cases for timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP for scenario#1 and scenario#2.

# 3. Conclusion

In this paper, we present our views on the performance part impact and test cases for this WI. Our observations and proposals are captured below:

- Introduce test case to verify UE initiated beam reporting delay in FR2.

RAN4 to define mapping tables for time offset, frequency offset, phase offset reporting based on the respective of range and resolution for CJT calibration reporting.

- Based on the results collected for FO/ TO accuracy the results are very diverse.

RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part.

RAN4 to discuss defining test cases for timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP for scenario#1 and scenario#2.