# 3GPP TSG-RAN WG4 Meeting #115	R4-2505938

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.18.5

Source:	Apple

Title:	On testing for CSSF enhancement in CA/DC

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#114bis, and the open issues have been summarized in the topic summary for [114bis][208] NR_RRM_Ph5_Part1 [2].

In this contribution, we continue the discussion for testing of FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

# Discussion

Issue 2-2-1: TC list for CSSF optimization

- TC for solution 1: Agreement in online session: - Define the test cases for solution 1 of CSSF enhancement following the table below: TC index; Scenario; Need or not; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; Nokia. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; Ericsson. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; ZTE. - TC for solution 3: [Way forward]: - FFS: - Define the test cases for solution 3 of CSSF enhancement following the table below: TC index; Aspects; CA/DC mode; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA.

In last RAN4 meeting, it was agreed that,

Agreement:

- the CSSF optimization can be applied to the CSSF tables for the related CA/DC scenarios.

  - For solution 1: CSSF tables for CA, NR-DC, EN-DC and NE-DC

  - For solution 3: CSSF tables for CA, NR-DC, EN-DC with FR1 NR only

- Details of whether/how the CSSF factor is modified can be left to CR discussion.

Agreement:

- UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR.

- UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

Test case for solution 1:

For solution 1, if the NR carrier is FR2, we didn’t see the real deployment for NE-DC in any network, and therefore we prefer to remove NE-DC case from the test.

Proposal 1: remove the NE-DC case from solution 1 test.

Three are three operation modes to consider for solution 1 testing: FR2 CA, EN-DC, and NR-DC. To verify one carrier measurement per-band indicated by network, the FR2 target cell shall be a SCell rather than a PCell or PSCell. Thus, the current operation mode needs some extension to:

CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC

EN-DC: LTE PCC + FR1 PSCC + 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC

NR-DC: FR1 PCC + FR2 PSCC + FR2 SCC in same band as PSCC, network configures MOs on both FR2 CCs and UE by-default measures FR2 PSCC in FR2 band.

Proposal 2: the operation mode for solution 1 test shall be:

- CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC

- EN-DC: LTE PCC + FR1 PSCC + 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC

- NR-DC: FR1 PCC + FR2 PSCC + FR2 SCC in same band as PSCC, network configures MOs on both FR2 CCs and UE by-default measures FR2 PSCC in FR2 band.

| TC index | Scenario | Configuration |
| --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode: - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR1 PSCC + 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - NR-DC: FR1 PCC + FR2 PSCC + FR2 SCC in same band as PSCC, network configures MOs on both FR2 CCs and UE by-default measures FR2 PSCC in FR2 band. UE is only required to pass one of them |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG |  |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG |  |

Test case for solution 3:

For solution 3, we think UE supporting DC operation is typically support CA first, e.g., UE supporting FR1+FR2 DC shall support FR1+FR2 CA first in a typical case, and then the operation modes for consideration can be: FR1 only CA, FR1 only EN-DC, FR1+FR2 CA. To test the 3 searchers use case, the detailed configurations can be:

FR1 only CA: inter-band FR1 CA with 3CCs on 3bands

FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC

FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC

Proposal 3: operation modes for solution 3 test are FR1 only CA, FR1 only EN-DC, FR1+FR2 CA

Proposal 4: detailed operation modes for solution 3 test are:

- FR1 only CA: inter-band FR1 CA with 3CCs on 3bands

- FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC

- FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC

| TC index | Scenario | Configuration |
| --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode: - FR1 only CA: inter-band FR1 CA with 3CCs on 3bands - FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC - FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC UE is only required to pass one of them |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG |  |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG |  |

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: remove the NE-DC case from solution 1 test.

Proposal 2: the operation mode for solution 1 test shall be:

- CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC

- EN-DC: LTE PCC + FR1 PSCC + 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC

- NR-DC: FR1 PCC + FR2 PSCC + FR2 SCC in same band as PSCC, network configures MOs on both FR2 CCs and UE by-default measures FR2 PSCC in FR2 band.

| TC index | Scenario | Configuration |
| --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode: - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR1 PSCC + 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - NR-DC: FR1 PCC + FR2 PSCC + FR2 SCC in same band as PSCC, network configures MOs on both FR2 CCs and UE by-default measures FR2 PSCC in FR2 band. UE is only required to pass one of them |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG |  |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG |  |

Proposal 3: operation modes for solution 3 test are FR1 only CA, FR1 only EN-DC, FR1+FR2 CA.

Proposal 4: detailed operation modes for solution 3 test are:

- FR1 only CA: inter-band FR1 CA with 3CCs on 3bands

- FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC

- FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC

| TC index | Scenario | Configuration |
| --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode: - FR1 only CA: inter-band FR1 CA with 3CCs on 3bands - FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC - FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC UE is only required to pass one of them |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG |  |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG |  |

# References

[1] R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#114bis

[2] R4-2503618, Topic summary for [114bis][208] NR_RRM_Ph5_Part1, Moderator (Apple), RAN4#114bis